Forms and Explanatory Items

To be applied from
data submission 2009 onwards

Version 5.3f

Last update 4 Oct 2011

Contents

List of Abbreviations and Acronyms	3

1	Introduction	4

2	General Remarks	4

3	Amendment Index	5

3.1	Amendments to Version 5.3d	5

3.2	Amendments to Version 5.3c	6

3.3	Amendments to Version 5.3b	6

3.4	Amendments to Version 5.3a	6

3.5	Amendments to Version 5.3	6

3.6	Amendments to Version 5.2i	6

3.7	Amendments to Version 5.2g	7

3.8	Amendments to Version 5.2e	7

3.9	Amendments to Version 5.2c	7

3.10	Amendments to Version 5.2a	8

3.11	Amendments to Version 5.1	8

3.12	Amendments from version V 5.1 to document version V 4.7c	9

3.13	Amendments to last form defintions for data years 2007 and 2008	10

4	Forms	14

4.1	System Installation	14

4.2	Crown Condition	15

4.3	Tree vitality (D1)	18

4.4	Soil water	20

4.5	Sampling and Analysis of Soil	22

4.6	Soil Solution Collection and Analysis	31

4.7	Sampling and Analysis of Needles and Leaves	35

4.8	Ground Vegetation Biomass and Nutrients Analyses	39

4.9	Assessment of Growth and Increment	42

4.10	Sampling and Analysis of Deposition	47

4.11	Meteorological Measurements	51

4.12	Assessment of Ground Vegetation	54

4.13	Phenological Observations	56

4.14	Monitoring of Air Quality	59

4.15	Assessment of Ozone Injury	64

4.16	Sampling and Analysis of Litterfall	67

4.17	Leaf Area Index (LAI) and Radiation Measurements	71

5	Explanatory Items	75

# List of Abbreviations and Acronyms

AAS	Atomic Absorption Spectrometer

BD		Bulk Density

BS		Base Saturation

CEC	Cation Exchange Capacity

DAR-Q	Data Accompanying Report Questionnaire

Ec		Electrical conductivity

F		Fermentation horizon

FAO	Food and Agriculture Organization

FES	Flame Emission Spectrometer

GPS	Global Positioning System

H		Humus horizon

ICP		Inductivity Coupled Plasma Spectrometer

IRM	International Reference Material

ISO	International Organization for Standardization

JRC	Joint Research Centre, Ispra, European Commission

L		Litter horizon

LAI	Leaf Area Index

LRM	Local Reference Material

M		Mandatory parameter

MBD 	Mineral Bulk Density

NFC	National Focal Centre of the Intensive Monitoring Programme

NRM	National Reference Material

O		Optional parameter

OM	Organic Matter

QA/QC	Quality Assurance and Quality Control

SA		Soil Analysis method

SAG	Scientific Advisory Group of the Intensive Monitoring Programme

SD		Standard Deviation

SFC	Standing Forestry Committee

WRB	World Reference Base for Soil Resources

# Introduction

This document concludes all forms which are used for data submission from the year 2009 onwards in the frame of the FutMon programm as well as of the ICP Forests programme. Changes which are made in comparison with the latest adopted version of the respective ICP Forests forms are highlighted by using bold font and blue colour. In addition all amendments and changes are summarized in the amendment index.

# General Remarks

Some general remarks are made here in order to allow a high quality data submission:

- As the combination of parameters which are submitted with a specific form may vary over time due to manual changes it is necessary to document in the submission files which fields/parameters are submitted with this file. The submitting project partners will do so by including a first line into each submission file which starts with an exclamation mark followed by a comma separated list of the submitted fields/parameters.

Example for first (comment) line submitted within the file XXGENER.PLT:

!Sequence, country, plot, latitude, longitude, altitude, orientation, date_installation, plot_size, trees, sub_plot_size, mean_age, tree_species, yield_abs, yield_relative, other_observations

Those fields which are expected to be continuous and not to change over time are underlined. Fields which are part of the key of the respective form/table are printed in bold font. Thus, each line in a submitted file must be identified by its own and unique combination of the values in those key fields.

- Further comments may made before the data specification line but will not be tested automatically during the validation process. Each comment paragraph or line has to start with an exclamation mark. The last comment line before the first data record must be the data specification line which is specified on top of each form (s. below).

- Only in case of the last table fields which are named “other_observations” (or similar) please use left alignment.

- Data submission will be done using the submission module of the FutMon data base. The data will be submitted survey by survey and year by year. Thus, each project partner (FutMon partner or ICP Forests NFC) has to submit a complete set of a survey of a specific year to the submission module. This set includes in general a reduced plot file, data files, data accompanying reports (word documents) and in case of surveys with data from laboratory analyses a laboratory QA file (.LQA).

- The data will be submitted using fixed format ASCII files. The formats are described in this document including start and end column of each parameter. During a data submission workshop the FutMon data centre will introduce into methods for data preparation (e.g. EXPORT of fixed format file from an EXCEL sheet).

- For each parameter the number of digits is defined in the forms below. Metric values are not defined to have a specific number of decimal units but will be stored in the database as floating point values. Thus, a parameter with 4 digits could have values as:

“3456” or “22.4” or “2.63” or “02.6” or “ 2.6” or “2.6 “

The parameter values are separated by blanks. A decimal point is used.

In case that not all digits are needed for data submission the data should be stored right-aligned. If e.g. a field with 5 digits is defined for a form and the three digit code "BHI" should be submitted in this specific field it is recommended to insert it in the data submission file in form of "  BHI" instead of "BHI  ".

- In case of values which are too high to be submitted with the specified number of digits (e.g. “10243.1” in case of a parameter with only 4 digits “9999” should be submitted instead and the true value (“10243.1”) should be submitted in the text field “other observations”. This should be done following the form “VALUE FOR <param> is <value>” where <param> is the name of the submitted parameter and <value> is the value of the parameter.

- If values are below the quantification limit a “-1” should be submitted. The quantification limit for the respective parameters has to be submitted in the accompanying data report. A “0” (zero) should be used only in case that this is the assessed or measured value, e.g. “0” for “precipitation” in case that no precipitation during the respective period. Other parameters could be “Weight of oranic layer”, “Carbonates” (Soil), or alkalinity (Soil Solution, Depo).

- The format of the parameters to be submitted will be given using 
I X 			for Integer values with X digits,
F X 			for floating point values with X digits,
C X			for character values with X digits, and
DATE/TIME 	for date or time values (exact format will be specified in 
			the respective explanatory item or in the form definition).
Y/N			for indicating a “yes” the code  “Y” is specified, “N” for “no”

- In case that a specification of a parameter (e.g. F 4) is different from the numbers which are used in the columns specification (e.g. 15 – 17), please, immediately contact the FutMon data centre in order to allow for a clarification with the next update of this document and to get a valid decision on how to define the data submission form.

- The column “Ref_Tab” is an index (X) if the respective value of the parameter must be concluded in a reference table.

- The respective explanatory item where some details for the codifying of the parameter value is explained is specified in the column “Item #”. If a reference table is indicated (s. point above; column “Ref_Tab”) the reference table is included in this specific explanatory item.

- Make sure that the two letters “ and ; (quotation mark and semicolon) are not included at any place in the submitted files. Both letters are used by the system internally during the data validation and dissemination process.

# Amendment Index

## Amendments to Version 5.3d

- DP, PLD: for sampler surface in case of stemflow it was clarified in explanatory item (58) that the basal area of the sampled trees is to be submitted.

- SW, SWC: The format for field bulk density was changed from F4 to I4.

- LA, LAM: It was clarified, that photo_file is also part of the key. Please, submit 
”-9” in case of data which are not related to any photo file.

## Amendments to Version 5.3c

- LAI, LAM and LAP: Text in forms was revised in order to adapt name specification for photo files to the format of the explanatory item (125).

## Amendments to Version 5.3b

- MM, MEO: The unit for Matric Potential in explanatory item (62) was corrected to hPa (as already stated in explanatory item (64) instead of kPa.

## Amendments to Version 5.3a

- SO, SOM and LQA: Parameter It was corrected in parameter list in explanatory item (113) that parameter CaCO3 is specified to be “Carbonates” instead of C_CO3. In addition, in form SOM carbonates was amended by “CaCO3”: Carbonates (g/kg CaCO3)

- LQAs: In all LQA forms the format for the coefficient of variation of the control charts was changed from I 3 to F 3 in order to allow for submission of values from .01 to 999. Lower or higher values are to be reported as defined in the General Remarks section for higher values.

- FO, FOM and FOO, sample code: In order to allow for a submission of data on leaves which are older than current year (code “0”) but without the possibility of distinction between “current year +1” (code “1”) and “oder than current + 1” (code “2”) an additional code “3” was introduced in explanatory item (38).

## Amendments to Version 5.3

- MM, Wind direction: Explanatory item (64) was corrected according to Amendment indes to Version 5.2, point b): 12 sections with 30° instead of 8 sections with 45°. The examples, which were correct already, remain unchanged.

## Amendments to Version 5.2i

- LA, LAP: The field <parameter> is part of the key of this form in order to allow for the submission of more than one parameter per photo file.

- MM, WD: It was clarified in explanatory item (64) that wind direction has to be submitted not with 8 but with 12 possible sections, each with 30° range. In any case, as before, the middle of the range has to be submitted. E.g. “0”, “30”, “60”, …, “330”.

- <TREE_species> code: Actually, there are codes for tree species higher than 199 in explanatory item (20). Accordingly the wording “001 to 199” was deleted from all related forms.

- LQA: If there was no participation in a ring test for a specific parameter the fields <ring_test_number>, <Laboratory_ID>, and <percentage_within> have to be left blank as described in explanatory item (107).

## Amendments to Version 5.2g

- SW, SWC and SWA: <depth layer>, a part of the key of SWC table, is not longer defined in a reference table which would always be incomplete. Instead, the definition of this key between the tables SWC and SWA and the code for <depth layer> is described in more detail and with examples in explanatory item (71).

## Amendments to Version 5.2e

- General Remarks: Code Y/N is now defined in the explanatory items; used already in several forms

- General Remarks: Hint that neither a semicolon nor a quotation mark shall be used in the submitted files

- D1G: some corrections and clarifications to old definitions, air temperature skipped from the form;

- Growth, PLI: Total plot parameters (area and number of trees) were skipped from the form PLI

- Growth, IPM: according to text in ICP Forests manual the field <crown_length> was renamed to <crown_base_height>. In the description text accordingly:
“Crownlength rounded off to the nearest 0.1 meters”  “Height to crown base” (as was already defined in the manual text).

- AQ, COL: unit of inlet height is same as in PAC: [m]. Anyhow, the accuracy should be up to at least 0.1m in both forms.

## Amendments to Version 5.2c

- The general remarks section was amended by a paragraph at end of the first bullet point in order to clarify how key fields are designated and what the meaning and use of key fields is.

- Soil, layer: Explanatory item (72) was adapted to codes used in BioSoil project.

- Growth, removal: code for removal_mortality 04 was introduced (tree not longer in growth sample due to heavy disturbances (e.g. heavy storm damage))

- Crown, TRD und TRF: the key field names (written in bold font) were adapted for TRF according to those in TRD

- Crown, TRD and TRF: field "cause_sc_name" became part of the key

- Soil Water: SWA “Matric preassure” became part of the key of form SWA

- In PLM explanatory item (70) was introduced for pit ID

- Leaf Area Index, LAM:  date_analysis, determination, parameter became part of the key

- Meteo, explanatory item (64): all specifications concerning the number of digits were revised according to the general remarks (floating point values instead of fixed decimal places)

- Ozone, active sampler, AQA: Unit for ozone has been corrected from ppm to ppb

- Litterfall, LFM and LFO: field "trap" number has been defined to be of code "-9" in case of plot averages instead of blank (Null value) which is not appropriate in a key field.

- Tree species codes, Explanatory Item (20): Codes 201 to 208 were added for Anatolian oak species; code 89 for Ceratonia siliqua was deleted, code 75 has to be used for this species.

- Deposition, LQA form and Explanatory item (105): pretreatment method codes for deposition samples were introduced.

## Amendments to Version 5.2a

- Air quality: Sequence Number in form AQA was widened to 6 digits, all subsequent fields (columns) were shifted accordingly.

## Amendments to Version 5.1

- Foliage: sample ID consists of 5 digits first 3 digits is the code for tree species, digit 4 is a dot (“.”) and digit 5 is the leaves type Examples for valid entries are introduced in the respective forms.

- Deposition; form DEO: columns for start_date and end_date were corrected

- Soil solution, form PSS: Format and reference for “layer” corrected

- Soil: Expalanatory Items 68 and 69 were removed in order to avoid redundancy.

- Soil; PFH; Horizon discontinuity: If no discontinuity is detected the value 0 (zero) shall be submitted instead of submitting a blank (for technical reasons)

- Soil; PFH; Horizon vertical: If no discontinuity is detected the value 0 (zero) shall be submitted instead of submitting a blank.

- Soil, LQA, pretreatment: The key has now format C 4 instead of I 4 in order to allow for the distinction between 3.1 and 3.10

- Pheno, PHD and LAI, LAP and LAM; file name: field was corrected from C18 to C28!!! All subsequent columns are changed correspondingly!!!

## Amendments from version V 5.1 to document version V 4.7c

Compared to Version 4.7c, which has been provided on the FutMon webpage, (internal login) some significant changes and amendments were done:

- In general remarks two new points: 
1) comment lines in data files are specified! 
2) data fields in data files should be right-aligned

- XXGENER.PLT: Description of date corrected from DD-MM-YY to DDMMYY

- Tree Vitality, D1T: comments on removal and mortality shifted behind the respective data field, additional "other observations" field added at end of file.

- Table Score of event , explanatory item (123) was clarified: codes for flowering are 6, 7, 7.1, 7.2 and 7.3, codes for damage 6 and 7.

- Soil .PRF: Reference tables for mean highes and lowest ground water table” and for “water table type” (old explanatory items (137) and (138)) are replaced by respective tables in which the code 0 is replaced by code 9 (explanatory items (151) and (152)).; in form SOM “X” added for field “texture class”

- Soil .SOM: it must be pronounced that from data submission 2009 onwards the sequence of particle size data fields changed from clay – sand – silt (up to data submission 2008) to clay – silt – sand!!!

- Soil solution: SSO: format of water content was corrected to F4

- Growth forms: Wrong data formats and inconsistent form definition were corrected

- Depo, form DEO: seqence of start and end date corrected

- Meteo data form: columns in MEM were corrected for "other observations"; form PLM: Formats were corrected and completed; Field “Variable” is with reference table (“X” inserted) in form MEM and MEO

- Ground Vegetation Biomass: form PGB: Format of sequence number was corrected; “X” for reference table inserted in form .GBO, field “species code” and in form .GBH, field “species code”

- Phenology: In PHD the parameter “event” was included in order to allow for a respective search for photo documentations in the database.

- Descriptions for Air quality and Ozone forms were added

- Inconsistencies on ring test numbers in LQ- forms were solved; format changed from C 4 to C3

- LAI: LAM: column numbers had to be changed due to inconsistencies with format of field parameter code; LAC: 8 digits were needed for X-coordinate => changes in columns; PLA: columns for Exposition in degree shortened from 36 – 39 to 36 – 38 as only 3 digits needed.

- OZ: Format was inserted for first four fields and X inserted for field country code in form LTF

- AQ: form AQB, AQP: Format was corrected for field "Compound air quality"; form COL: columns for field "Passive sampler manufacturer" was shortened and consecutive fields where shifted accordingly; form AQA: Formats were corrected and completed!

## Amendments to last form defintions for data years 2007 and 2008

All changes and amendments which were made compared to the latest adopted version of the ICP Forests Manual are highlighted in the text by bold blue font and listed below. Some amendments which clarify the existing definitions in the explanatory items are only highlighted in the text but not listed below (e.g. Explanatory item #(3): An example is made for the values which have to be submitted).

| Survey | Form | Description |
| --- | --- | --- |
| System Installation | XXGENER.PLT | „Only information concerning new plots or plots with information updates have to be submitted.“ |
| Tree vitality | .D1T, .D1G | The forms D1T and D1G were introduced for the submission of tree vitality and permanent an continouos dbh measurements, respectively, concerning project D1 |
| Soil | .PLS, .SOM | The exisiting forms are adapted to the BioSoil project definition as far as possible before getting the BioSoil data and latest information on the used coding. A respective update will follow as soon as BioSoil data will be available to the FutMon project. |
| Soil | .SOM | The sequence of data fields in form .SOM was changed from Clay – Sand – Silt to Clay – Silt – Sand !!! |
| Soil | .PFH | A new file .PFH is introduced in order to submit information on soil profile horizons description according to the soil classification under BioSoil. |
| Soil | .PRF | A new file .PRF is introduced in order to submit information on soil profile description according to the soil classification under BioSoil. |
| Soil water | .SWC, .SWA | New forms for data submission on soil water retention are defined, .SWC for submission of sample definition and .SWA for submission of respective (multiple) analysis results. UPDATE in version 4.7 |
| Foliage | .PLF, .FOM, .FOO | Other obersvations (text) enlarged to 40 digits. |
| Foliage | .FOM & .FOO | Leaves type 2 = older than current + 1 was introduced in foliage assessments (FOM and FOO and explanatory item (38)); this code 2 will be of use only on the D2 plots where the FutMon participants have to analyse older needles in addition. |
| Foliage | .FOM | In order to allow for the submission of further tree number (only 5 tree number may be submitted with .FOM yet) it is foreseen that those further tree numbers will be submitted in a second line with the same analysis results for nutrients, biomass, etc. as those in the first respective line. |
| Foliage | XX2009FO.LQA | The new form for submission of laboratory QA data is introduced. |
| Biomass | .PGB, .GBM, .GBO, .GBH, XX2009GB.LQA | For the assessment of biomass and nutrient analyses of ground vegetation 4 new forms will be used for the submission of respective data on sample analyses together with a Laboratory QA file (based on foliage ring test results). |
| Deposition | .PLD | The fields Sampler model, Sampler Height, Sampler Surface and No of samplers is introduced |
| Deposition | .DEM and .DEO | The field V_sampling (assessment/estimation method) is introduced |
| Deposition | XX2009DP.LQA | The new form for submission of laboratory QA data is introduced. |
| Phenology | .PHE | <sequence number> was enlarged to 5 digits => all following column numbers changed |
| Phenology | .PHE | if event code for damage on leaves (“4”) or other damage (“5”) is indicated, a respective data record is submitted with the foreseen damage form (.TRD); fields concerning damage are excluded from the phenology forms! |
| Phenology | .PHE | <Date before> and <Date after> are replaced by date of observation |
| Phenology | .PHE | Score of event is amended by scores from 1 (<1%) to 5 (>99%); the score is defined by thirds of trees being involved by the event. |
| Phenology | .PHE | Stage of event is amended by codes from 1 (<1%) to 5 (>99%); |
| Phenology | .PLP | An explanatory Item for <Installation Date> was introduced; no new information, only in order to avoid misunderstandings |
| Phenology | .PLP | An explanatory Item for <visible part of the crown> was introduced; no new information, only in order to avoid misunderstandings |
| Phenology | .PLP | Code for vertical direction FROM where the observations were made is introduced including a respective explanatory item;  following column numbers changed |
| Phenology | .PHI | <sequence number> was enlarged to 5 digits => all following column numbers changed |
| Phenology | .PHI | if event code for damage on leaves (“4”) or other damage (“5”) is indicated, a respective data record is submitted with the foreseen damage form (.TRD); fields concerning damage are excluded from the phenology forms! |
| Phenology | .PHI | Score of event at observed tree is introduced by codes from 1 (<1%) to 5 (>99%); |
| Phenology | .PHI | Stage of event is amended by codes from 1 (<1%) to 5 (>99%); |
| Phenology | .PHI | <Method used for observation> is introduced |
| Phenology | .PHD | A new form (.PHD) was introduced for submission of information on submitted digital photographs and images. |
| Meteorology | .PLM | New column Profile Pit ID was introduced in order to get the linkage to the soil pit analyses values which are submitted with the SWC file |
| Meteorology | .MEM | Fields Origin and Status were introduced in order to document origin and status of the submitted data |
| Meteorology | .MEO | Fields Origin and Status were introduced in order to document origin and status of the submitted data |
| Litterfall | .LFM | unit of C changed from [mg/g] to [g/100g] in order to have uniform format with foliage and Ground vegetation biomass |
| Litterfall | .LFO | Al and Na were dropped following ICP Forests Task Force meeting 2007; respective columns will be empty (blanks to be submitted) |
| Litterfall | .LFO | Cd introduced with unit [ng/g] in order to have uniform format with foliage and Ground vegetation biomass; Cd will be submitted with LFO. |
| Litterfall | .LFO | unit of Zn, Mn, Fe, Cu, Pb and B changed from [mg/g] to [µg/g] in order to have uniform format with foliage and Ground vegetation biomass |
| LAI | .PLA | Reduced plot file for LAI survey was introduced |
| LAI | .LAC | Form for submission of relative coordinates of devices and measurement point (LAI and other surveys) was introduced |
| LAI | .LAP | Form for submission of data on hemispherical photos and photos of photo mosaic method was introduced |
| LAI | .LAM | Form for submission of LAI measurement / assessment values and additional output was introduced |
| SO, SS, FO, DP, LF, AQ | .LQA | For all surveys with parameters which are analysed at laboratories so called “Laboratory Quality Assurance” forms (.LQA) are defined and have to be submitted with the data of the respective survey. |
| Air Quality | .PAC | Form was revised in order to assure link between plot file of active samplers with measurement data (now in .AQA) |
| Air Quality | .PPS | Form was revised in order to assure link between plot file of passive samplers with measurement data (now in .AQP) |
| Air Quality | .AQM | Form not longer valid, submission of measurement data now with .AQA for active sampler and with .AQP for passive samplers |
| Air Quality | .AQA | Form introduced for submission of measurement data from active samplers |
| Air Quality | .AQP | Form introduced for submission of measurement data from passive samplers |
| Air Quality | .AQB and .COL | Forms introduced for documentation of quality assureance/control measures |
| Litterfall | .LFM and .LFO | Explanatory Item (154) will be used instead of explanatory item (40), thus the new codes are not listed in explanatory item 40 any longer |

# Forms

## System Installation

The file for system instalment has to be submitted if new plots have to be defined or in case that respective data have to be changed in the data base. Only the information concerning new plots or plots with information updates has to be submitted. All other plots may be re-submitted with the XXGENER.PLT file.

### XXGENER.PLT
Contents of file with the information on Plot level

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, latitude, longitude, altitude, orientation, date_installation, plot_size, trees, sub_plot_size, mean_age, tree_species, yield_abs, yield_relative, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country Code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 9 – 12 | Observation plotnumber (max. 9999) | I 4 |  | (2) |
| 14 – 20 | Latitude in +DDMMSS (e.g.+505852) | C 7 |  | (4) |
| 22 – 28 | Longitude in (+ or -)DDMMSS (e.g. +035531) | C 7 |  | (4) |
| 30 – 31 | Altitude (in 50 metre classes from 1 to 51) | I 2 | X | (7) |
| 33 | Orientation (N =1, NE = 2, etc.) | I 1 | X | (8) |
| 35 – 40 | Installation date in DDMMYY | Date |  | (3) |
| 42 – 47 | Total plot size (in 0.0001 ha) | F 6 |  | (16) |
| 49 – 52 | Number of trees in total plot | I 4 |  | (17) |
| 54 – 59 | Size of sub-plot (in 0.0001 ha) | F 6 |  | (16) |
| 61 – 62 | Mean age of dominant storey (in 20 year classes from 1 to 8) | I 2 | X | (9) |
| 64 – 66 | Main tree species code | I 3 | X | (20) |
| 68 | Yield estimate – absolute | I 1 | X | (18) |
| 70 | Yield estimate – relative | I 1 | X | (18) |
| 72 – 111 | Other observations (text) | C 40 |  | (84) |

Part II

Visual Assessment of 
Crown Condition

## Crown Condition

### XX2009.PLO 
Contents of file with the information on Plot level to be used in combination with the tree vitality inventory Level I

Each data file has to start with a comment line. This line starts with an exclamation mark !Sequence, country, plot, date, latitude, longitude, water, humus, altitude, orientation, mean_age, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country code | I 2 | X | (1) |
| 9 – 12 | Observation plotnumber (max. 9999) | I 4 |  | (2) |
| 14 – 19 | Date | Date |  | (3) |
| 21 – 27 | Latitude | C 7 |  | (4) |
| 29 – 35 | Longitude | C 7 |  | (4) |
| 37 | Water availability | I 1 | X | (5) |
| 39 | Humus type | I 1 | X | (6) |
| 41 – 42 | Altitude | I 2 | X | (7) |
| 44 | Orientation | I 1 | X | (8) |
| 46 – 47 | Mean age of dominant storey | I 2 | X | (9) |
| 49 – 76 | Soil unit (e.g. e.g. Hypercalcaric-Chromic LUVISOL) | C 27 | X |  |
| 78 – 117 | Other observations (text) | C 40 |  | (84) |
| Note |  |  |  |  |

### XX2007.TRE
Crown condition parameters Level I

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, date, tree, tree_species, defol, discol, fruiting_assess, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number of trees (1 to 99999) | I 5 |  |  |
| 7 – 10 | Observation plot number (max. 9999) | I 4 |  | (2) |
| 12 – 17 | Date of survey in DDMMYY (e.g. 220704) | Date |  | (3) |
| 19 – 22 | Treenumber (as marked during installation) | I 4 |  | (19) |
| 24 – 26 | Tree Species code | I 3 | X | (20) |
| 28 – 30 | Defoliation (0,5,10,15 ... 95,99,100%) | I 3 | X | (25) |
| 32 | Discolouration (0,1,2,3,4) [optional] | I 1 | X | (26) |
| 34 | Fruiting in assessable crown (1,2,3) [optional] | I 1 | X | (29) |
| 36 – 75 | Other observations (text) | C 40 |  | (84) |

### XX2007.TRF
Damage parameters Level I

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, date, tree, affected_part, symptom, symptom_spec, crown_loc, damage_age, cause, cause_sc_name, extent, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number of trees (1 to 99999) | I 5 |  |  |
| 7 – 10 | Observation plot number (max. 9999) | I 4 |  | (2) |
| 12 – 17 | Date of survey in DDMMYY (e.g. 220704) | Date |  | (3) |
| 19 – 22 | Treenumber (as marked during installation) | I 4 |  | (19) |
| 24 – 25 | Specification of affected part (11, ..., 34) | I 2 | X | (32) |
| 27 – 28 | Symptom (01,..., 22) | I 2 | X | (33) |
| 30 – 31 | Specification of symptom (31, ..., 67) [optional] | I 2 | X | (33) |
| 33 | Location in crown (1,2,3,4) [optional] | I 1 | X | (32) |
| 35 | Age of damage (1,2,3) [optional] | I 1 | X | (34) |
| 37 – 41 | Cause (e.g. 81001) | I 5 | X | (35) |
| 43 – 49 | Scientific name of cause (e.g. LOPHSED) | C 7 | X | (36) |
| 51 | Extent (0,1,2,3,4,5,6,7) | I 1 | X | (37) |
| 53 – 92 | Other observations (text) | C 40 |  | (84) |

### XX2007.PLT
Contents of file with the information on plot level to be used with the crown assessment on Level II

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date, latitude, longitude, altitude, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country code | I 2 | X | (1) |
| 9 – 12 | Observation plot # | I 4 |  | (2) |
| 14 – 19 | Date of assessment (DDMMYY) | Date |  | (3) |
| 21 – 27 | Latitude in + DDMMSS (e.g. +505852) | C 7 |  | (4) |
| 29 – 35 | Longitude in (+ or -) DDMMSS (e.g. -035531) | C 7 |  | (4) |
| 37 – 38 | Altitude (in 50 meter classes from 1 to 51) | I 2 | X | (7) |
| 40 – 79 | Other observations (text) | C 40 |  | (84) |

### XX2007.TRC
Crown condition parameters Level II

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, date, tree, tree_species, removal_mortality, social_class, shading_crown, visibility, defol, discol, flowering_assess, flowering_whole, fruiting_assess, fruiting_whole, transparency, form_crown, secondary_shoots, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number of trees (1 to 99999) | I 5 |  |  |
| 7 – 10 | Observation plot number (max. 9999) | I 4 |  | (2) |
| 12 – 17 | Date of survey in DDMMYY (e.g. 220704) | Date |  | (3) |
| 19 – 22 | Treenumber (as marked during installation) | I 4 |  | (19) |
| 24 – 26 | Tree Species code | I 3 | X | (20) |
| 28 – 29 | Removals & mortality | I 2 | X | (21) |
| 31 | Social class (1,2,3,4) | I 1 | X | (22) |
| 33 | Crown shading (1,2,3,4,5,6) | I 1 | X | (23) |
| 35 | Visibility (1,2,3,4) | I 1 | X | (24) |
| 37- 39 | Defoliation (0,5,10,15 ... 95,99,100%) | I 3 | X | (25) |
| 41 | Discolouration (0,1,2,3,4) [optional] | I 1 | X | (26) |
| 43 | Flowering in assessable crown (1,2,3) [optional] | I 1 | X | (28) |
| 45 | Flowering in whole crown (1,2,3) [optional] | I 1 | X | (28) |
| 47 | Fruiting in assessable crown (1,2,3) [optional] | I 1 | X | (29) |
| 49 | Fruiting in whole crown (1,2,3) [optional] | I 1 | X | (29) |
| 51 – 53 | Foliage transparency (0,5,10,15,...,95,99,100) [optional] | I 3 | X | (27) |
| 55 – 56 | Crown form (11 to 39) [optional] | I 2 | X | (31) |
| 58 | Secondary shoots & epicormics (1,2,3) [optional] | I 1 | X | (30) |
| 60 – 99 | Other observations (text) | C 40 |  | (84) |

### XX2007.TRD
Damage parameters Level II

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, date, tree, affected_part, symptom, symptom_spec, crown_loc, damage_age, cause, cause_sc_name, extent, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number of trees (1 to 99999) | I 5 |  |  |
| 7 – 10 | Observation plot number (max. 9999) | I 4 |  | (2) |
| 12 – 17 | Date of survey in DDMMYY (e.g. 220704) | Date |  | (3) |
| 19 – 22 | Treenumber (as marked during installation) | I 4 |  | (19) |
| 24 – 25 | Specification of affected part (11, ..., 34) | I 2 | X | (32) |
| 27 – 28 | Symptom (01,..., 22) | I 2 | X | (33) |
| 30 – 31 | Specification of symptom (31, ..., 67) | I 2 | X | (33) |
| 33 | Location in crown (1,2,3,4) | I 1 | X | (32) |
| 35 | Age of damage (1,2,3) [optional] | I 1 | X | (34) |
| 37 – 41 | Cause (e.g. 81001) | I 5 | X | (35) |
| 43 – 49 | Scientific name of cause (e.g. LOPHSED) | C 7 | X | (36) |
| 51 | Extent (0,1,2,3,4,5,6,7) | I 1 | X | (37) |
| 53 – 92 | Other observations (text) | C 40 |  | (84) |

## Tree vitality (D1)

### XX2009.D1T
FutMon D1 parameters on tree level (Tree age, Fruiting, CDRD_N, Apical shoot architecture, Removal/Mortality)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, tree, date, tree_age_class, age_tree, age_method, age_comment, fruiting_assess_ext, fruiting_comment, CDRD_N, CDRD_N_comment, apical_shoot, apical_shoot_comment, removal _ext, removal_ext_comment, affected_part, symptom, symptom_spec, cause, cause_sc_name, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 - 5 | Sequence number (1-99999) | I 5 |  |  |
| 7 - 8 | Country code | I 2 | X | (1) |
| 10 - 13 | Plot Number | I 4 |  | (2) |
| 15 - 18 | Treenumber (as marked during installation; same as in .TRC) | I 4 |  | (19) |
| 20 - 25 | Date of Assessment | Date |  | (3) |
| 27 - 27 | Tree Age class | I 1 | X | (10) |
| 29 - 31 | Tree age (years, if exact data of +/- 5 years available; optional) | I 3 |  |  |
| 33 - 33 | method of age assessment | I 1 | X | (11) |
| 35 - 74 | comments on age assessment | C 40 |  | (84) |
| 76 - 78 | Fruiting in assessable crown (1.1, 1.2, 2, 3); mandatory on Fagus ssp., Picea abies and Pine ssp. | F 3 | X | (12) |
| 80 - 119 | comments on fruiting | C 40 |  | (84) |
| 121 - 125 | Crown related distance to neighbour; CDRD_N | F 5 |  | (13) |
| 127 - 166 | comments on CDRD_N | C 40 |  | (84) |
| 168 - 168 | Apical shoot architecture (Beech) | I 1 | X | (14) |
| 170 - 209 | comments on Crown Architecture | C 40 |  | (84) |
| 211 - 211 | Removal/Mortality (new) | I 1 | X | (15) |
| 213 - 252 | Comments on Removal/Mortality | C 40 |  | (84) |
|  | Coding accoring to .TRD: |  |  |  |
| 254 - 255 | Specification of affected part (11, ..., 34) | I 2 | X | (32) |
| 257 - 258 | Symptom (01,..., 22) | I 2 | X | (33) |
| 260 - 261 | Specification of symptom (31, ..., 67) | I 2 | X | (33) |
| 263 - 267 | Cause (e.g. 81001) | I 5 | X | (35) |
| 269 - 275 | Scientific name of cause (e.g. LOPHSED) (optional) | C 7 | X | (36) |
| 277 - 316 | Other observations | C 40 |  | (84) |

### XX2009.D1G
Form to be used for submission of diameter measurements (girthband or dendrometer)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, tree, date, diameter, time, measurement_average, dendrometer, sensor_exchanged, other_observations

| Column | Description | Format | Ref_Tab | Item # |  |
| --- | --- | --- | --- | --- | --- |
| 1 - 6 | Sequence number (1-999999) | I 6 |  |  |  |
| 8 – 9 | Country code | I 2 | X | (1) |  |
| 11 – 14 | Plot Number | I 4 |  | (2) |  |
| 16 – 19 | Treenumber (as marked during installation; same as in .TRC; if not in TRC same as IPM-tree number; if not in IPM same as FOM) | I 4 |  | (19) |  |
| 21 – 26 | Date of Assessment | Date |  | (3) |  |
| 28 – 34 | Actual diameter (girthband or dendrometer measurement) or diameter of period [cm] | F 7 |  |  |  |
| 36 – 41 | Time of diameter measurement or reference time for which period diameter calculated [HHMMSS] (e.g. 094357) | Time |  |  |  |
| 43 | Measurement (code 1) or period value (2) | I 1 | X | (157) |  |
| 45 – 47 | Continuous dendrometer (point dentrometer code: 1.1; circumference dendrometer code: 1.2) or permanent girthband measurement (code: 2) | C 3 | X | (158) |  |
| 49 | Sensor was exchanged before this measurement or girthband adjusted [Yes=Y, No = N] | Y/N |  |  |  |
| 51 – 90 | Comments on diameter measurement | C 40 |  | (84) |  |

## Soil water

### XX2009.SWC
FutMon D3 Soil water content – sample definition and dry soil bulk density
one record/line/observation on each sample unit

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date, PFH_pit, horizon, SW_pit, depth_layer, ring_depth_upper, ring_depth_lower, replicate, bulk_density, date_analysis, other_observations

| columns | parameter | Format | Ref_Tab | Item# |
| --- | --- | --- | --- | --- |
| 1 - 4 | Sequence number (1-9999) | I 4 |  |  |
| 6 - 7 | Country code | I 2 | X | (1) |
| 9 - 12 | Plot Number | I 4 |  | (2) |
| 14 - 19 | Sampling date | Date |  | (3) |
| 21 – 24 | PFH pit ID (maximum 4 characters, same as in BioSoil) | C 4 |  | (69) |
| 26 – 27 | Horizon number | I 2 |  | (143) |
| 29 - 33 | SW pit ID | C 5 |  | (70) |
| 35 - 37 | Code depth layer | C 3 | X | (71) |
| 39 - 41 | Sample ring depth (upper side of ring) in cm below the top of the mineral soil; negative values for sampling rings taken in organic layer. | I 3 |  | (71) |
| 43 - 45 | Sample ring depth (lower side of ring) in cm below the top of the mineral soil; so negative values for sampling rings taken in organic layer. | I 3 |  | (71) |
| 47 - 47 | Replicate (in case multiple samples are taken in one SW pit: 1, 2, 3…) | I 1 |  | (71) |
| 49 – 52 | Dry soil bulk density of the fine earth (kg m-3) | IF 4 |  |  |
| 54 – 59 | Date laboratory analysis (DDMMYY) | Date |  | (3) |
| 61 - 100 | Other observations (text) | C 40 |  | (84) |

### XX2009.SWA
FutMon D3 Soil water content – sample analysis results on water retention

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date, SW_pit, depth_layer, replicate, water_content, matric_pressure, date_analysis, other_observations

| columns | parameter | Format | Ref_Tab | Item# |
| --- | --- | --- | --- | --- |
| 1 - 5 | Sequence number (1-99999) | I 5 |  |  |
| 7 - 8 | Country code | I 2 | X | (1) |
| 10 - 13 | Plot Number | I 4 |  | (2) |
| 15 - 20 | Sampling date | Date |  | (3) |
| 22 - 25 | SW pit ID (maximum 4 characters) | C 4 |  | (70) |
| 27 - 29 | Code depth layer | C 3 | X | (71) |
| 31 - 31 | Replicate (in case multiple samples are taken in one SW pit: 1, 2, 3…) | I 1 |  | (71) |
| 33 – 38 | Volumetric water content in m3.m-3 at matric pressure specified in field "matric pressure" | F 6 |  | (130) |
| 40 – 47 | Matric pressure [kPa]; e.g. value -5 indicating -5kPa; mandatory for new calculations under FutMon: 0kPa, -1kPa, -5kPa, -33kPa, -1500kPa | F 8 |  | (130) |
| 49 – 54 | Date laboratory analysis (DDMMYY) | Date |  | (3) |
| 56 – 95 | Other observations (text) | C 40 |  | (84) |

Part IIIa

## Sampling and Analysis of Soil

During the FutMon project we will rely on the BioSoil definitions and recommendations. Concerning the data submission the forms below will be used for data on plots which were not already submitted to the BioSoil data base.

Some amendments to BioSoil were made already and highlighted by blue bold font.

### XX2009.PLS
Contents of reduced plot file

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date, latitude, longitude, altitude, water, humus, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| l – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country Code | I 2 | X | (1) |
| 9 – 12 | Plot number (max. 9999) | I 4 |  | (2) |
| 14 – 19 | Date of sampling in DDMMYY (e.g. 220690) | Date |  | (3) |
| 21 – 27 | Latitude in +DDMMSS (e.g. + 505852) | C 7 |  | (4) |
| 29 – 35 | Longitude in (+ or -) DDMMSS (e.g. + 035531) | C 7 |  | (4) |
| 37 – 38 | Altitude (in 50 meter classes from 1 to 51) | I 2 | X | (7) |
| 40 – 66 | Soil unit | C 27 | X |  |
| 68 – 71 | Parent material code | I 4 | X |  |
| 73 | Water availability (insufficient = 1, sufficient = 2, excessive = 3) | I 1 | X | (5) |
| 75 | Humus type ( Mull = 1, Moder = 2, etc.) | I 1 | X | (6) |
| 77 – 78 | ID of soil profile used for the pedological characterisation (e.g. P04) | I 2 |  |  |
| 81 – 120 | Other observations | C 40 |  | (84) |

### XX2009.prf
Soil profile description

The information on the WR B soil classification name is not longer part of the PLS file. The same file can be used on Level I and Level II. Mainly based on BioSoil submission forms; amended parameter in blue bold font; Mandatory/Optional also picked from BioSoil; final discussion and definition after submission of the BioSoil database to FutMon data centre and FSCC.

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, profile_pit, date, latitude, longitude, elevation, soil_group, qualifier_1, specifier_1, qualifier_2, specifier_2, qualifier_3, specifier_3, qualifier_4, specifier_4, qualifier_5, specifier_5, qualifier_6, specifier_6, diagnostic_1, diagnostic_depth_1, diagnostic_2, diagnostic_depth_2, diagnostic_3, diagnostic_depth_3, diagnostic_4, diagnostic_depth_4, diagnostic_5, diagnostic_depth_5, diagnostic_6, diagnostic_depth_6, diagnostic_7, diagnostic_depth_7, diagnostic_8, diagnostic_depth_8, diagnostic_9, diagnostic_depth_9, diagnostic_10, diagnostic_depth_10, WRB_publication, parent_material_1, parent_material_2, ground_water_highest, ground_water_lowest, water_table, rooting_depth, rock_depth, obstacle_depth, other_observations

| Column | Description | Format | Ref_Tab | Item # | M/O |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 - 4 | Sequence number of plots (1 to 9999) | I 4 |  |  | M |  |
| 6 - 7 | Country code | I 2 | X | (1) | M |  |
| 9 - 12 | Observation plot number (maximum 9999) | I 4 |  | (2) | M |  |
| 14 - 17 | Profile pit ID (maximum 4 characters) (same as used in BioSoil) | C 4 |  | (69) | M |  |
| 19 - 24 | Date profile description (DDMMYY) | Date |  | (3) | M |  |
| 26 - 32 | Latitude of profile pit (in +DDMMSS) | C 7 |  | (4) | M |  |
| 34 - 40 | Longitude of profile pit in (+/-DDMMSS) | C 7 |  | (4) | M |  |
| 42 - 45 | Elevation of profile pit in metres above sea level | I 4 |  |  | O |  |
| 47 - 48 | Code of WRB Reference Soil Group | C 2 | X | (131) | M |  |
| 50 - 51 | WRB Qualifier 1 (1 = most important) | C 2 | X | (132) | O |  |
| 53 - 53 | WRB Specifier 1 | C 1 | X | (133) | O |  |
| 55 - 56 | WRB Qualifier 2 (2 = second most important,…) | C 2 | X | (132) | O |  |
| 58 - 58 | WRB Specifier 2 | C 1 | X | (133) | O |  |
| 60 - 61 | WRB Qualifier 3 | C 2 | X | (132) | O |  |
| 63 - 63 | WRB Specifier 3 | C 1 | X | (133) | O |  |
| 65 - 66 | WRB Qualifier 4 | C 2 | X | (132) | O |  |
| 68 - 68 | WRB Specifier 4 | C 1 | X | (133) | O |  |
| 70 - 71 | WRB Qualifier 5 | C 2 | X | (132) | O |  |
| 73 - 73 | WRB Specifier 5 | C 1 | X | (133) | O |  |
| 75 - 76 | WRB Qualifier 6 | C 2 | X | (132) | O |  |
| 78 - 78 | WRB Specifier 6 | C 1 | X | (133) | O |  |
| 80 - 82 | Diagnostic 1 | C 3 | X | (134) | O |  |
| 84 - 86 | Depth of appearance of diagnostic 1 | I 3 |  | (135) | O |  |
| 88 – 90 | Diagnostic 2 | C 3 | X | (134) | O |  |
| 92 – 94 | Depth diagnostic 2 | I 3 |  | (135) | O |  |
| 96 - 98 | Diagnostic 3 | C 3 | X | (134) | O |  |
| 100 - 102 | Depth diagnostic 3 | I 3 |  | (135) | O |  |
| 104 - 106 | Diagnostic 4 | C 3 | X | (134) | O |  |
| 108 - 110 | Depth diagnostic 4 | I 3 |  | (135) | O |  |
| 112 - 114 | Diagnostic 5 | C 3 | X | (134) | O |  |
| 116 - 118 | Depth diagnostic 5 | I 3 |  | (135) | O |  |
| 120 - 122 | Diagnostic 6 | C 3 | X | (134) | O |  |
| 124 - 126 | Depth diagnostic 6 | I 3 |  | (135) | O |  |
| 128 - 130 | Diagnostic 7 | C 3 | X | (134) | O |  |
| 132 - 134 | Depth diagnostic 7 | I 3 |  | (135) | O |  |
| 136 - 138 | Diagnostic 8 | C 3 | X | (134) | O |  |
| 140 - 142 | Depth diagnostic 8 | I 3 |  | (135) | O |  |
| 144 - 146 | Diagnostic 9 | C 3 | X | (134) | O |  |
| 148 - 150 | Depth diagnostic 9 | I 3 |  | (135) | O |  |
| 152 - 154 | Diagnostic 10 | C 3 | X | (134) | O |  |
| 156 - 158 | Depth diagnostic 10 | I 3 |  | (135) | O |  |
| 160 - 163 | WRB publication code | C 4 | X | (136) | O |  |
| 165 - 168 | Parent Material Code 1 | I 4 | X | (73) | M |  |
| 170 - 173 | Parent Material Code 2 | I 4 | X | (73) | O |  |
| 175 - 175 | Mean highest ground water level | I 1 | X | (151) | M |  |
| 177 - 177 | Mean lowest ground water level | I 1 | X | (151) | M |  |
| 179 - 179 | Type of water table | I 1 | X | (152) | M |  |
| 181 - 183 | Effective rooting depth (in cm from mineral soil surface) | I 3 |  |  | M |  |
| 185 - 187 | Rock depth of the soil profile (in cm from mineral soil surface) | I 3 |  |  | M |  |
| 189 - 191 | Obstacle depth of the soil profile (in cm from mineral soil surface) | I 3 |  |  | M |  |
| 193 - 232 | Other observations (text) | C 40 |  | (84) | O |  |

### XX2009.PFH	
Soil profile horizons

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, profile_pit, horizon, date_analysis, hor_discontinuity, hor_master, hor_subordinate, hor_vertical, hor_upper, hor_lower, hor_distinctness, hor_topography, structure, colour_moist, colour_dry, hor_texture_class, hor_clay, hor_silt, hor_sand, hor_coarse_vol, hor_coarse_weight, hor_organic_carbon, hor_total_nitrogen, hor_total_CaCO3, hor_gypsum, hor_pH, hor_conductivity, hor_exchange_Ca, hor_exchange_Mg, hor_exchange_K, hor_exchange_Na, hor_exchange_cation_cap, hor_porosity, hor_bulk_density_measure, hor_bulk_density_estimate, roots_very_fine, roots_fine, roots_medium, roots_coarse, other_observations

| Column | Description | Format | Ref_Tab | Item # | M/O |
| --- | --- | --- | --- | --- | --- |
| 1 - 4 | Sequence number (maximum 9999) | I 4 |  |  | M |
| 6 - 7 | Country code | I 2 | X | (1) | M |
| 9 - 12 | Observation plot number (maximum 9999) | I 4 |  | (2) | M |
| 14 - 17 | Profile pit ID (maximum 4 characters) (same as used in BioSoil) | C 4 |  | (69) | M |
| 19 - 20 | Horizon number | I 2 |  | (143) | M |
| 22 - 27 | Date laboratory analysis (DDMMYY) | Date |  | (3) | M |
| 29 | Horizon_discontinuity | I 1 | X | (144) | M if exists |
| 31 - 33 | Horizon_master | C 3 | X | (144) | M |
| 35- 38 | Horizon_subordinate | C 4 | X | (144) | M if exists |
| 40 | Horizon_vertical | I 1 |  | (144) | M if exists |
| 42 - 44 | Upper horizon limit (in cm) | I 3 |  | (145) | M |
| 46 - 48 | Lower horizon limit (in cm) | I 3 |  | (145) | M |
| 50 | Horizon distinctness | I 1 | X | (146) | O |
| 52 | Horizon topography | I 1 | X | (147) | O |
| 54 - 55 | Structure | I 2 | X | (148) | M for mineral hor. |
| 57 - 69 | Moist colour of the soil matrix (Munsell soil colour charts) | C 13 |  |  | M for mineral hor. |
| 71 - 83 | Dry colour of the soil matrix (Munsell soil colour charts) | C 13 |  |  | M for mineral hor. |
| 85 - 88 | Horizon Textural class | C 4 | X | (68) | M for mineral hor. |
| 90 - 93 | Horizon Clay (0 – 2 micrometer fraction) (%) | F 4 |  |  | M for mineral hor. |
| 95 - 98 | Horizon Silt (2 – 63 micrometer fraction) (%) | F 4 |  |  | M for mineral hor. |
| 100 - 103 | Horizon Sand (63 – 2000 micrometer fraction) (%) | F 4 |  |  | M for mineral hor. |
| 105 | Horizon code coarse fragments (code based on volume %) | I 1 | X | (149) | M for mineral hor. |
| 107 - 109 | Horizon coarse fragments (weight % in g/100g) | I 3 |  |  | O for mineral hor. |
| 111 - 115 | Horizon Total Organic Carbon content (g/kg) | F 5 |  |  | M |
| 117 - 121 | Horizon Total Nitrogen (g/kg) | F 5 |  |  | M |
| 123 - 125 | Horizon Total Calcium Carbonate (g/kg) | F 3 |  |  | M if present |
| 127 - 129 | Horizon Gypsum content (g/kg) | F 3 |  |  | O |
| 131 - 134 | Horizon pH | F 4 |  |  | M |
| 136 - 139 | Horizon Electrical conductivity (dS.m-1) | F 4 |  |  | O |
| 141 - 146 | Horizon Exchangeable Ca (cmol(+)/kg) | F 6 |  |  | M |
| 148 - 153 | Horizon Exchangeable Mg (cmol(+)/kg) | F 6 |  |  | M |
| 155 – 160 | Horizon Exchangeable K (cmol(+)/kg) | F 6 |  |  | M |
| 162 - 167 | Horizon Exchangeable Na (cmol(+)/kg) | F 6 |  |  | M |
| 169 - 174 | Horizon Cation Exchange Capacity (cmol(+)/kg) | F 6 |  |  | M for mineral hor. |
| 176 | Horizon Code Porosity | I 1 | X | (150) | M |
| 178 - 181 | Horizon Measured Bulk Density (in kg/m3) | F 4 |  |  | M |
| 183 - 186 | Horizon Estimated Bulk Density (in kg/m3) | F 4 |  |  | M if not measured |
| 188 | Very fine roots: abundance class | I 1 | X | (153) | O |
| 190 | Fine roots: abundance class | I 1 | X | (153) | O |
| 192 | Medium roots: abundance class | I 1 | X | (153) | O |
| 194 | Coarse roots: abundance class | I 1 | X | (153) | O |
| 196 - 245 | Other observations | C 50 |  | (84) | O |

### XX2006XX2009.SOM
Contents of datafile with soil analysis information (Mandatory and optional as required by the BioSoil project) 
Level I + Level II 
Concerning the distinction between mandatory and optional parameters the definition used in the ICP Forests manual (version from 2006, also applied during the BioSoil project) is still valid. Mandatory data as well as optional data will be submitted during FutMon using this form .SOM:

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, layer, repetition, layer_superior, layer_inferior, subsamples, date_analysis, moisture_content, clay, silt, sand, texture, bulk_density_dry_fine_earth, coarse_fragments_vol, organic_layer_dry_weight, pH_CaCl, pH_H2O, total_organic_carbon, total_nitrogen, carbonates, exchange_acidity, exchange_Al, exchange_Ca, exchange_Fe, exchange_K, exchange_Mg, exchange_Mn, exchange_Na, Free_H_acidity, extract_Al, extract_Ca, extract_Cd, extract_Cr, extract_Cu, extract_Fe, extract_Hg, extract_K, extract_Mg, extract_Mn, extract_Na, extract_Ni, extract_P, extract_Pb, extract_S, extract_Zn, total_Al, total_Ca, total_Fe, total_K, total_Mg, total_Mn, total_Na, reactive_Al, reactive_Fe, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number | I 5 |  |  |
| 7 – 8 | Country code | I 2 | X | (1) |
| 10 - 13 | Observation plot number | I 4 |  | (2) |
| 15 - 17 | Code layer | C 3 | X | (72) |
| 19 - 20 | Repetition (New) | I 2 |  | (140) |
| 22 - 25 | Layer limit superior | F 4 |  | (141) |
| 27 - 30 | Layer limit inferior | F 4 |  | (141) |
| 32 - 33 | Subsamples (N° in the composite) | I 2 |  | (142) |
| 35 - 40 | Date laboratory analysis (DDMMYY) of most recent analysis on the concerning sample | Date |  | (3) |
| 42 - 45 | Moisture content (difference between air dry and oven dry moisture) in % | F 4 |  |  |
| 47 - 50 | Clay (0 – 2 micrometer fraction) (%) | F 4 |  |  |
| 52 – 55 | Silt (2 – 63 micrometer fraction) (%) | F 4 |  |  |
| 57 - 60 | Sand (63 – 2000 micrometer fraction) (%) | F 4 |  |  |
| 62 - 65 | Texture class | C 4 | X | (68) |
| 67 - 70 | Mean dry bulk density of the fine earth in kg/m3 | F 4 |  |  |
| 72 - 74 | Volume coarse fragments (volume %) | I 3 |  |  |
| 76 - 80 | Total dry weight of the organic layer (kg/m2) | F 5 |  |  |
| 82 - 85 | pH(CaCl2) | F 4 |  |  |
| 87 - 90 | pH(H2O) | F 4 |  |  |
| 92 - 96 | Total Organic Carbon (g/kg) | F 5 |  |  |
| 98 - 102 | Total Nitrogen (g/kg) | F 5 |  |  |
| 104 - 106 | Carbonates (g/kg CaCO3) | F 3 |  |  |
| 108 - 113 | Exchangeable acidity (cmol(+)/kg) | F 6 |  |  |
| 115 - 120 | Exchangeable Al (cmol(+)/kg) | F 6 |  |  |
| 122 - 127 | Exchangeable Ca (cmol(+)/kg) | F 6 |  |  |
| 129 - 134 | Exchangeable Fe (cmol(+)/kg) | F 6 |  |  |
| 136 - 141 | Exchangeable K (cmol(+)/kg) | F 6 |  |  |
| 143 - 148 | Exchangeable Mg (cmol(+)/kg) | F 6 |  |  |
| 150 - 155 | Exchangeable Mn (cmol(+)/kg) | F 6 |  |  |
| 157 - 162 | Exchangeable Na (cmol(+)/kg) | F 6 |  |  |
| 164 - 169 | Free H+ acidity (cmol(+)/kg) | F 6 |  |  |
| 171 - 178 | Extractable Al (mg/kg) | F 8 |  |  |
| 180 - 187 | Extractable Ca (mg/kg) | F 8 |  |  |
| 189 - 192 | Extractable Cd (mg/kg) | F 4 |  |  |
| 194 - 198 | Extractable Cr (mg/kg) | F 5 |  |  |
| 200 - 206 | Extractable Cu (mg/kg) | F 7 |  |  |
| 208 - 215 | Extractable Fe (mg/kg) | F 8 |  |  |
| 217 - 222 | Extractable Hg (mg/kg) | F 6 |  |  |
| 224 - 230 | Extractable K (mg/kg) | F 7 |  |  |
| 232 - 239 | Extractable Mg (mg/kg) | F 8 |  |  |
| 241 - 247 | Extractable Mn (mg/kg) | F 7 |  |  |
| 249 - 254 | Extractable Na (mg/kg) | F 6 |  |  |
| 256 - 260 | Extractable Ni (mg/kg) | F 5 |  |  |
| 262 - 267 | Extractable P (mg/kg) | F 6 |  |  |
| 269 - 274 | Extractable Pb (mg/kg) | F 6 |  |  |
| 276 - 281 | Extractable S (mg/kg) | F 6 |  |  |
| 283 - 288 | Extractable Zn (mg/kg) | F 6 |  |  |
| 290 - 297 | Total Al (mg/kg) | F 8 |  |  |
| 299 - 306 | Total Ca (mg/kg) | F 8 |  |  |
| 308 - 315 | Total Fe (mg/kg) | F 8 |  |  |
| 317 - 324 | Total K (mg/kg) | F 8 |  |  |
| 326 - 333 | Total Mg (mg/kg) | F 8 |  |  |
| 335 - 342 | Total Mn (mg/kg) | F 8 |  |  |
| 344 - 351 | Total Na (mg/kg) | F 8 |  |  |
| 353 - 359 | Reactive Al (acid oxalate extractable Al) (mg/kg) | F 7 |  |  |
| 361 - 367 | Reactive Fe (acid oxalate extractable Fe) (mg/kg) | F 7 |  |  |
| 369 - 418 | Other observations | C 50 |  | (84) |
| 371 – 372 | Current interlaboratory comparison test number (eg. 08) | C 2 |  | (71) |
| 374 – 376 | Laboratory codenumber in this test (eg. 003 or 36a) | C 3 |  | (71) |

- Methods and recomputations that have been used shall be described in detail in the Data Accompanying Report.

### XX2009SO.LQA
Soil analysis – Laboratory QA/QC information

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date_start, date_end, parameter, extraction, sieving, removal_compounds, determination, quantification_limit, control_chart_mean, control_chart_std, ring_test_participation, ring_test_number, Laboratory_ID, percentage_within, requalification, percentage_within_requal, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 01 – 04 | Sequence number (1 to 9999) | I 4 |  |  |
| 06 – 07 | Country code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 09 – 12 | Observation Plot number (maximum 9999) | I 4 |  | (2) |
| 14 – 19 | start date | Date |  | (117) |
| 21 – 26 | end date | Date |  | (117) |
| 28 – 36 | Parameter Code* (N, S, Ca etc.) | C 9 | X | (113) |
| 38 – 41 | Digestion/Extraction method (pretreatment) | C 4 | X | (116) |
| 43 – 45 | Sieving/milling method | F 3 | X | (115) |
| 47 – 49 | code removal compounds | F 3 | X | (114) |
| 51 – 54 | Determination method (see reference list) | F 4 | X | (106) |
| 56 – 61 | Quantification limit (unit of parameter) | F 6 |  | (107) |
| 63 – 68 | Mean of control chart | F 6 |  | (107) |
| 70 – 72 | Standard deviation [% of mean] => coefficient of variation | F 3 |  | (107) |
| 74 | Participated at ring Test (yes = 1, no = 0) | I 1 |  | (107) |
| 76 – 78 | ICP Forests Ring Test Number | C 3 |  | (107) |
| 80 – 82 | ID of laboratory (e.g. H45, B78, etc.) | C 3 |  | (107) |
| 84 – 86 | Percentage [%] of the results of the ring tests within tolerable limits for each ring test | I 3 |  | (107) |
| 88 | Requalification information (yes = 1, no = 0) | I 1 |  | (107) |
| 90 – 92 | Percentage [%] of the results of the ring tests within tolerable limits for each ring test in requalification | I 3 |  | (107) |
| 94 – 133 | Other observations (freetext) | C 40 |  | (84) |

Part IIIb

## Soil Solution Collection and Analysis

### XX2006XX2009.PSS 
Contents of reduced plot file to be used in combination with the soil solution measurements

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, latitude, longitude, altitude, sampler, sampler_type, layer, depth, date_monitoring_1st, date_monitoring_last, periods, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country Code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 9 – 12 | Observation plot number | I 4 |  | (2) |
| 14 – 20 | Latitude in +DDMMSS (e.g.+505852) | C 7 |  | (4) |
| 22 – 28 | Longitude in (+ or -)DDMMSS (e.g. +035531) | C 7 |  | (4) |
| 30 – 31 | Altitude (in 50 meter classes from 1 to 51) | I 2 | X | (7) |
| 33 – 59 | Soil unit code blanks to be submitted from 2009 onwards | C 27 | X |  |
| 61 – 62 | Sampler Number (1 – 99) | I 2 |  | (74) |
| 64 | Sampler Type (1 = Tension lysimeter, 2 = Zero tension lysimeter, 3 = Centrifugation, 4 = Saturation extraction) | I 1 | X | (75) |
| 66 | Layer (H, O = Organic, M = Mineral) | C 1 | X | (72) |
| 68 – 72 | Sampling depth (in meters below surface) | F 5 |  | (76) |
| 74 – 79 | 1st period start date (DDMMYY) | Date |  | (53) |
| 81 – 86 | last period final date (DDMMYY) | Date |  | (53) |
| 88 – 89 | Number of (equal) monitoring periods | I 2 |  | (54) |
| 91 – 130 | Other observations (text) | C 40 |  | (84) |

### XX20069.SSM
Contents of datafile with soil solution measurements (mandatory)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, date_start, date_end, period, sampler, pH, conductivity, K, Ca, Mg, N_NO3, S_SO4, alkalinity, Al, DOC, other_observations

| Column | Description |  |  |  |  | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number of samples (1 to 99999) |  |  |  |  | I 5 |  |  |
| 7 – 10 | Observation plot number (max. 9999) |  |  |  |  | I 4 |  | (2) |
| 12 – 17 | Start date (DDMMYY) |  |  |  |  | Date |  | (53) |
| 19 – 24 | End date (DDMMYY) |  |  |  |  | Date |  | (53) |
| 26 – 27 | Period number (max. 99) |  |  |  |  | I 2 |  | (55) |
| 29 – 30 | Sampler number (see plot file) |  |  |  |  | I 2 |  | (74) |
| 32 – 34 | pH |  |  |  |  | F 3 |  |  |
| 36 – 39 | Conductivity |  |  | (S/cm) |  | F 4 |  |  |
| 41 – 45 | K |  |  | (mg/l) |  | F 5 |  |  |
| 47 – 51 | Ca |  |  | (mg/l) |  | F 5 |  |  |
| 53 – 57 | Mg |  |  | (mg/l) |  | F 5 |  |  |
| 59 – 63 | N-NO3 |  |  | (mg/l) |  | F 5 |  |  |
| 65 – 69 | S-SO4 |  |  | (mg/l) |  | F 5 |  |  |
| 71 – 74 | Alkalinity |  |  | (µmolc/l) |  | F 4 |  |  |
| 76 – 80 | Al**) |  |  | (mg/l) |  | F 5 |  |  |
| 82 – 86 | DOC |  |  | (mg/l) |  | F 5 |  |  |
| 88 – 127 | Observation Text |  |  |  |  | C 40 |  | (84) |

Methods and recomputations that have been used shall be described in detail in an annex to the Data Accompanying Report on soil solution.

**)	Mandatory if pH<5

### XX2006XX2009.SSO
Contents of datafile with soil solution measurements (Optional)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, date_start, date_end, period, sampler, water_vol, Na, Al_labile, Fe, Mn, P, N_NH4, Cl, Cr, Ni, Zn, Cu, Pb, Cd, Si, other_observations

| Column | Description |  | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number of samples (1 to 99999) |  | I 5 |  |  |
| 7 – 10 | Corresponding plot number (max. 9999) |  | I 4 |  | (2) |
| 12 – 17 | Start date (DDMMYY) |  | Date |  | (53) |
| 19 – 24 | End date (DDMMYY) |  | Date |  | (53) |
| 26 – 27 | Period number (max. 99) |  | I 2 |  | (55) |
| 29 – 30 | Sampler number (see plot file) |  | I 2 |  | (74) |
| 32 – 35 | Water content (extraction only) (cm3/cm3) |  | F 4 |  |  |
| 37 – 41 | Na | (mg/l) | F 5 |  |  |
| 43 – 47 | Al-labile | (mg/l) | F 5 |  |  |
| 49 – 53 | Fe | (mg/l) | F 5 |  |  |
| 55 – 59 | Mn | (mg/l) | F 5 |  |  |
| 61 – 65 | P | (mg/l) | F 5 |  |  |
| 67 – 71 | N-NH4 | (mg/l) | F 5 |  |  |
| 73 – 77 | Cl | (mg/l) | F 5 |  |  |
| 79 – 83 | Cr | (µg/l) | F 5 |  |  |
| 85 – 89 | Ni | (µg/l) | F 5 |  |  |
| 91 – 94 | Zn | (µg/l) | F 4 |  |  |
| 96 – 99 | Cu | (µg/l) | F 4 |  |  |
| 101 – 104 | Pb | (µg/l) | F 4 |  |  |
| 106 – 110 | Cd | (µg/l) | F 5 |  |  |
| 112 – 116 | Si | (mg/l) | F 5 |  |  |
| 118 – 119 | Current interlaboratory comparison test number (eg. 08) |  | C 2 |  | (71) |
| 121 – 123 | Laboratory codenumber in this test (eg. 003 or 36a) |  | C 3 |  | (71) |
| 118 – 157 | Observations | Text | C 40 |  | (84) |

Methods and recomputations that have been used shall be described in detail in an annex to the Data Accompanying Report on soil solution.

### XX2009SS.LQA
Soil Solution – Laboratory QA/QC information

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date_start, date_end, parameter, determination, quantification_limit, control_chart_mean, control_chart_std, ring_test_participation, ring_test_number, Laboratory_ID, percentage_within, requalification, percentage_within_requal, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 01 – 04 | Sequence number (1 to 9999) | I 4 |  |  |
| 06 – 07 | Country code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 09 – 12 | Observation Plot number (maximum 9999) | I 4 |  | (2) |
| 14 – 19 | start date | Date |  | (117) |
| 21 – 26 | end date | Date |  | (117) |
| 28 – 36 | Parameter Code (Ca, Mg, etc.) | C 9 | X | (109) |
| 38 – 41 | Determination method (see reference list) | F 4 | X | (106) |
| 43 – 48 | Quantification limit (unit of parameter) | F 6 |  |  |
| 50 – 55 | Mean of control chart | F 6 |  | (107) |
| 57 – 59 | Standard deviation [% of mean] => coefficient of variation | F 3 |  | (107) |
| 61 | Participated at ring Test (yes = 1, no = 0) | I 1 |  | (107) |
| 63 – 65 | ICP Forests Ring Test Number | C 3 |  | (107) |
| 67 – 69 | ID of laboratory (e.g. H45, B78, etc.) | C 3 |  | (107) |
| 71 – 73 | Percentage [%] of the results of the ring tests within tolerable limits for each ring test | I 3 |  | (107) |
| 75 | Requalification information (yes = 1, no = 0) | I 1 |  | (107) |
| 77 – 79 | Percentage [%] of the results of the ring tests within tolerable limits for each ring test in requalification | I 3 |  | (107) |
| 81 – 120 | Other observations (freetext) | C 40 |  | (84) |

Part IV

## Sampling and Analysis of Needles and Leaves

### XX2005.PLF
Contents of reduced plot file to be used in combination with the survey of chemical content of needles and leaves

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date, latitude, longitude, altitude, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country Code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 9 – 12 | Plotnumber (max. 9999) | I 4 |  | (2) |
| 14 – 19 | Date of sampling in DDMMYY (e.g. 220690) | Date |  | (3) |
| 21 – 27 | Latitude in +DDMMSS (e.g.+505852) | C 7 |  | (4) |
| 29 – 35 | Longitude in (+ or -)DDMMSS (e.g. +035531) | C 7 |  | (4) |
| 37 – 38 | Altitude (in 50 meter classes from 1 to 51) | I 2 | X | (7) |
| 40 – 79 | Other observations (text) | C 40 |  | (84) |

### XX2005XX2009.FOM
Contents of file with foliar analysis information (mandatory)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, sample, date_analysis, tree_1_6, tree_2_7, tree_3_8, tree4_9, tree_5_10, mass_leaves, mass_needles, N, S, P, Ca, Mg, K, other_observations

| Column | Description |  | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- | --- |
| 1 – 5 | Sequence Number (1 to 99999) |  | I 5 |  |  |
| 7 – 10 | Observation Plot number (max. 9999) |  | I 4 |  | (2) |
| 12-16 | Sample number: | Tree species code a dot (“.”) and leaves type (0 = current, 1 = current + 1, 2 = older than current + 1) digit 4 is a dot (“.”) Examples: 134.2 or 020.1 | C 5 | X X | (20) (38) |
| 18-23 | Date of analysis (ddmmyy) |  | Date |  | (3) |
| 25-28 | Number of first tree in sample |  | C 4 |  | (39) |
| 30-33 | Number of second tree in sample |  | C 4 |  | (39) |
| 35-38 | Number of third tree in sample |  | C 4 |  | (39) |
| 40-43 | Number of fourth tree in sample |  | C 4 |  | (39) |
| 45-48 | Number of fifth tree in sample |  | C 4 |  | (39) |
| 50-53 | Mass of 100 leaves | Dry mass of 100 current year leaves [g] | F 4 |  | (41) |
| 55-58 | Mass of 1000 needles | Dry mass of 1000 current year or 1000 current+1 year needles or 1000 needles older than current+1 year[g] | F 4 |  | (41) |
|  |  | By reference at 105° dried material: |  |  |  |
| 60-64 | N | mg/g (99.99) | F 5 |  |  |
| 66-70 | S | mg/g (99.99) | F 5 |  |  |
| 72-75 | P | mg/g (9.99) | F 4 |  |  |
| 77-81 | Ca | mg/g (99.99) | F 5 |  |  |
| 83-87 | Mg | mg/g (99.99) | F 5 |  |  |
| 89-93 | K | mg/g (99.99) | F 5 |  |  |
| 95 – 134 | other observations (text) |  | C 40 |  | (84) |

### XX2005XX2009.FOO
Contents of file with foliar analysis information (optional)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, sample, date_analysis, C, Zn, Mn, Fe, Cu, Pb, Cd, B, other_observations

| Column | Description |  | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- | --- |
| 1 – 5 | Sequence Number (1 to 99999) |  | I 5 |  |  |
| 7 – 10 | Observation Plot number (max. 9999) |  | I 4 |  | (2) |
| 12 – 16 | Sample number: Tree species, a dot (“.”) and leaves type (0 = current, 1 = current + 1, 2 = older than current + 1)) digit 4 is a dot (“.”) Examples: 134.2 or 020.1 |  | C 5 | X X | (20) (38) |
| 18 – 23 | Date of analysis (ddmmyy) |  | Date |  | (3) |
|  |  | By reference at 105° dried material: |  |  |  |
| 25 – 29 | C | g/100g (99.99) | F 5 |  |  |
| 31 – 35 | Zn | µg/g (99999) | F 5 |  |  |
| 37 – 42 | Mn | µg/g (999999) | F 6 |  |  |
| 44 – 48 | Fe | µg/g (99999) | F 5 |  |  |
| 50 – 54 | Cu | µg/g (99.99) | F 5 |  |  |
| 56 – 60 | Pb | µg/g (99.99) | F 5 |  |  |
| 62 – 66 | Cd | ng/g (99999) | F 5 |  |  |
| 68 – 72 | B | µg/g (999.9) | F 5 |  |  |
| 74 – 75 | Current interlaboratory comparison test number (e.g. 08) |  | C 2 |  | (71) |
| 77 – 79 | Laboratory codenumber in this test (e.g. 003 or 36a) |  | C 3 |  | (71) |
| 74 – 113 | other observations (text) |  | C 40 |  | (84) |

### XX2009FO.LQA
Foliar analysis – Laboratory QA/QC information

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date_start, date_end, parameter, pretreatment, determination, quantification_limit, control_chart_mean, control_chart_std, ring_test_participation, ring_test_number, Laboratory_ID, percentage_within, requalification, percentage_within_requal, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 01 – 04 | Sequence number (1 to 9999) | I 4 |  |  |
| 06 – 07 | Country code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 09 – 12 | Observation Plot number (maximum 9999) | I 4 |  | (2) |
| 14 – 19 | start date | Date |  | (117) |
| 21 – 26 | end date | Date |  | (117) |
| 28 – 29 | Parameter Code* (N, S, Ca etc.) | C 2 | X | (108) |
| 31 – 34 | Pretreatment method (see reference list) | F 4 | X | (105) |
| 36 – 39 | Determination method (see reference list) | F 4 | X | (106) |
| 41 – 46 | Quantification limit (unit of parameter) | F 6 |  | (107) |
| 48 – 53 | Mean of control chart | F 6 |  | (107) |
| 55 – 57 | Standard deviation [% of mean] => coefficient of variation | F 3 |  | (107) |
| 59 | Participated at ring Test (yes = 1, no = 0) | I 1 |  | (107) |
| 61 – 63 | ICP Forests Ring Test Number | C 3 |  | (107) |
| 65 – 67 | ID of laboratory (e.g. H45, B78, etc.) | C 3 |  | (107) |
| 69 – 71 | Percentage [%] of the results of the ring tests within tolerable limits for each ring test | I 3 |  | (107) |
| 73 | Requalification information (yes = 1, no = 0) | I 1 |  | (107) |
| 75 – 77 | Percentage [%] of the results of the ring tests within tolerable limits for each ring test in requalification | I 3 |  | (107) |
| 79 – 118 | Other observations (text) | C 40 |  | (84) |

## Ground Vegetation Biomass and Nutrients Analyses

### XX2009.PGB
Contents of reduced plot file to be used in combination with the survey of chemical content of ground vegetation

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date, latitude, longitude, altitude, frame_area, frames, sampled_area, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country Code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 9 – 12 | Plotnumber (max. 9999) | I 4 |  | (2) |
| 14 – 19 | Date of sampling in DDMMYY (e.g. 220690) | Date |  | (3) |
| 21 – 27 | Latitude in +DDMMSS (e.g.+505852) | C 7 |  | (4) |
| 29 – 35 | Longitude in (+ or -)DDMMSS (e.g. +035531) | C 7 |  | (4) |
| 37 – 38 | Altitude (in 50 meter classes from 1 to 51) | I 2 | X | (7) |
| 40 – 43 | Frame area [m²] | F 4 |  |  |
| 45 – 46 | No of frames | I 2 |  |  |
| 48 – 52 | Sampled area [m²] | F 5 |  |  |
| 54 – 93 | Other observations (text) | C 40 |  | (84) |

### XX2009.GBM
Contents of file with ground vegetation analysis information (mandatory)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, sample, date_analysis, mass, N, S, P, Ca, Mg, K, C, other_observations

| Column | Description |  | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- | --- |
| 1 – 5 | Sequence Number (1 to 99999) |  | I 5 |  |  |
| 7 – 10 | Observation Plot number (max. 9999) |  | I 4 |  | (2) |
| 12 – 13 | Sample number: | Functional group | C 2 | X | (155) |
| 15 – 20 | Date of analysis (ddmmyy) |  | Date |  | (3) |
| 22 – 26 | Mass of the sample [g] |  | F 5 |  |  |
|  | By reference at 105° dried material: |  |  |  |  |
| 28 – 32 | N [mg/g] |  | F 5 |  |  |
| 34 – 38 | S [mg/g] |  | F 5 |  |  |
| 40 – 43 | P [mg/g] |  | F 4 |  |  |
| 45 – 49 | Ca [mg/g] |  | F 5 |  |  |
| 51 – 55 | Mg [mg/g] |  | F 5 |  |  |
| 57 – 61 | K [mg/g] |  | F 5 |  |  |
| 63 – 67 | C [g/100g] |  | F 5 |  |  |
| 69 – 108 | other observations (text) |  | C 40 |  | (84) |

all parameters by reference at 105° dried material

### XX2009.GBO 
Contents of file with ground vegetation analysis information (optional)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, sample, date_analysis, Zn, Mn, Fe, Cu, Pb, Cd, B, other_observations

| Column | Description |  | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- | --- |
| 1 – 5 | Sequence Number (1 to 99999) |  | I 5 |  |  |
| 7 – 10 | Observation Plot number (max. 9999) |  | I 4 |  | (2) |
| 12 – 13 | Sample number: | Functional group | C 2 | X | (155) |
| 15 – 20 | Date of analysis (ddmmyy) |  | Date |  | (3) |
|  | By reference at 105° dried material: |  |  |  |  |
| 22 – 26 | Zn [μg/g] |  | F 5 |  |  |
| 28 – 32 | Mn [μg/g] |  | F 5 |  |  |
| 34 – 39 | Fe [μg/g] |  | F 6 |  |  |
| 41 – 45 | Cu [μg/g] |  | F 5 |  |  |
| 47 – 51 | Pb [μg/g] |  | F 5 |  |  |
| 53 – 57 | Cd [ng/g] |  | F 5 |  |  |
| 59 – 63 | B [μg/g] |  | F 5 |  |  |
| 65 – 104 | other observations (text) |  | C 40 |  | (84) |

all parameters by reference at 105° dried material

### XX2009.GBH
form to be used for submission of height of sampled species (at least 5 most abundant) within frames

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, sample, date_analysis, species, height, other_observations

| Column | Description |  | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- | --- |
| 1 – 5 | Sequence Number (1 to 99999) |  | I 5 |  |  |
| 7 – 10 | Observation Plot number (max. 9999) |  | I 4 |  | (2) |
| 12 – 13 | Sample number: | Functional group | C 2 | X | (155) |
| 15 – 20 | Date of analysis (ddmmyy) |  | Date |  | (3) |
| 22 – 32 | Species code (Ground Vegetation Suvey) |  | C 11 | X | (82) |
| 34 – 37 | Height of species [cm] |  | F 4 |  |  |
| 39 – 78 | Other observations (text) |  | C 40 |  | (84) |

### XX2009GB.LQA
Ground Vegetation Biomass – Laboratory QA/QC information

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date_start, date_end, parameter, pretreatment, determination, quantification_limit, control_chart_mean, control_chart_std, ring_test_participation, ring_test_number, Laboratory_ID, percentage_within, requalification, percentage_within_requal, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 01 – 04 | Sequence number (1 to 9999) | I 4 |  |  |
| 06 – 07 | Country code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 09 – 12 | Observation Plot number (maximum 9999) | I 4 |  | (2) |
| 14 – 19 | start date | Date |  | (117) |
| 21 – 26 | end date | Date |  | (117) |
| 28 – 29 | Parameter Code* (N, S, Ca etc.) | C 2 | X | (108) |
| 31 – 34 | Pretreatment method (see reference list) | F 4 | X | (105) |
| 36 – 39 | Determination method (see reference list) | F 4 | X | (106) |
| 41 – 46 | Quantification limit (unit of parameter) | F 6 |  | (107) |
| 48 – 53 | Mean of control chart | F 6 |  | (107) |
| 55 – 57 | Standard deviation [% of mean] => coefficient of variation | F 3 |  | (107) |
| 59 | Participated at ring Test (yes = 1, no = 0) | I 1 |  | (107) |
| 61 – 63 | ICP Forests Ring Test Number (Needle/Leaf Interlaboratory Test) | C 3 |  | (107) |
| 65 – 67 | ID of laboratory (e.g. H45, B78, etc.) | C 3 |  | (107) |
| 69 – 71 | Percentage [%] of the results of the ring tests within tolerable limits for each ring test | I 3 |  | (107) |
| 73 | Requalification information (yes = 1, no = 0) | I 1 |  | (107) |
| 75 – 77 | Percentage [%] of the results of the ring tests within tolerable limits for each ring test in requalification | I 3 |  | (107) |
| 79 – 118 | Other observations (freetext) | C 40 |  | (84) |

Part V

## Assessment of Growth and Increment

### XX2009.PLI
Contents of reduced plotfile to be used for increment

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date, latitude, longitude, sample_plot_size, trees_growth, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 9 – 12 | Plot number (maximum 9999) | I 4 |  | (2) |
| 14 – 19 | Date of observation in DDMMYY (e.g. 220694) | Date |  | (3) |
| 21 – 27 | Latitude in +DDMMSS (e.g. + 501027) | C 7 |  | (4) |
| 29 – 35 | Longitude in + or –DDMMSS (e.g. – 011532) | C 7 |  | (4) |
| 37 – 42 | Skipped (please leave those digits empty): Total plot size in hectares (maximum 9.9999 ha) | F 6 |  | (16) |
| 44 – 48 | Skipped (please leave those digits empty): Number of all trees in total plot | I 5 |  | (17) |
| 50 – 55 | Growth plot size in hectares | F 6 |  | (16) |
| 57 – 61 | Number of all standing trees (living and newly dead trees) in Growth plot. The total number of trees (shoots in coppice forests) in the growth plot. All trees (shoots) from 5 (3) cm (DBH) and more are counted. | I 5 |  |  |
| 63 – 102 | Other observations (text) | C 40 |  | (84) |

### XX2009.IPM
Contents of file with increment information – periodic measurements

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, tree, tree_species, diameter, diameter2, bark, height, volume, crown_base_height, crown_width, removal, other_observations

| Column | Description | Format | Ref_Tab | Item # |  |
| --- | --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number records (1 to 99999) | I 5 |  |  | Mandatory |
| 7 – 10 | plot number (maximum 9999) | I 4 |  | (2) | Mandatory |
| 12-15 | Tree number | C 4 |  | (19) | Mandatory |
| 17-19 | Tree Species code | I 3 | X | (20) | Mandatory |
| 21-25 | Diameter (maximum 999.9 cm) | F 5 |  | (42) | Mandatory |
| 27-31 | Diameter (maximum 999.9 cm) | F 5 |  | (42) | Mandatory+) |
| 33-35 | Bark (maximum 9.9 cm) | F 3 |  | (43) | Optional |
| 37-40 | Height rounded off to the nearest 0.1 meters (maximum 99.9 m) | F 4 |  | (44) | Optional |
| 42-47 | Tree volume (maximum 99.999 m3) | F 6 |  | (45) | Optional |
| 49-52 | Height to crown base rounded off to the nearest 0.1 meters (maximum 99.9 m) | F 4 |  | (46) | Mandatory ++) |
| 54-57 | Crownwidth rounded off to the nearest 0.1 meters (maximum 99.9 m) | F 4 |  | (47) | Optional |
| 59-60 | Mortality and removal code (01, 02, 03) | I 2 | X | (88) | Mandatory |
| 62-101 | Other observations (text) | C 40 |  | (84) | Optional |

+)   when calipers are used

++)  when tree height is measured and the crown base is visible

### XX2007.IRA
Contents of file of increment information – ring analysis and stemdisk analysis (optional)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, tree, tree_species, dbh, diameter, diameter_5, diameter_10, diameter_15, diameter_20, diameter_25, diameter_30, diameter_35, diameter_40, diameter_45, diameter_50, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number (1 to 9999) | I 4 |  |  |
| 6 – 9 | Plot number (maximum 9999) | I 4 |  | (2) |
| 11-14 | Tree number with initial R for Ring analysis and D for Disk sampling | C 4 |  | (39) |
| 16-18 | Tree Species code | I 3 | X | (20) |
| 20-24 | Actual DBH over bark | F 5 |  | (42) |
| 26-30 | Actual diameter under bark | F 5 |  | (48) |
| 32-36 | Diameter under bark 5 years ago | F 5 |  | (48) |
| 38-42 | Diameter under bark 10 years ago | F 5 |  | (48) |
| 44-48 | Diameter under bark 15 years ago | F 5 |  | (48) |
| 50-54 | Diameter under bark 20 years ago | F 5 |  | (48) |
| 56-60 | Diameter under bark 25 years ago | F 5 |  | (48) |
| 62-66 | Diameter under bark 30 years ago | F 5 |  | (48) |
| 68-72 | Diameter under bark 35 years ago | F 5 |  | (48) |
| 74-78 | Diameter under bark 40 years ago | F 5 |  | (48) |
| 80-84 | Diameter under bark 45 years ago | F 5 |  | (48) |
| 86-90 | Diameter under bark 50 years ago | F 5 |  | (48) |
| 92-131 | Other observations (text) | C 40 |  | (84) |

### XX2007.IEV
Contents of evaluated data on increment (optional)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, trees, basal_plot_per, volume_plot_per, basal_plot, volume_plot, thinning_5, basal_5, volume_5, thinning_10, basal_10, volume_10, thinning_15, basal_15, volume_15, thinning_20, basal_20, volume_20, thinning_25, basal_25, volume_25, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number | I 4 |  |  |
| 6 – 9 | Plot number (maximum 9999) | I 4 |  | (2) |
|  | data |  |  |  |
| 11 – 14 | Number of trees per plot (maximum 9999 trees per plot) | I 4 |  | (17) |
| 16 – 20 | Basal area per plot (maximum 999.9 m2/plot) | F 5 |  | (49) |
| 22 – 26 | Volume per plot (maximum 999.9 m3/plot) | F 5 |  | (50) |
|  | above: periodic measurements |  |  |  |
|  | below: evaluation of ring and stem disk samples |  |  |  |
| 28 – 32 | Basal area per plot (maximum 999.9 m2/plot) | F 5 |  | (49) |
| 34 – 38 | Volume per plot (maximum 999.9 m3/plot) | F 5 |  | (50) |
| 40 | Thinning between now and 5 years ago (yes = 1, no = 0) | I 1 |  | (51) |
| 42 – 46 | Basal area per plot 5 years ago (maximum 999.9 m2/plot) | F 5 |  | (49) |
| 48 – 52 | Volume per plot 5 years ago (maximum 999.9 m3/plot) | F 5 |  | (50) |
| 54 | Thinning between 5 and 10 years ago (yes = 1, no = 0) | I 1 |  | (51) |
| 56 – 60 | Basal area per plot 10 years ago (maximum 999.9 m2/plot) | F 5 |  | (49) |
| 62 – 66 | Volume per plot 10 years ago (maximum 999.9 m3/plot) | F 5 |  | (50) |
| 68 | Thinning between 10 and 15 years ago (yes = 1, no = 0) | I 1 |  | (51) |
| 70 – 74 | Basal area per plot 15 years ago (maximum 999.9 m2/plot) | F 5 |  | (49) |
| 76 – 80 | Volume per plot 15 years ago (maximum 999.9 m3/plot) | F 5 |  | (50) |
| 82 | Thinning between 15 and 20 years ago (yes = 1, no = 0) | I 1 |  | (51) |
| 84 – 88 | Basal area per plot 20 years ago (maximum 999.9 m2/plot) | F 5 |  | (49) |
| 90 – 94 | Volume per plot 20 years ago (maximum 999.9 m3/plot) | F 5 |  | (50) |
| 96 | Thinning between 20 and 25 years ago (yes = 1, no = 0) | I 1 |  | (51) |
| 98 – 102 | Basal area per plot 25 years ago (maximum 999.9 m2/plot) | F 5 |  | (49) |
| 104 – 108 | Volume per plot 25 years ago (maximum 999.9 m3/plot) | F 5 |  | (50) |
| 110 – 149 | Other observations (text) | C 40 |  | (84) |

### XX2007.INV

Contents of reduced plot file to be used to report the plot volumes

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, latitude, longitude, altitude, date, volume_stemwood_remaining, volume_stemwood_newlydead, volume_stemwood_removed, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1- 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country Code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 9 -12 | Plotnumber (max. 9999) | I 4 |  | (2) |
| 14 – 20 | Latitude in +DDMMSS (e.g.+505852) | C 7 |  | (4) |
| 22 – 28 | Longitude in (+ or -)DDMMSS (e.g. +035531) | C 7 |  | (4) |
| 30 – 31 | Altitude (in 50 meter classes from 1 to 51) | I 2 | X | (7) |
| 33 – 38 | Date of sampling in DDMMYY (e.g. 221199) | Date |  | (3) |
| 40 – 42 | Stemwood volume (remaining = alive trees only) (in m3/ha) | I 3 |  |  |
| 44 – 46 | Stemwood volume (newly dead) | I 3 |  |  |
| 48 – 50 | Stemwood volume (removed = alive in previous ass. trees missing since last inventory ) (in m3/ha) | I 3 |  |  |
| 52 – 91 | Other observations (text) | C 40 |  | (84) |

Part VI

## Sampling and Analysis of Deposition

### XX20062009.PLD
Contents of reduced plot file 
to be used in combination with the deposition measurements

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, sampler, latitude, longitude, altitude, date_monitoring_1st, date_monitoring_last, periods, sampler_model, sampler_height, sampler_surface, samplers, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country Code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 9 – 12 | Observation plot number | I 4 |  | (2) |
| 14 | Sampler code | I 1 | X | (52) |
| 16 – 22 | Latitude in +DDMMSS (e.g.+505852) | C 7 |  | (4) |
| 24 – 30 | Longitude in (+ or -)DDMMSS (e.g. +035531) | C 7 |  | (4) |
| 32 – 33 | Altitude (in 50 meter classes from 1 to 51) | I 2 | X | (7) |
| 35 – 40 | First date of monitoring period | Date |  | (53) |
| 42 – 47 | Final date of monitoring period | Date |  | (53) |
| 49 – 50 | Number of (equal) measuring periods | I 2 |  | (54) |
| 52 – 52 | Sampler model (1=national sampler, 2=harmonised samplers) | I 1 | X | (56) |
| 54 – 57 | Sampler Height [m] | F 4 |  | (57) |
| 59 – 63 | Sampler Surface [m²] | F 5 |  | (58) |
| 65 – 66 | N of samplers (number of used samplers) | I 2 |  |  |
| 68 – 107 | Other observations (text) | C 40 |  | (84) |

### XX20062009.DEM
Contents of datafile with deposition measurements (mandatory)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, date_start, date_end, period, sampler, V_sampling, quantity, pH, conductivity, K, Ca, Mg, Na, N_NH4, Cl, N_NO3, S_SO4, alkalinity, N_total, DOC, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number of samples (1 to 99999) | I 5 |  |  |
| 7 – 10 | Observation Plot number (max.9999) | I 4 |  | (2) |
| 12 – 17 | Start date (DDMMYY) | Date |  | (53) |
| 19 – 24 | End date (DDMMYY) | Date |  | (53) |
| 26 – 27 | Period number (max. 99) | I 2 |  | (55) |
| 29 | Sampler code | I 1 | X | (52) |
| 31 | V_sampling | I 1 | X | (59) |
| 33 – 36 | Quantity of the total collected sample expressed in mm (max. 9999) | F 4 |  | (87) |
| 38 – 40 | pH | F 3 |  |  |
| 42 – 45 | Conductivity (µS/cm) | F 4 |  |  |
| 47 – 50 | K (mg/l) | F 4 |  |  |
| 52 – 56 | Ca (mg/l) | F 5 |  |  |
| 58 – 61 | Mg (mg/l) | F 4 |  |  |
| 63 – 67 | Na (mg/l) | F 5 |  |  |
| 69 – 73 | N-NH4 (mg/l) | F 5 |  |  |
| 75 – 79 | Cl (mg/l) | F 5 |  |  |
| 81 – 85 | N-NO3 (mg/l) | F 5 |  |  |
| 87 – 91 | S-SO4 (mg/l) | F 5 |  |  |
| 93 – 96 | Alkalinity (µeq/l) | F 4 |  |  |
| 98 – 102 | N (total) (mg/l) | F 5 |  |  |
| 104 – 108 | DOC (Dissolved organic carbon) (mg/l) | F 5 |  |  |
| 108 – 109 | QA/QC information: current working ring test number (e.g. 02) | C 2 |  | (71) |
| 111 – 113 | QA/QC information: laboratory code number in this test (e.g. 033) | C 3 |  | (71) |
| 110 – 149 | Observation text (e.g. orientation, , liming ...) | C 40 |  | (84) |

*)	Methods and recomputations that have been used shall be described in detail in an annex to the deposition report.

### XX20062009.DEO
Contents of datafile with deposition measurements (Optional)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, date_start, date_end, period, sampler, V_sampling, quantity, Al, Mn, Fe, P_PO4, Cu, Zn, Hg, Pb, Co, Mo, Ni, Cd, S_total, C_total, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number of samples (1 to 99999) | I 5 |  |  |
| 7 – 10 | Observation Plot number (max.9999) | I 4 |  | (2) |
| 12 – 17 | Start date (DDMMYY) | Date |  | (53) |
| 19 – 24 | End date (DDMMYY) | Date |  | (53) |
| 26 – 27 | Period number (max. 99) | I 2 |  | (55) |
| 29 | Sampler code (1 = Throughfall, 2 = Bulk, 3 = Wet-only, 4 = Stemflow, 5= Fog, 6 = Fogfrozen) | I 1 | X | (52) |
| 31 | V_sampling | I 1 | X | (59) |
| 33 – 36 | Quantity of the total collected sample expressed in mm (max. 9999) | F 4 |  | (87) |
| 38 – 41 | Al3+ (µg/l) | F 4 |  |  |
| 43 – 47 | Mn2+(µg/l) | F 5 |  |  |
| 49 – 53 | Fe3+(µg/l) | F 5 |  |  |
| 55 – 58 | P-PO43- (mg/l) | F 4 |  |  |
| 60 – 63 | Cu (µg/l) | F 4 |  |  |
| 65 – 68 | Zn (µg/l) | F 4 |  |  |
| 70 – 73 | Hg (µg/l) | F 4 |  |  |
| 75 – 78 | Pb (µg/l) | F 4 |  |  |
| 80 – 83 | Co (µg/l) | F 4 |  |  |
| 85 – 88 | Mo (µg/l) | F 4 |  |  |
| 90 – 93 | Ni (µg/l) | F 4 |  |  |
| 95 – 98 | Cd (µg/l) | F 4 |  |  |
| 100 – 104 | Stotal (mg/l) | F 5 |  |  |
| 106 – 109 | Ctotal (mg/l) | F 4 |  |  |
| 122 – 123 | QA/QC information: current working ring test number (e.g. 02) | C 2 |  | (71) |
| 125 – 127 | QA/QC information: laboratory code number in this test (e.g. 033) | C 3 |  | (71) |
| 111 – 150 | Observation text (e.g. orientation, liming ...) | F 40 |  | (84) |

*)	Methods and recomputations that have been used shall be described in detail in an annex to the deposition report

### XX2009DP.LQA
Deposition – Laboratory QA/QC information

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date_start, date_end, parameter, pretreatment, determination, quantification_limit, control_chart_mean, control_chart_std, ring_test_participation, ring_test_number, Laboratory_ID, percentage_within, requalification, percentage_within_requal, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 01 – 04 | Sequence number (1 to 9999) | I 4 |  |  |
| 06 – 07 | Country code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 09 – 12 | Observation Plot number (maximum 9999) | I 4 |  | (2) |
| 14 – 19 | start date | Date |  | (117) |
| 21 – 26 | end date | Date |  | (117) |
| 28 – 34 | Parameter Code (K, Ca, etc.) | C 7 | X | (110) |
| 36 – 39 | Pretreatment method (see reference list) | F 4 | X | (105) |
| 41 – 44 | Determination method (see reference list) | F 4 | X | (106) |
| 46 – 51 | Quantification limit (unit of parameter) | F 6 |  | (107) |
| 53 – 58 | Mean of control chart | F 6 |  | (107) |
| 60 – 62 | Standard deviation [% of mean] => coefficient of variation | F 3 |  | (107) |
| 64 | Participated at ring Test (yes = 1, no = 0) | I 1 |  | (107) |
| 66 – 68 | ICP Forests Ring Test Number | C 3 |  | (107) |
| 70 – 72 | ID of laboratory (e.g. H45, B78, etc.) | C 3 |  | (107) |
| 74 – 76 | Percentage [%] of the results of the ring tests within tolerable limits for each ring test | I 3 |  | (107) |
| 78 | Requalification information (yes = 1, no = 0) | I 1 |  | (107) |
| 80 – 82 | Percentage [%] of the results of the ring tests within tolerable limits for each ring test in requalification | I 3 |  | (107) |
| 84 – 123 | Other observations (freetext) | C 40 |  | (84) |

Part VII

## Meteorological Measurements

### XX2009.PLM
Contents of reduced plot file 
to be used in combination with the meteorological measurements

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, location, latitude, longitude, altitude, variable, vertical_position, instrument, scanning, storing, profile_pit, date_monitoring_1st, date_monitoring_last, measuring_days, instrument_description, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country Code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 9 – 15 | Observation plot/instrument code (the code consists of the plotnumber and instrument sequence number) divided by a dot “.”. E.g.: 0156.03 | C 7 |  | (60) |
| 17 | Location (S = stand, F = open field in forest area, W = weather station, O = other) | C 1 | X | (61) |
| 19 – 25 | Latitude in +DDMMSS (e.g.+505852) | C 7 |  | (4) |
| 27 – 33 | Longitude in (+ or -)DDMMSS (e.g. +035531) | C 7 |  | (4) |
| 35 – 36 | Altitude (in 50 meter classes from 1 to 51) | I 2 | X | (7) |
| 38 – 39 | Variable (AT= air temp, ST = soil temp, PR= precipitation, etc.) | C 2 | X | (62) |
| 41 – 46 | Vertical position (in meters above(+) or below (-) the ground) | F 6 |  | (63) |
| 48 – 49 | Instrument code (10 = manual reading, 20 = mechanical recording, 30 = paper recording, etc.) | I 2 | X | (63) |
| 51 – 53 | Scanning interval in seconds (for automatic stations only) | F 3 |  | (63) |
| 55 – 58 | Storing interval in minutes (for automatic stations only) | F 4 |  | (63) |
| 60 – 63 | Profile pit ID (maximum 4 characters). Only for soil moisture measurements; as mentioned in X2009.SWC | C 4 |  | (70) |
| 65 – 70 | First date of monitoring period | Date |  | (3) |
| 72 – 77 | Final date of monitoring period | Date |  | (3) |
| 79 – 81 | Number of (measuring) days | I 3 |  |  |
| 83 – 94 | Description of instrument | C 12 |  | (63) |
| 96 – 135 | Other observations (text) | C 40 |  | (84) |

### XX1996XX2009.MEM
Contents of datafile with meteorological measurements

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, variable, date, mean_sum, min, max, completeness, origin, status, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 6 | Sequence number of samples (1 to 999999) | I 6 |  |  |
| 8 – 14 | Observation Plot/ instrument code Corresponding plot/instrument number (max. 9999.99) | C 7 |  | (60) |
| 16 – 17 | Variable code (PR, AT, RH, WS, WD, or SR) | C 2 | X | (62) |
| 19 – 24 | Date (in DDMMYY) | Date |  | (3) |
| 26 – 31 | Daily mean (e.g. temperature) or sum (precipitation) values | F 6 |  | (64) |
| 33 – 38 | Daily minimum value | F 6 |  | (64) |
| 40 – 45 | Daily maximum value | F 6 |  | (64) |
| 47 – 49 | Completeness of measurements over the day (in % of measurements that should have been recorded) | I 3 |  | (65) |
| 51 | Origin of data; if calculated, method must be specified in the DAR | I 1 | X | (66) |
| 53 | Status of data | I 1 | X | (67) |
| 55 – 94 | Observations (text) | C 40 |  | (84) |

### XX1996XX2009.MEO
Contents of datafile with meteorological measurements

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, variable, date, mean_sum, min, max, completeness, origin, status, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 6 | Sequence number of samples (1 to 999999) | I 6 |  |  |
| 8 – 14 | Observation Plot/ instrument code Corresponding plot/instrument number (max. 9999.99) | C 7 |  | (60) |
| 16 – 17 | Variable code (UR, TF, SF, ST, MP, WC, or others) | C 2 | X | (62) |
| 19 – 24 | Date (in DDMMYY) | Date |  | (3) |
| 26 – 31 | Daily mean (e.g. temperature) or sum (precipitation) values | F 6 |  | (64) |
| 33 – 38 | Daily minimum value | F 6 |  | (64) |
| 40 – 45 | Daily maximum value | F 6 |  | (64) |
| 47 – 49 | Completeness of measurements over the day (in % of measurements that should have been recorded) | I 3 |  | (65) |
| 51 | Origin of data; if calculated, method must be specified in the DAR | I 1 | X | (66) |
| 53 | Status of data | I 1 | X | (67) |
| 55 – 94 | Observations (text) | C 40 |  | (84) |

*)	Methods and recomputations that have been used shall be described in detail in an annex to the Data Accompanying Report on meteorology

Part VIII

## Assessment of Ground Vegetation

### XX2007.PLV
Contents of reduced plot file to be used in combination with the survey of ground vegetation

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, survey_nr, date, latitude, longitude, altitude, fence, sampled_area, tree_cover, shrub_height, shrub_cover, herb_height, herb_cover, mosses_cover, bare_soil_cover, litter_cover, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country Code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 9 – 12 | Plot number (max. 9999) | I 4 |  | (2) |
| 14 – 15 | Survey number (max 99) | I 2 |  | (77) |
| 17 – 22 | Date of sampling in DDMMYY (e.g. 220690) | Date |  | (3) |
| 24 – 30 | Latitude in +DDMMSS (e.g.+505852) | C 7 |  | (4) |
| 32 – 38 | Longitude in (+ or -)DDMMSS (e.g. +035531) | C 7 |  | (4) |
| 40 – 41 | Altitude (in 50 meter classes from 1 to 51) | I 2 | X | (7) |
| 43 | Fence (Yes = 1, No = 2) | I 1 | X | (78) |
| 45 – 48 | Total sampled area (in m2) | I 4 |  | (80) |
| 50 – 52 | Tree layer cover (in % of total area) | I 3 |  | (80) |
| 54 – 57 | Shrub layer height (in m) | F 4 |  | (80) |
| 59 – 62 | Shrub layer cover (in % of total area) | F 4 |  | (80) |
| 64 – 67 | Herb layer height (in m) | F 4 |  | (80) |
| 69 – 72 | Herb layer cover (in % of total area) | F 4 |  | (80) |
| 74 – 77 | Mosses cover (in % of total area) | F 4 |  | (80) |
| 79 – 82 | Bare soil cover (in % of total area) | F 4 |  | (80) |
| 84 – 87 | Litter cover (in % of total area) | F 4 |  | (80) |
| 89 –128 | Other observations (text) | C 40 |  | (84) |

For covers of shrub, herb, moss bare soil and litter layers: indicate a zero “0” if assessed but not present and leave a blank if not assessed!

### XX2007.VEM
Contents of datafile with ground vegetation assessments

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, survey_nr, species, layer, species_cover, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number of plots (1 to 99999) | I 5 |  |  |
| 7 – 10 | Plot number (max. 9999) | I 4 |  | (2) |
| 12 – 13 | Survey number | I 2 |  | (77) |
| 15 – 25 | Species code (see codelists in Annex) | C 11 | X | (82) |
| 27 | Layer (1 = tree, 2 = shrub, 3 = herb, 4 = moss) | I 1 | X | (81) |
| 29 – 33 | Cover of the species in the layer (in %) | F 5 |  | (83) |
| 35 – 76 | Other observations (text) | C 42 |  | (84) |

Part IX

## Phenological Observations

### XX2009.PHE 
Phenological phenomena (plot level – extensive)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, tree_species, event, date, score, other_observations

| Column | Description | Format | Ref_Tab |  | Item # |
| --- | --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number record (1 to 99999) | I 5 |  |  |  |
| 7 – 10 | Plot number (maximum 9999) | I 4 |  |  | (2) |
| 12 – 14 | Tree species code | I 3 | X |  | (20) |
| 16 | Event code | I 1 | X |  | (118) |
| 18 – 23 | Date of observation | Date |  |  | (3) |
| 25 - 27 | Score of the event | C 3 | X |  | (123) |
| 18 – 23 | Date before = last date that the event/phenomena was not yet visible or had not yet occurred | Date |  |  | (3) |
| 24 – 29 | Date after = first date of observation of completion or first date after the event had occurred | Date |  |  | (3) |
| Columns 31 – 49 only to be completed in case of damaging events (if event code in column 15 = 4 or 5) |  |  |  |  |  |
| 31 | Extent of the damage using the codes: | I 1 | X |  | See left |
|  | 2 Infrequent or slight |  |  |  |  |
|  | 3 Common or moderate |  |  |  |  |
|  | 4 Abundant or severe |  |  |  |  |
| 33 – 34 | Symptom (01,..., 22) | I 2 | X |  | (27) |
| 36 – 41 | Cause (e.g. 81001) | I 6 | X |  | (29) |
| 43 – 49 | Scientific name of cause (e.g. LOPHSED) | C 7 | X |  | (30) |
| 51 – 56 | Estimated starting date = estimated date based on informations outside the plot | Date |  |  | (3) |
| 58 – 63 | Estimated completion date = estimated date based on informations outside the plot | Date |  |  | (3) |
| 65 – 104 | Other observations (text) | C 40 |  |  | (84) |

### XX2009.PLP 
Form for registration of trees selected for intensive phenological monitoring

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, tree_species, date_installation, tree, visible_part, visible_from, vertical_from, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number record (1 to 99999) | I 5 |  |  |
| 7 – 10 | Plot number (maximum 9999) | I 4 |  | (2) |
| 12 – 14 | Tree species code | I 3 | X | (20) |
| 16 – 21 | Installation date in DDMMYY | Date |  | (119) |
| 23 – 26 | Tree number is the existing identification number on tree or newly given number preceeded by an M | C 4 |  | (19) |
| 28 – 29 | Codes for visible part crown (during all year): | I 2 | X | (120) |
| 31 | Codes for visible direction FROM where the crown is observed crown | I 1 | X | (121) |
| 33 | Code for vertical direction from where the observations are made | I 1 | X | (122) |
| 35 – 74 | Other observations (text) | C 40 |  | (84) |

### XX2009.PHI
Recording of phenological phenomena (tree level – intensive)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, tree, event, date, score, method, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number record (1 to 99999) | I 5 |  |  |
| 7 – 10 | Plot number (maximum 9999) | I 4 |  | (2) |
| 12 – 15 | Tree number | C 4 |  | (19) |
| 17 | Event code | I 1 | X | (118) |
| 19 – 24 | date of the observation | Date |  | (3) |
| 26 - 28 | score of the event | C 3 | X | (123) |
| 30 | Method used for making the observation | C 1 | X | (124) |
| 18 – 23 | Starting date = last date that the event/phenomena was not yet visible or had not yet occured | Date |  | (3) |
| 25 – 30 | Completion date = first date of observation of completion or first date after the event had occurred | Date |  | (3) |
| 32 – 33 | Specification of affected part (11, ..., 34) |  | X | (26) |
| 35 – 36 | Symptom (01,..., 22) |  | X | (27) |
| 38 – 43 | Cause (e.g. 81001) |  | X | (29) |
| 45 – 51 | Scientific name of cause (e.g. LOPHSED) |  | X | (30) |
| 53 | Extent (0,1,2,3,4,5,6,7) |  | X | (31) |
| 56 – 95 | Other observations (text) | C 40 |  | (84) |

### XX2009.PHD
Submission of information related to digital images and movies on phenological observations (tree level – intensive)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, tree, tree_species, event, file, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number record (1 to 99999) | I 5 |  |  |
| 7 – 10 | Plot number (maximum 9999) | I 4 |  | (2) |
| 12 – 15 | Tree number | C 4 |  | (19) |
| 17 – 19 | Tree species code | I 3 | X | (20) |
| 21 – 21 | Event code | I 1 | X | (118) |
| 23 – 50 | File name of image / movie consisting of country code, plot number, date of observation, plot and date specific sequence number | C 28 |  | (125) |
| 52 – 91 | other observations | C 40 |  | (84) |

Part X

## Monitoring of Air Quality

### XX2009.PAC
Form with plot info on the station with the active sampler(s)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, latitude, longitude, altitude, compound, sampler, inlet_height, date_monitoring_1st, hour_start, date_monitoring_last, hour_end, col, altitude_m, elevation_lowest2500, elevation_lowest5000, other_observations

| Column | Description | Format | Ref_Tab |  | Item # |
| --- | --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number | I 4 |  |  |  |
| 6 – 7 | Country code (France = 01, etc.) | I 2 | X |  | (1) |
| 9 – 12 | Plot number | I 4 |  |  |  |
| 14 – 18 | Number of active sampler AS001- AS999 (active sampler = ASxxx) |  |  |  |  |
| 20 – 22 | Compound air quality |  |  | (74) |  |
| 14 – 20 | Latitude in +DDMMSS | C 7 |  |  | (4) |
| 22 – 28 | Longitude in (+ or -) DDMMSS | C 7 |  |  | (4) |
| 30 – 31 | Altitude (in 50 m classes 1-51) | I 2 | X |  | (7) |
| 33 – 35 | Compound air quality | C 3 | X |  | (89) |
| 37 – 38 | Sampler ID | I 2 |  |  | (92) |
| 40 – 43 | Inlet height (in m; accuracy 0.1m) | F 4 |  |  |  |
| 45 – 50 | Start date measurements (DDMMYY) | Date |  |  | (3) |
| 52 – 53 | Start hour (HH) | I 2 |  |  | (94) |
| 55 – 60 | End date measurements (DDMMYY) | Date |  |  | (3) |
| 62 – 63 | End hour (HH) | I 2 |  |  | (95) |
| 65-65 | Continuous analyzers co-located with passive samplers (Y/N) | Y/N |  |  | (96) |
| 67-70 | Altitude (in m)* | I 4 |  |  |  |
| 72-75 | Lowest elevation in a circular area of 2.5 km radius* (in 9999 m) | I 4 |  |  | (97) |
| 77-80 | Lowest elevation in a circular area of 5.0 km radius* (in 9999 m) | I 4 |  |  | (97) |
| 82 – 121 | Other observations (text) | C 40 |  |  | (84) |
| *Only for ozone measurements Note: For continuous analyzers: If for a given pollutant and plot measurements are taken with more than one continuous analyzer, identify them with a different sampler_ID (e.g. with succesive numbers: 01, 02, 03, ...) |  |  |  |  |  |

### XX2009.pps
Form with information on passive sampler(s) on intensive monitoring plot and at stations

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, latitude, longitude, altitude, compound, sampler, manufacturer, date_monitoring_1st, date_monitoring_last, measurements, col, altitude_m, elevation_lowest2500, elevation_lowest5000, other_observations

| Column | Description |  | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number |  | I 4 |  |  |
| 6 – 7 | Country code (France = 01, etc.) |  | I 2 | X | (1) |
| 9 – 12 | Observation plot number |  | I 4 |  |  |
| 14 – 18 |  | Number of active sampler AS001- AS999 (active sampler = ASxxx) |  |  |  |
| 20 – 22 |  | Compound air quality |  |  | (74) |
| 14 – 20 | Latitude in +DDMMSS |  | C 7 |  | (4) |
| 22 – 28 | Longitude in (+ or -) DDMMSS |  | C 7 |  | (4) |
| 30 – 31 | Altitude (in 50 m classes 1-51) |  | I 2 | X | (7) |
| 33 – 35 | Compound air quality |  | C 3 | X | (89) |
| 37 – 38 | Sampler ID |  | I 2 |  | (92) |
| 40-41 | Passive sampler manufacturer |  | C 2 | X | (93) |
| 43-48 | Start date measurements (DDMMYY) |  | Date |  | (3) |
| 50-55 | End date measurements (DDMMYY) |  | Date |  | (3) |
| 57-58 | Number of measurements with passive sampler (99) |  | I 2 |  |  |
| 60-60 | Passive sampler co-located with continuous analyzers (Y/N) |  | Y/N |  | (96) |
| 62-65 | Altitude (in m)* |  | I 4 |  |  |
| 67-70 | Lowest elevation in a circular area of 2.5 km radius* (in 9999 m) |  | I 4 |  | (97) |
| 72-75 | Lowest elevation in a circular area of 5.0 km radius* (in 9999 m) |  | I 4 |  | (97) |
| 77-116 | Observations |  | C 40 |  | (84) |

*Only for ozone measurements Note: For continuous analyzers: If for a given pollutant and plot measurements are taken with more than one continuous analyzer, identify them with a different sampler_ID (e.g. with succesive numbers: 01, 02, 03, ...)Note that each passive sampler has a unique code. This is Country code – plotnumber – sampler ID

### .XX2009.AQA
data file to be used for data from active samplers

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, sampler, date, hour, O3, SO2, NO2, NH3, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 6 | Sequence number | I 6 |  |  |
| 8 – 9 | Country code (France = 01, etc.) | I 2 | X | (1) |
| 11 – 14 | Plot number | I 4 |  | (2) |
| 16 – 17 | Sampler ID | I 2 |  | (92) |
| 19 – 24 | Date (DDMMYY) | Date |  | (3) |
| 26 – 27 | Hour (HH) | I 2 |  | (94) |
| 29 – 34 | Hourly O3 concentration (ppb)* | F 6 |  |  |
| 36 – 41 | Hourly SO2 concentration (µg SO2/m³)* | F 6 |  |  |
| 43 – 48 | Hourly NO2 concentration (µg NO2/m³)* | F 6 |  |  |
| 50 – 55 | Hourly NH3 concentration (µg NH3/m³)* | F 6 |  |  |
| 57 – 96 | Other observations (text) | C 40 |  | (84) |
|  | *missing data blank |  |  |  |

### XX2009.AQP
data file to be used for data from passive samplers

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, sampler, date_start, date_end, compound, value, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number | I 4 |  |  |
| 6 – 7 | Country code (France = 01, etc.) | I 2 | X | (1) |
| 9 – 12 | Plot number | I 4 |  | (2) |
| 14 – 15 | Sampler ID | I 2 |  | (92) |
| 17 – 22 | Start date measurement period (DDMMYY) | Date |  | (3) |
| 24 – 29 | End date measurement period (DDMMYY) | Date |  | (3) |
| 31 – 33 | Compound air quality | C 3 | X | (89) |
| 35 – 40 | Value | F 6 |  |  |
| 42 – 81 | Other observations (text) | C 40 |  | (84) |

### XX2009.COL
Co-located passive samplers and continuous analyzers at intensive monitoring plots or at air quality station

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, latitude, longitude, altitude, compound, inlet_height, date_start, hour_start, date_end, hour_end, manufacturer, replicates, mean, stdev, mean_cont, percentage_valid, detection_limit, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 - 4 | Sequence number | I 4 |  |  |
| 6 - 7 | Country code (France = 01, etc.) (1) | I 2 | X | (1) |
| 9 - 12 | Plot number (9999) or Station number (S999) | I 4 |  | (2) |
| 14 - 20 | Latitude in +DDMMSS | C 7 |  | (4) |
| 22 - 28 | Longitude in (+ or -) DDMMSS | C 7 |  | (4) |
| 30 - 31 | Altitude (in 50 m classes 1-51) | I 2 | X | (7) |
| 33 - 35 | Compound air quality measured (NH3, NO2, O3, SO2) | C 3 | X | (89) |
| 37 - 40 | Inlet height (in m; accuracy 0.1m; only for active samplers) | F 4 |  |  |
| 42 - 47 | Start date measurement period (DDMMYY) | Date |  | (3) |
| 49 - 50 | Start time measurement period (HH) | I 2 |  | (94) |
| 52 - 57 | End date measurement period (DDMMYY) | Date |  | (3) |
| 59 - 60 | End time measurement period (HH) | I 2 |  | (95) |
| 62 - 63 | Passive sampler manufacturer | C 2 | X | (93) |
| 65 - 66 | Number of co-located passive samplers (replicates) used for the measurement | I 2 |  |  |
| 68 - 73 | Mean value of replicates of co-located passive samplers | F 6 |  |  |
| 75 - 80 | Standard deviation of replicates of co-located passive samplers | F 6 |  |  |
| 82 - 87 | Related continuous analyzer mean value for the measurement period | F 6 |  |  |
| 89 - 91 | Related continuous analyzer percentage of valid hourly data for the measurement period (%) | I 3 |  |  |
| 93 - 98 | Lowest detection limit of the continuous analyzer | F 6 |  |  |
| 100 - 139 | Observations (text) | C 40 |  | (84) |

### XX2009.AQB – Submission of Analyses of Blanks (blank passive samplers)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, sampler, date_start, date_end, compound, value, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number | I 4 |  |  |
| 6 – 7 | Country code (France = 01, etc.) | I 2 | X | (1) |
| 9 – 12 | Plot number | I 4 |  | (2) |
| 14 – 15 | Sampler ID | I 2 |  | (92) |
| 17 – 22 | Start date of the exposure period of the exposed samplers for which this is a travel blank (DDMMYY) | Date |  | (3) |
| 24 – 29 | End date of the exposure period of the exposed samplers for which this is a travel blank (DDMMYY) | Date |  | (3) |
| 31 – 33 | Compound air quality | C 3 | X | (89) |
| 35 – 40 | Value | F 6 |  |  |
| 42 – 81 | Other observations (text) | C 40 |  | (84) |

Part X

## Assessment of Ozone Injury

### XX2004.PLL
OZONE INJURY ASSESSMENT – reduced plot file

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, survey_tp, latitude, longitude, altitude, rectangles, precision, soil_moisture, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number (1 to 99999) | I 5 |  |  |
| 7 – 8 | Country code | I 2 | X | (1) |
| 10 – 13 | Plotcode | I 4 |  | (2) |
| 15 – 17 | Survey type | C 3 | X | (98) |
| 19 – 25 | Latitude (+DDMMSS) | C 7 |  | (4) |
| 27 – 33 | Longitude (+/-DDMMSS) | C 7 |  | (4) |
| 35 – 36 | Altitude: altitude class code (50m classes from 1-51 ) | I 2 | X | (7) |
| 38 – 39 | Total number of rectangles (quadrats) established*) | I 2 |  |  |
| 41 – 42 | Precision level of the sampling (10 or 20, corresponding to 10% or 20% error, see Table 4 of submanual) *) | I 2 | X | (101) |
| 44 | Soil Moisture | I 1 | X | (99) |
| 46 – 85 | Other observations (text) | C 40 |  | (84) |

*) only to be reported if survey type is LSS

### XX2004.LTF
OZONE INJURY ASSESSMENT – assessment on main tree species (foliar trees)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, tree, tree_species, tree_species_sc_name, sample, date, date_analysis, percentage_symptomatic_actual_c, percentage_symptomatic_cl, validated, validation, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number of plots (1 to 99999) | I 5 |  |  |
| 7 – 8 | Country code | C 2 | X | (1) |
| 10 – 13 | Plot number | I 4 |  | (2) |
| 15 – 18 | Tree number (identical to the number in the foliar assessment, e.g. F003) | C 4 | X | (39) |
| 20 – 22 | Tree species code | I 3 | X | (20) |
| 24 – 61 | Scientific name of tree species | C 38 | X | (20) |
| 63 – 64 | Sample number | I 2 |  |  |
| 66 – 71 | Date sampling (in DDMMYY) | Date |  | (3) |
| 73 – 78 | Date analysis (in DDMMYY) | Date |  | (3) |
| 80 | Percentage of symptomatic leaves for actual year's leaves or needles (C). | I 1 | X | (100) |
| 82 | Percentage of symptomatic needles of last year (C+1) in code. | I 1 | X | (100) |
| 84-85 | Validated (Y/N/NR) | C 2 | X | (102) |
| 87-89 | Type of validation: The ozone symptom has been validated by the validation centre based on code | C 3 | X | (103) |
| 91-130 | Other observations (e.g. presence of other biotic or abiotic factors) | C 40 |  | (84) |

### XX2004.LSS
OZONE INJURY ASSESSMENT – Less Exposed Sampling Site (LESS)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, rectangle, date, species_sc_name, species, perennial, symptoms, collected_leaves, collected_seeds, validated, validation, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number | I 5 |  |  |
| 7 – 8 | Country code | I 2 | X | (1) |
| 10 – 13 | Plot number | O 4 |  | (2) |
| 15 – 16 | Rectangle (quadrat) number | I 2 |  | (104) |
| 18 – 23 | Date sampling (in DDMMYY) | Date |  | (3) |
| 25 – 62 | Scientific name | C 38 | X | (82) |
| 64 – 75 | Species code (code of ground vegetation, Flora Europaea) | C 12 | X | (82) |
| 77 | Perennial/Annual (P/A) | C 1 | X | (159) |
| 79 | Ozone symptoms? (Y/N) | Y/N |  |  |
| 81 | Leaves collected (Y/N) | Y/N |  |  |
| 83 | Seeds collected (Y/N) | Y/N |  |  |
| 85 – 86 | Validated (Y/N/NR) | C 2 | X | (102) |
| 88 – 90 | Type of validation: The ozone symptom has been validated by the validation centre based on code | C 3 | X | (103) |
| 92-131 | Other observations (text) | C 40 |  | (84) |

### XX2004.OTS
OZONE INJURY ASSESSMENT - OTHER SYMPTOMATIC SPECIES

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date, species_sc_name, species, perennial, validated, validation, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence number | I 5 |  |  |
| 7 – 8 | Country code | I 2 | X | (1) |
| 10 – 13 | Plot number | I 4 |  | (2) |
| 15 – 20 | Date sampling (in DDMMYY) | Date |  | (3) |
| 22 – 59 | Scientific name | C 38 | X | (82) |
| 61 – 72 | Species code (code of ground vegetation, Flora Europaea) | C 12 | X | (82) |
| 74 | Perennial/Annual (P/A) | C 1 | X | (159) |
| 76-77 | Validated (Y/N/NR) | C 2 | X | (102) |
| 79 – 81 | Type of validation: The ozone symptom has been validated by the validation centre. | C 3 | X | (103) |
| 83-122 | Other observations (text) | C 40 |  | (84) |

Part XI

## Sampling and Analysis of Litterfall

### XX1996.LFP
Contents of reduced plot file to be used in combination with the survey on litterfall

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, latitude, longitude, altitude, traps, collecting_area, date_monitoring_1st, date_monitoring_last, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country Code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 9 – 12 | Plotnumber (max. 9999) | I 4 |  | (2) |
| 14 – 20 | Latitude in +DDMMSS (e.g.+505852) | C 7 |  | (4) |
| 22 – 28 | Longitude in (+ or -)DDMMSS (e.g. +035531) | C 7 |  | (4) |
| 30 – 31 | Altitude (in 50 meter classes from 1 to 51) | I 2 | X | (7) |
| 33 – 34 | Number of traps | I 2 |  |  |
| 36 – 39 | Total Collecting area (in m2) | F 4 |  |  |
| 41 – 46 | Active sampling period (from) in DDMMYY (e.g. 010690) | Date |  | (3) |
| 48 – 53 | Active sampling period (till) in DDMMYY (e.g. 300690) | Date |  | (3) |
| 55 – 94 | Other observations (text) | C 40 |  | (84) |

### xx20079.LFM
Contents of data file with litterfall analysis information (mandatory)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, date_start, date_end, trap, tree_species, sample, dry_weight, dry_mass, area, C, N, S, P, Ca, Mg, K, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 5 | Sequence Number (1 to 99999) | I 5 |  |  |
| 7 – 10 | Observation Plot number (max. 9999) | I 4 |  | (2) |
| 12 – 17 | Collection period (from) | Date |  | (3) |
| 19 – 24 | Collection period (till) | Date |  | (3) |
| 26 – 27 | Trap number: "-9" means plot average | I 2 |  |  |
| 29 – 31 | Tree species code: dominant or co-dominant species | I 3 | X | (20) |
| 33 – 36 | Sample code (see below) | I 2F 4 | X | (154) |
|  | 10 Total |  |  |  |
|  | 11 Foliar litter |  |  |  |
|  | 12 Non foliar litter |  |  |  |
|  | 13 Flowering |  |  |  |
|  | 14 Fruiting/seeds |  |  |  |
|  | 15 Budshells |  |  |  |
|  | 16 Twigs/branches |  |  |  |
|  | 19 Other biomass |  |  |  |
| 38 – 42 | Dry weight per m2 [kg/m2] | F 5 |  |  |
| 49 – 52 | Area of 100 leaves or of 1000 needles [m2] | F 4 |  |  |
| 44 – 47 | Dry mass of 100 leaves or of 1000 needles [g] | F 4 |  | (41) |
|  | Parameters Units **) |  |  |  |
| 54 – 58 | C [mg/g g/100g] | F 5 |  |  |
| 60 – 64 | N [mg/g] | F 5 |  |  |
| 66 – 70 | S [mg/g] | F 5 |  |  |
| 72 – 75 | P [mg/g] | F 4 |  |  |
| 77 – 81 | Ca [mg/g] | F 5 |  |  |
| 83 – 87 | Mg [mg/g] | F 5 |  |  |
| 89 – 93 | K [mg/g] | F 5 |  |  |
| 93 – 94 | Current interlaboratory comparison test number (eg. 08) | C 2 |  | (71) |
| 96 – 98 | Laboratory codenumber in this test (eg. 003 or 36a) | C 3 |  | (71) |
| 95 – 134 | Other observations text | C 40 |  | (84) |

**)     	By reference at 105° dried material

### xx20079.LFO
Contents of data file with litterfall analysis information (Optional)

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, plot, date_start, date_end, trap, tree_species, sample, Zn, Mn, Fe, Cu, Pb, B, Cd, other_observations

| Column | Description |  | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- | --- |
| 1 – 5 | Sequence Number (1 to 99999) |  | I 5 |  |  |
| 7 – 10 | Observation Plot number (max. 9999) |  | I 4 |  | (2) |
| 12 – 17 | Collection period (from) |  | Date |  | (3) |
| 19 – 24 | Collection period (till) |  | Date |  | (3) |
| 26 – 27 | Trap number: "-9" means plot average |  | I 2 |  |  |
| 29 – 31 | Tree species code: dominant or co-dominant species |  | I 3 | X | (20) |
| 33 – 36 | Sample code (see below) |  | I2 F 4 | X | (154) |
|  | 10 Total |  |  |  |  |
|  | 11 Foliar litter |  |  |  |  |
|  | 12 Non foliar litter |  |  |  |  |
|  | 13 Flowering |  |  |  |  |
|  | 14 Fruiting/seeds |  |  |  |  |
|  | 15 Budshells |  |  |  |  |
|  | 16 Twigs/branches |  |  |  |  |
|  | 19 Other biomass |  |  |  |  |
|  | By reference at 105° dried material: |  |  |  |  |
| 38 – 42 | Na mg/g not to be submitted from 2007 onwards;  submit blanks in these columns |  | F 5 |  |  |
| 44 – 48 | Zn μg/g mg/g |  | F 5 |  |  |
| 50 – 55 | Mn μg/g mg/g |  | F 6 |  |  |
| 57 – 61 | Fe μg/g mg/g |  | F 5 |  |  |
| 63 – 68 | Cu μg/g mg/g |  | F 6 |  |  |
| 70 – 74 | Pb μg/g mg/g |  | F 5 |  |  |
| 76 – 79 | Al mg/g not to be submitted from 2007 onwards;  submit blanks in these columns |  | F 4 |  |  |
| 81 – 85 | B μg/g mg/g |  | F 5 |  |  |
| 87 – 91 | Cd ng/g |  | F 5 |  |  |
| 85 – 86 | Current interlaboratory comparison test number (eg. 08) |  | C 2 |  | (71) |
| 88 – 90 | Laboratory codenumber in this test (eg. 003 or 36a) |  | C 3 |  | (71) |
| 93 – 132 | Other observations | text | C 40 |  | (84) |

**)     	By reference at 105° dried material

### XX2009LF.LQA
Litterfall – Laboratory QA/QC information

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date_start, date_end, parameter, pretreatment, determination, quantification_limit, control_chart_mean, control_chart_std, ring_test_participation, ring_test_number, Laboratory_ID, percentage_within, requalification, percentage_within_requal, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 01 – 04 | Sequence number (1 to 9999) | I 4 |  |  |
| 06 – 07 | Country code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 09 – 12 | Observation Plot number (maximum 9999) | I 4 |  | (2) |
| 14 – 19 | start date | Date |  | (117) |
| 21 – 26 | end date | Date |  | (117) |
| 28 – 29 | Parameter Code (C, N, etc.) | C 2 | X | (111) |
| 31 – 34 | Pretreatment method (see reference list) | F 4 | X | (105) |
| 36 – 39 | Determination method (see reference list) | F 4 | X | (106) |
| 41 – 46 | Quantification limit (unit of parameter) | F 6 |  | (107) |
| 48 – 53 | Mean of control chart | F 6 |  | (107) |
| 55 – 57 | Standard deviation [% of mean] => coefficient of variation | F 3 |  | (107) |
| 59 | Participated at ring Test (yes = 1, no = 0) | I 1 |  | (107) |
| 61 – 63 | ICP Forests Ring Test Number | C 3 |  | (107) |
| 65 – 67 | ID of laboratory (e.g. H45, B78, etc.) | C 3 |  | (107) |
| 69 – 71 | Percentage [%] of the results of the ring tests within tolerable limits for each ring test | I 3 |  | (107) |
| 73 | Requalification information (yes = 1, no = 0) | I 1 |  | (107) |
| 75 – 77 | Percentage [%] of the results of the ring tests within tolerable limits for each ring test in requalification | I 3 |  | (107) |
| 79 – 118 | Other observations (freetext) | C 40 |  | (84) |

## Leaf Area Index (LAI) and Radiation Measurements

### XX2009.PLA
reduced plot file on LAI measurements

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, latitude, longitude, altitude, slope, exposition, tree_cover, shrub_height, shrub_cover, storey, date_monitoring_1st, date_monitoring_last, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country Code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 9 – 12 | Plotnumber (max. 9999) | I 4 |  | (2) |
| 14 – 20 | Latitude in +DDMMSS (e.g.+505852) | C 7 |  | (4) |
| 22 – 28 | Longitude in (+ or -)DDMMSS (e.g. +035531) | C 7 |  | (4) |
| 30 – 31 | Altitude (in 50 meter classes from 1 to 51) | I 2 | X | (7) |
| 33 – 34 | Slope in degree | I 2 |  |  |
| 36 – 38 | Exposition in degree 1 to 360 | I 3 |  |  |
| 40 – 42 | Tree layer cover (in % of LAI survey area) | I 3 |  | (80) |
| 44 – 47 | Shrub layer height (in m) if above 1.5m | F 4 |  | (80) |
| 49 – 52 | Shrub layer cover (in % of LAI survey area) | F 4 |  | (80) |
| 54 – 56 | Storey description | C 3 | X | (126) |
| 58 – 63 | Direct method: Active sampling period (from) in DDMMYY (e.g. 010690) | Date |  | (3) |
| 65 – 70 | Direct method: Active sampling period (till) in DDMMYY (e.g. 300690) | Date |  | (3) |
| 72 – 111 | Other observations (text) | C 40 |  | (84) |

### XX2009.LAC
coordinates of LAI measurement points and other surveys

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, survey, device, x_coordinate, y_coordinate, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 1 – 4 | Sequence number of plots (1 to 9999) | I 4 |  |  |
| 6 – 7 | Country Code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 9 – 12 | Observation plotnumber (max. 9999) | I 4 |  | (2) |
| 14 – 15 | survey | C 2 | X | (156) |
| 17 – 19 | device ID (e.g. measurement point on LAI, trap number on Litterfall, sampler on deposition survey) | C 3 |  |  |
| 21 – 28 | X coordinate (metric system; relative plot coordinate) | F 8 |  |  |
| 30 – 37 | Y coordinate (metric system; relative plot coordinate) | F 8 |  |  |
| 39 – 78 | Other observations (text) | C 40 |  | (84) |

### XX2009.LAP
Leaf Area Index (LAI) photo documentation

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date, time, measurement_point, photo_file, parameter, value, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 01 – 04 | Sequence number (1 to 9999) | I 4 |  |  |
| 06 – 07 | Country code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 09 – 12 | Observation Plot number (maximum 9999) | I 4 |  | (2) |
| 14 – 19 | Date of field observation (photography) | Date |  | (3) |
| 21 – 26 | Time of file observation (HHMMSS) (e.g. 095401) | C 6 |  |  |
| 28 – 29 | number of measurement point (99 = plot mean calculated) | I 2 |  |  |
| 31 – 58 | photo file name in case of photo documentation [XXPPPPNNNNDDDDDDTTTTTTSS.jpg] | C 28 |  | (125) |
| 60 – 65 | Parameter to be specified | C 6 | X | (127) |
| 67 – 106 | Specified value | C 40 |  |  |
| 108 – 147 | Other observation (text) | C 40 |  | (84) |

### XX2009.LAM 
Leaf Area Index (LAI) measurement outcome

Each data file has to start with a comment line. This line starts with an exclamation mark:

!Sequence, country, plot, date, measurement_point, date_analysis, determination, parameter, photo_file, value, other_observations

| Column | Description | Format | Ref_Tab | Item # |
| --- | --- | --- | --- | --- |
| 01 – 04 | Sequence number (1 to 9999) | I 4 |  |  |
| 06 – 07 | Country code (France = 01, Belgium = 02, etc.) | I 2 | X | (1) |
| 09 – 12 | Observation Plot number (maximum 9999) | I 4 |  | (2) |
| 14 – 19 | Date of field observation | Date |  | (3) |
| 21 – 22 | number of measurement point (99 = plot mean calculated) | I 2 |  |  |
| 24 – 29 | Date of data processing / laboratory analysis | Date |  | (3) |
| 31 – 32 | Method used for determination | I 2 | X | (128) |
| 34 – 36 | Parameter Code (100, 110,.) | I 3 | X | (129) |
| 38 – 65 | photo file name in case of photo documentation [XXPPPPNNNNDDDDDDTTTTTTSS.jpg] | C 28 |  | (125) |
| 67 – 74 | Value of parameter | F 8 |  |  |
| 76 – 115 | Other observation (text) | C 40 |  | (84) |

# Explanatory Items

- Country code (France = 01, Belgium = 02, etc.) D_COUNTRY

(all)

| Code | Country |  | Code | Country |
| --- | --- | --- | --- | --- |
| 1 | France |  | 59 | Estonia |
| 2 | Belgium |  | 60 | Slovenia |
| 3 | Netherlands |  | 61 | Republic of Moldova |
| 4 | Germany |  | 62 | Russia |
| 5 | Italy |  | 63 | Bulgaria |
| 6 | United Kingdom |  | 64 | Latvia |
| 7 | Ireland |  | 65 | Belarus |
| 8 | Denmark |  | 66 | Cyprus |
| 9 | Greece |  | 67 | Serbia |
| 10 | Portugal |  | 68 | Andorra |
| 11 | Spain |  | 69 | Malta |
| 12 | Luxembourg |  | 70 | Monaco |
| 13 | Sweden |  | 71 | Albania |
| 14 | Austria |  | 72 | Turkey |
| 15 | Finland |  | 73 | Liechtenstein |
| 50 | Switzerland |  | 74 | Ukraine |
| 51 | Hungary |  | 75 | Iceland |
| 52 | Romania |  | 76 | Holy See (Vatican City State) |
| 53 | Poland |  | 77 | San Marino |
| 54 | Slovak Republic |  | 78 | Former Yugoslavian Republic of Macedonia |
| 55 | Norway |  | 79 | Bosnia and Herzegovina |
| 56 | Lithuania |  | 95 | Canaries (Spain) |
| 57 | Croatia |  | 96 | Azores (Portugal) |
| 58 | Czech Republic |  |  |  |

- Observation plot number (maximum 9999)

(all)

The observation plot number corresponds to a unique number given to the permanent plot during the selection or installation.

- Date of observation, date of assessment, date of analysis

(all)

Dates shall be completed in the following order (day, month and year):

DDMMYY

| Day | Month | Year |
| --- | --- | --- |
| 31 | 03 | 09 |

 310309

- Latitude-longitude coordinates

(all)

| Fill in the full six figure latitude and longitude coordinates of the centre of the observation plot, e.g: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | +/- | Degress |  | Minutes |  | Seconds |  |
| — latitude | + | 5 | 0 | 2 | 0 | 2 | 7 |
| — longitude | - | 0 | 1 | 1 | 5 | 3 | 2 |
| the first box/digit is used to indicate a + or – coordinate |  |  |  |  |  |  |  |

 +502027     and 
-011532	to be submitted

- Availability of water to principal species (estimate) D_WATER

(CC, SO)

| Code | water availability |
| --- | --- |
| 1 | Insufficient |
| 2 | Sufficient |
| 3 | Excessive |

- Humus type  D_HUMUS

(CC, SO)

| Code | Humus form |
| --- | --- |
| 1 | Mull |
| 2 | Moder |
| 3 | Mor |
| 4 | Amphi (or Amphihumus) |
| 5 | Anmoor |
| 6 | Histomull |
| 7 | Histomoder |
| 8 | Histomor |
| 9 | Histoamphi |

- Altitude D_ALTITUDE

(all)

| Code | lower |  |  | upper | code |  | lower | upper |  | code | lower | upper | code | lower | upper |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | 50 m | 14 |  | 651 | 700 m |  | 27 | 1301 | 1350 m | 40 | 1951 | 2000 m |
| 2 |  | 51 | 100 m |  |  | 15 | 701 | 750 m | 28 |  | 1351 | 1400 m | 41 | 2001 | 2050 m |
| 3 |  | 101 | 150 m |  |  | 16 | 751 | 800 m | 29 |  | 1401 | 1450 m | 42 | 2051 | 2100 m |
| 4 |  | 151 | 200 m |  |  | 17 | 801 | 850 m | 30 |  | 1451 | 1500 m | 43 | 2101 | 2150 m |
| 5 |  | 201 | 250 m |  |  | 18 | 851 | 900 m | 31 |  | 1501 | 1550 m | 44 | 2151 | 2200 m |
| 6 |  | 251 | 300 m |  |  | 19 | 901 | 950 m | 32 |  | 1551 | 1600 m | 45 | 2201 | 2250 m |
| 7 | 301 |  |  | 350 m | 20 |  | 951 | 1000 m |  | 33 | 1601 | 1650 m | 46 | 2251 | 2300 m |
| 8 | 351 |  |  | 400 m | 21 |  | 1001 | 1050 m |  | 34 | 1651 | 1700 m | 47 | 2301 | 2350 m |
| 9 | 401 |  |  | 450 m | 22 |  | 1051 | 1100 m |  | 35 | 1701 | 1750 m | 48 | 2351 | 2400 m |
| 10 | 451 |  |  | 500 m | 23 |  | 1101 | 1150 m |  | 36 | 1751 | 1800 m | 49 | 2401 | 2450 m |
| 11 | 501 |  |  | 550 m | 24 |  | 1151 | 1200 m |  | 37 | 1801 | 1850 m | 50 | 2451 | 2500 m |
| 12 | 551 |  |  | 600 m | 25 |  | 1201 | 1250 m |  | 38 | 1851 | 1900 m | 51 | > | 2500 m |
| 13 | 601 |  |  | 650 m | 26 |  | 1251 | 1300 m |  | 39 | 1901 | 1950 m |  |  |  |

- Orientation D_ORIENTATION

(SI, C1)

| Code | orientation |
| --- | --- |
| 1 | N |
| 2 | NE |
| 3 | E |
| 4 | SE |
| 5 | S |
| 6 | SW |
| 7 | W |
| 8 | NW |
| 9 | flat |

- Mean age of dominant storey (years) D_MEAN_AGE

(SI, C1)

| Code | mean age |
| --- | --- |
| 1 | ≤ 20 |
| 2 | 21— 40 |
| 3 | 41— 60 |
| 4 | 61— 80 |
| 5 | 81—100 |
| 6 | 101—120 |
| 7 | > 120 |
| 8 | Irregular stands |

- Tree age (D1) D_TREE_AGE (NEW!)

(TV)

| Code | tree age |
| --- | --- |
| 1 | ≤ 20 |
| 2 | 21— 40 |
| 3 | 41— 60 |
| 4 | 61— 80 |
| 5 | 81—100 |
| 6 | 101—120 |
| 7 | 121—140 |
| 8 | 141—160 |
| 9 | >160 |

- Method of age determination (D1) D_AGE_DETERMINATION (NEW

(TV)

| Code | Tree age determination method |
| --- | --- |
| 1 | assured dates of stand establishment |
| 2 | tree stumps |
| 3 | age determination of the lowermost twigs (add estimated time it has taken to grow to that height) |
| 4 | increment borer, stem discs (from similar sized trees/median sized trees) outside the plot |
| 5 | assessment (impossible in most cases) |
| 6 | estimation without any exact information |

- Intenstity of Fruiting (D1) D_INTENSITy_FRUITING (NEW!)

(TV)

| Code | Description |  |
| --- | --- | --- |
| 1.1 | absent | Fructification is absent or inconsiderable. Even reasonably lengthy observation of the crown with binoculars yielded no signs of fruiting. |
| 1.2 | scarce | Sporadic occurrence of fruiting, not noticeable at first sight. It must be looked for on purpose with binoculars. |
| 2 | medium | Fructification is such that it can be observed with the naked eye. The appearance of the tree is influenced but not dominated by fructification. |
| 3 | high | Fructification is obvious and immediately meets the eye, determines the tree's appearance. |

- Crown diameter related distance to neighbours (D1) D_CDRD_N (NEW!)

(TV)

Calculation:

[Score1 + Score2 + Score3 + Score4] / 4 = CDRD_N

| Score for calculation | Description |  |
| --- | --- | --- |
| 1 | cramped. | Canopies overlap. |
| 2 | closed. | Crowns touch one another. |
| 3 | loose spread. | Gap between crowns up to one third of average crown diameter |
| 4 | spread. | Gap between crowns up to two thirds of average crown diameter |
| 5 | Distant. | Gap between crowns from two thirds up to one whole of average crown diameter |
| 6 | very distant. | Gap between crowns > than 1/1 of average crown diameter |

Fig. 3: Example: Crown diameter related distance to neighbours

Example:

[2+2+6+5]/4= 3,75   => 3,75 to be submitted with form.

- Apical shoot architecture (D1) D_APICAL_shoot (NEW!)

(TV)

| Code | Description |
| --- | --- |
| 1 | Exploratory phase: Apical shoots and upper side buds form long shoots. Flat, longitudinal, expansive shoot development. |
| 2 | Intermediary form 1/3 |
| 3 | Degeneration phase: Only apical bud forms long shoot. Shoots of side buds are stunted. Spear-shaped development of main shoots with reduced side shoot formation "spear-shaped". |
| 4 | Intermediary form 3/5 |
| 5 | Stagnation phase: Stunted long shoots, claw-like appearance because of pluriannual short shoot chains. |
| 6 | Intermediary form 5/7 |
| 7 | Resignation phase: Die-back of twigs of the topmost part of the crown or even the whole crown itself. |

- Removals and mortality (D1) D_REMOVal_MORTALITY_TV (NeW!)

(TV)

Removals and mortality will be reported using the following codes:

| Code | Description |
| --- | --- |
| 1 | tree alive in current and previous inventory |
| 2 | new alive tree (ingrowth) |
| 3 | alive tree (present but not assessed in previous inventory) |
| 4 | dead tree |
| 5 | tree is removed |
| 6 | tree is present and alive but no assessment could be carried out (e.g. due to crown breakage) |
| 7 | information on this tree is missing for this years inventory (e.g. tree was forgotten during field work) |

- Total plot size in hectares

(Chapter 1, 5)

The size of the total plot or sub-plot shall be stated in hectares with 1m² accuracy (e.g. 0.0001 ha).

- Number of trees in total plot

(Chapter 1, 5, 10b)

The total number of trees (shoots in coppice forests) in the total plot. All trees (shoots) from 5 (3) cm (DBH) and more are counted.

- Yield estimate D_YIELD_ABS, D_YIELD_REL

(Chapter 1)

The yield estimates consist of an absolute and a relative yield estimate.

The absolute estimate will be the estimated average yield over the total life period of the stand. The relative yield will indicate whether the absolute yield estimate is considered to be low, normal or high for the stand. The following codes will be used:

| Absolute yield code | Relative yield code |
| --- | --- |
| 0 = 0.0 — 2.5 m3 per hectare per year | 1 = Low |
| 1 = 2.5 — 7.5 m3 per hectare per year | 2 = Medium |
| 2 = 7.5 — 12.5 m3 per hectare per year | 3 = High |
| 3 = 12.5 — 17.5 m3 per hectare per year |  |
| 4 = 17.5 — 22.5 m3 per hectare per year |  |
| 5 = >22.5 m3 per hectare per year |  |

- Sample tree number

(Chapter 2, 5, 9, 10b)

The tree number is the number which has been assigned to the tree during the installation of the plot.

Note: a copy of the numbers of sample trees that were assessed the year before and which must be included in the assessment in the current year should be provided to the surveyors each year. Further information should not be supplied as repeated assessments of, for example, species, will act as a control on the quality of the observations.

- Main tree species (Reference Flora Europaea) D_ TREE_SPEC

(Chapter 1, 2, 5, 9, 10b, 11)

Broadleaves (* = species to be used for the foliage inventory)

| 000: Unknown 001: Acer campestre* 002: Acer monspessulanum* 003: Acer opalus 004: Acer platanoides 005: Acer pseudoplatanus* 006: Alnus cordata* 007: Alnus glutinosa* 008: Alnus incana 009: Alnus viridis 010: Betula pendula* 011: Betula pubescens* 012: Buxus sempervirens 013: Carpinus betulus* 014: Carpinus orientalis 015: Castanea sativa (C. vesca)* 016: Corylus avellana* 017: Eucalyptus sp.* 018: Fagus moesiaca* 019: Fagus orientalis 020: Fagus sylvatica* 021: Fraxinus angustifolia spp. oxycarpa (F. oxyphylla)* 022: Fraxinus excelsior* 023: Fraxinus ornus* 024: Ilex aquifolium 025: Juglans nigra 026: Juglans regia 027: Malus domestica 028: Olea europaea* 029: Ostrya carpinifolia* 030: Platanus orientalis 031: Populus alba 032: Populus canescens 033: Populus hybrides* 034: Populus nigra* 035: Populus tremula* 036: Prunus avium* 037: Prunus dulcis (Amygdalus communis) 038: Prunus padus 039: Prunus serotina 040: Pyrus coomunis 041: Quercus cerris* 042: Quercus coccifera (Q. calliprinos)* 043: Quercus faginea* 044: Quercus frainetto (Q. conferta)* 045: Quercus fruticosa (Q. lusitanica) 046: Quercus ilex* 047: Quercus macrolepis (Q. aegilops) 048: Quercus petraea* 049: Quercus pubescens* 050: Quercus pyrenaica (Q. toza)* | 051: Quercus robur (Q. pedunculata)* 052: Quercus rotundifolia* 053: Quercus rubra* 054: Quercus suber* 055: Quercus trojana 056: Robinia pseudoacacia* 057: Salix alba 058: Salix caprea 059: Salix cinerea 060: Salix eleagnos 061: Salix fragilis 062: Salix sp. 063: Sorbus aria 064: Sorbus aucuparia 065: Sorbus domestica 066: Sorbus torminalis 067: Tamarix africana 068: Tilia cordata 069: Tilia platyphyllos 070: Ulmus glabra (U. scabra, U. scaba, U. montana) 071: Ulmus laevis (U. effusa) 072: Ulmus minor (U. campestris, U. carpinifolia) 073: Arbutus unedo) 074: Arbutus andrachne 075: Ceratonia siliqua 076: Cercis siliquastrum 077: Erica arborea 078: Erica scoparia 079: Erica manipuliflora 080: Laurus nobilis 081: Myrtus communis 082: Phillyrea latifolia 083: Phyllyrea angustifolia 084: Pistacia lentiscus 085: Pistacia terebinthus 086: Rhamnus oleoides 087: Rhamnus alaternus 088: Betula tortuosa 089: deleted! 090: Crataegus monogyna 091: Ilex canariensis 092: Laurus azorica 093: Myrica faya 098: Quercus petrea_or_robur 099: Other broadleaves 201: Quercus hartwissiana 202: Quercus vulcanica 203: Quercus infectoria 204: Quercus macranthera 205: Quercus libani 206: Quercus brantii 207: Quercus ithaburensis 208: Quercus aucheri |
| --- | --- |

Conifers (* = species to be used for the foliage inventory)

| 100: Abies alba* 101: Abies borisii-regis* 102: Abies cephalonica* 103: Abies grandis 104: Abies nordmanniana 105: Abies pinsapo 106: Abies procera 107: Cedrus atlantica 108: Cedrus deodara 109: Cupressus lusitanica 110: Cupressus sempervirens 111: Juniperus communis 112: Juniperus oxycedrus* 113: Juniperus phoenicea | 114: Juniperus sabina 115: Juniperus thurifera* 116: Larix decidua* 117: Larix kaempferi (L.leptolepis) 118: Picea abies (P. excelsa)* 119: Picea omorika 120: Picea sichensis* 121: Pinus brutia* 122: Pinus canariensis 123: Pinus cembra 124: Pinus contorta* 125: Pinus halepensis* 126: Pinus heldreichii 127: Pinus leucodermis | 128: Pinus mugo (P. montana) 129: Pinus nigra* 130: Pinus pinaster* 131: Pinus pinea* 132: Pinus radiata (P.insignis)* 133: Pinus strobus 134: Pinus sylvestris* 135: Pinus uncinata* 136: Pseudotsuga menziesii* 137: Taxus baccata 138: Thuya sp. 139: Tsuga sp. 140: Chamaecyparis lawsonia 141: Cedrus brevifolia 199: Other conifers |
| --- | --- | --- |

- Removals and mortality D_REMOVAL_MORTALITY_CCGR; field “applicable_cc” = y

(Chapter 2)

The following classification must be used:

Code 0: tree alive and measurable (new, note this is different than a missing value)

01	tree alive, in current and previous inventory (formerly blanc)

02	new alive tree (ingrowth)

03	alive tree (present but not assessed in previous inventory)

Tree has been cut and removed, only its stump has been left

11		planned utilization, e.g. thinning

12		utilization for biotic reasons, e.g. insect damage

13		utilization for abiotic reasons, e.g. windthrow

14		cut, reason unknown

18		reason for disappearance unknown

Tree is still standing and alive, but crown condition parameters are no longer assessed

21		lop-sided or hanging tree

22		heavy crown break (over 50% of the crown) or broken stem

23		tree is no longer in Kraft classes 1, 2 or 3 (not applicable to the first inventory in a plot)

29		other reasons (specify)

Standing dead tree

31		biotic reasons, e.g. bark beetle attack

32		abiotic reasons, e.g. drought, lightning

38		unknown cause of death

Trees that have fallen (living or dead)

41		abiotic reasons (e.g. storm)

42		biotic reasons (e.g. beavers)

48		unknown cause

Note: Class 22 is only applicable in those countries that do not record trees with more than 50% crown damage.

Note: Class 23 is only applicable to those countries that restrict sampling to Kraft classes 1, 2 and 3.

Note: Mortality and the number of dead trees present in a plot are two different issues. Annual mortality can be calculated from the number of living trees that are dead the following year. The total number of dead trees in a plot at any one time provides no information on mortality rates, but provides information on the condition of a stand in the year of assessment.

Note: If trees in the plot have not been mapped, there may be some difficulty in identifying the fate of individual trees that have disappeared between surveys.

- Social class D_SOCIAL_CLASS (before 2006, at least since 2004 in form of expl. item)

(Chapter 2)

Four classes are recognized:

- dominant (including free-standing): Trees with upper crown standing above the general level of the canopy;

- codominant: Trees with crowns forming the general level of the canopy;

- subdominant: Trees extending into the canopy and receiving some light from above, but shorter than 1 or 2;

- suppressed: Trees with crowns below the general level of the canopy, receiving no direct light from above.

- Dying

Note: The assessment of the social class of a tree is in some cases difficult. Suppressed trees should not be equated with dying trees as, in a mixed-age stand, they represent future generations of trees. Classification on steep slopes presents a problem as even relatively short trees may receive direct light from above. In such cases, classification should be based on the relative heights of the trees.

- Crown shading D_SHADING

(Chapter 2)

Crown shading is assessed on a six-point scale as follows:

1	crown significantly affected (shading or physical interactions) on one side

2	crown significantly affected (shading or physical interactions) on two sides

3	crown significantly affected (shading or physical interactions) on three sides

4	crown significantly affected (shading or physical interactions) on four sides

5	crown open-grown or with no evidence of shading effects

6	suppressed trees

- Visibility D_VISIBILITY

(Chapter 2)

The following codes should be used for the assessable crown:

1	Whole crown is visible

2	Crown only partially visible

3	Crown only visible with backlighting (i.e. in outline)

4	Crown not visible

Note: Class 3 is distinguished from Class 4, as some parameters can still be assessed when only back-lighting is present.

- Defoliation D_DEFOLIATION

(Chapter 2)

Defoliation is assessed in 5% steps. These classes are 0, 5 (>0-5%), 10 (>5-10%) and so on. A tree with between >95% and 100% defoliation, which is still alive, is scored as 99. The score 100 is reserved for dead trees (EC Regulation). Trees should be reported in these 5% classes and not in aggregated groupings.

| Code | defoliation score |
| --- | --- |
| 0 | 0% |
| 5 | >0-5% |
| 10 | >5-10% |
| 15 | >10-15% |
| 20 | >15-20% |
| 25 | >20-25% |
| 30 | >25-30% |
| 35 | >30-35% |
| 40 | >35-40% |
| 45 | >40-45% |
| 50 | >45-50% |
| 55 | >50-55% |
| 60 | >55-60% |
| 65 | >60-65% |
| 70 | >65-70% |
| 75 | >70-75% |
| 80 | >75-80% |
| 85 | >80-85% |
| 90 | >85-90% |
| 95 | >90-95% |
| 99 | >95-<100% |
| 100 | 100% (dead) |

- Discolouration D_DISCOLOURATION

(Chapter 2)

| Class | Discolouration | Percentage of needles/leaves discoloured |
| --- | --- | --- |
| 0 | none | 0 – 10% |
| 1 | slight | >10 – 25% |
| 2 | moderate | >25 – 60% |
| 3 | severe | >60% |
| 4 | dead | dead |

- Foliage transparency D_FOLIAGE_TRANSPARANCY

(Chapter 2)

Estimate foliage transparency in 5% classes based on the live, normally foliated portion of the crown and branches using the transparency diagram in Fig. A1-2. Dead branches, crown dieback and missing branches where foliage is expected to be missing are deleted from the estimate (Fig. A1-3).

- Flowering D_FLOWERING

(Chapter 2)

Two assessments are made: of the assessable part of the crown and of the whole crown. Scoring is:

1	Absent or scarce. The flowers are not seen in a cursory examination.

2	Common. Flowering effect is clearly visible.

3	Abundant. Flowering dominates the appearance of the tree.

- Fruiting D_FRUITING

(Chapter 2)

As with flowering, two assessments are made: of the assessable part of the crown and of the whole crown. Scoring is:

1	Absent or scarce. The fruits are not seen in a cursory examination.

2	Common. Fruiting is clearly visible.

3	Abundant. Fruiting dominates the appearance of the tree.

Note:

Quantitative estimates of both flowering and fruiting can be obtained by the use of litter traps. However, such data cannot be readily related to individual trees.

- Secondary shoots and epicormics D_SHOOT_EPICORMIC

(Chapter 2)

Separate assessments are made of the frequency (3 classes) of epicormics in the assessable crown and on the stem. The assessment must include all epicormics, not only the ones of the current year. Scoring is in three classes:

- None or rare

- Medium: light development or only present in parts of the crown or stem

- Abundant: present throughout the majority of the crown or all over the stem

- Crown form/morphology (incl. Roloff) D_CROWN_FORM (at least since 1996)

(Chapter 2)

Crown form classifications have so far been developed for Picea spp., Fagus sylvatica and Pinus sylvestris.

Note: the use of the Roloff classification system for species other than Fagus sylvatica must be undertaken with special care and is not recommended.

Method:

Picea (Fig. A1-4)

11	comb

12	brush

13	plate

14 	mix

Fagus sylvatica (Fig. A1-5)

21	trees with vigorous growth both of apical and side shoots

22	reduced apical shoot growth, side shoots are still formed but at lower frequency (mainly consisting of short shoots)

23	strongly reduced apical shoot growth, no new lateral branches are formed. Shoot appearance is “claw-like”

24 	development of 23, with loss of side shoots

29	other

Pinus

31	pine, vigorous apical dominance with tree growing strongly upwards

32	pine, reduced or no apical dominance with crown showing signs of widening

33	pine, as 32, but lower branches being lost through suppression

34	platform developing, with dominant growth direction no longer upwards,

but crown still with some depth

35	platform fully developed, no vertical growth

39	other (specify)

- Affected part of the tree and location in crown D_AFFECTPART (since 2006), D_LOCCROWN

(Chapter 2, 9)

| Affected part | Specification of affected part (mandatory Level I and Level II) | Code | Location in crown (optional Level I, mandatory Level II) | Code |
| --- | --- | --- | --- | --- |
| Leaves/ Leedles | Current needle year Older needles Needles of all ages Broadleaves (incl. evergreen spec.) | 11 12 13 14 | Upper crown Lower crown Patches Total crown | 1 2 3 4 |
| Branches, shoots & buds | Current year shoots Twigs (diameter < 2 cm) Branches diameter 2 – < 10 cm Branches diameter  10 cm Varying size Top leader shoot Buds | 21 22 23 24 25 26 27 | Upper crown Lower crown Patches Total crown | 1 2 3 4 |
| Stem & collar | Crown stem: main trunk or bole within the crown Bole: trunk between the collar and the crown Roots (exposed) and collar ( 25 cm height) Whole trunk | 31 32 33 34 |  |  |
| Dead tree | see below | 04 |  |  |
| No symptoms on any part of tree | see below | 00 |  |  |
| No assessment | see below | 09 |  |  |

###### Special cases:

The following codes for special cases shall be reported in the column for ‘specification of affected part’ of the tree:

a. Dead trees:

Dead trees should be reported using code 04. The cause of death should be reported in the column for the causal agent / factor.

b. No symptoms at all are observed on any part of the tree:

In order to avoid that the observers have to report that there are no symptoms on the foliage, nor at the branches and the stem, this case should be reported using code 00.

c. No assessment of damage causes was made

Report code 09 in the column for specification of affected part.

- Symptoms and their specification d_symptom,  d_spec_symptom

(Chapter CC, D1)

Table A2-2: Symptoms/signs and specification of symptoms/signs; part I / II

Table A2-2: Symptoms/signs and specification of symptoms/signs; part II / II

In case that no value for symptom and/or no value for symptom specification was specified (e.g. when no assessment was done and affected part was submitted with code 09) the code "-9" should be submitted to the database in order to have a value in those key fields (see first bullet point in section "General Remarks" on key fields).

- Age of damage d_DAMAGE_AGE (since manual update 2006)

(CC)

| Code | Class damage age | Description |
| --- | --- | --- |
| 1 | Fresh | damage that has begun after the last year’s inventory |
| 2 | Old | damage that has begun earlier |
| 3 | Fresh and old | both, fresh and old damage is visible |

In case that no value for age of damage was specified (e.g. when no assessment was done and affected part was submitted with code 09) the code "-9" should be submitted to the database in order to have a value in this key field (see first bullet point in section "General Remarks" on key fields).

- Causal agents/factor D_cause

(CC)

In case that no value for age of damage was specified (e.g. when no assessment was done and affected part was submitted with code 09) the code "-9" should be submitted to the database in order to have a value in this key field (see first bullet point in section "General Remarks" on key fields).

| Agent group | Code |
| --- | --- |
| Game and grazing | 100 |
| Insects | 200 |
| Fungi | 300 |
| Abiotic agents | 400 |
| Direct action of men | 500 |
| FIRE | 600 |
| Atmospheric pollutants | 700 |
| Other factors | 800 |
| (Investigated but) unidentified | 999 |

Table A2-3: Main categories of causal agents / factors

Table A2-4: Codes for agent group 100 (game and grazing)

Table A2-5: Codes for agent group 200 (insects): Conifers

Table A2-6: Codes for agent group 200 (insects): Broadleaves

Table A2-7: Codes for agent group 300 (fungi)

Table A2-8: Codes for the agent group 400 (abiotic factors).

Table A2-9: Codes for the agent group 500 (direct action of man).

Table A2-10: Codes for the agent group 700 (atmospheric pollutants).

Table A2-11: Codes for the agent group 800 (other)

- Scientific name of cause d_cause_sc_name

(Chapter 2, 9)

If the organism involved can be identified the scientific name must be reported, using the codes of 7 letters. As a general rule the codes consist of the first 4 letters of the Genus name, followed by the first 3 letters of the species name (e.g. Lophodermium seditiosum = LOPHSED). If the Genus name has only 3 letters, these are followed by the first 4 letters of the species name (e.g. Ips typographus = IPSTYPO). Codes for the most common damaging species are listed in the internet file http://www.icp-forests.org/WGbiotic.htm  >> click on annex 3. This table also provides information on synonyms and tree species on which the damaging agents occur most frequently.

In case that no scientific name of case is specified (e.g. when no assessment was done and affected part was submitted with code 09) the code "-9" should be submitted to the database in order to have a value in this key field (see first bullet point in section "General Remarks" on key fields).

The following sources of information provide information for the field observers to facilitate the diagnosis:

- Tables A2-3 – A2-11 contain the coding system for damaging agents. Especially the sheets on insects and fungi provide information about specific symptoms caused by a selection of relevant organisms.

- http://www.icp-forests.org/WGbiotic.htm >> click on Annex 3, provides codes for the scientific names of causal agents.

- http://www.icp-forests.org/WGbiotic.htm >> click on Annex 4, provides examples, descriptions and photographs of damage caused by important categories of insects and fungi.

- http://www.icp-forests.org/WGbiotic.htm >> click on Annex 5, provides a key with symptoms linked to frequently occurring damage causes. However keep in mind that these are possible damage causes, other factors may cause similar symptoms. Diagnosis should always be confirmed by an expert phytopathologist whenever possible.

- Extent classes D_EXTENT

(Chapter 2, 9)

The damage extent will be reported in the following classes:

| Class | Code |
| --- | --- |
| 0 % | 0 |
| 1 – 10 % | 1 |
| 11 – 20 % | 2 |
| 21- 40 % | 3 |
| 41 – 60 % | 4 |
| 61 – 80 % | 5 |
| 81 – 99 % | 6 |
| 100 % | 7 |

- Sample number as tree species and leaves type d_leaves_type

(Chapter 4)

Tree species see explanatory item (20).

The definition of Leaves type:

Leaves type 0 = current = needle set 1

Leaves type 1 = current+1 = needle set 2

Leaves type 2 = older than current + 1

Leaves type 3 = older than current (e.g. for Qu. ilex)

- Tree number with initial F for Foliage, R for Ring and D for Disk sampling

(Chapter 4, 5, 10)

As in some samplings (foliar, increment, ozone injury) trees outside the normal plot (or sub plot) have to be used, special numbers have to be applied. The number of these trees will start with a letter (F=Foliage), R=Ring analysis by increment borings, D=Disc analysis) followed with a sequence number (e.g. F001). The numbers are to be reported. Ozone injury assessments are carried out on trees with foliar assessments and thus are numbered like these (e.g. F001). Only if additional trees are sampled specifically for ozone injury new codes are given (e.g. O001).

- Sample code [replaced by explanatory item (154) from 2009 onwards] D_litter_sample

(Chapter 11)

| code | description |
| --- | --- |
| 10 | Total |
| 11 | Foliar litter |
| 12 | Non foliar litter |
| 13 | Flowering |
| 14 | Fruiting/seeds |
| 15 | Budshells |
| 16 | Twigs/branches |
| 19 | Other biomass |

- Mass of 100 leaves or of 1000 needles

(Chapter 4, 11)

The mass is determined of 100 leaves or 1000 needles (oven-dry) in grams

- Diameter at breast height (DBH)

(Chapter 5)

The diameter at breast height (1.30 m) over bark in 0.1 centimetres.

When a diameter tape is used a single value will be needed. When calipers are used the maximum and the minimum diameter (over bark) shall be determined and reported (diameter 1 and diameter 2).

- Bark

(Chapter 5)

The thickness of the bark at 1.30 m, expressed in centimetres with one decimal.

- Height rounded off to the nearest 0.1 meters

(Chapter 5)

The height of the tree expressed in metres to the nearest 0.1 metres.

- Tree volume

(Chapter 5)

Based on the measured diameter(s) and height, the tree volume can be estimated using locally known form factors or through the use of valid volume tables. The tree volume shall be expressed in cubic metres (m3) with three decimals. The formula used to compute tree volume and the minimum diameter used for the volume calculation should be reported in the data accompanying form.

- Crownlength rounded off to the nearest 0.1 meters

(Chapter 5)

The length of the crown to the nearest 0.1 metres is determined from the tip of the stem to the lowest live branch excluding water shoots.

- Crownwidth rounded off to the nearest 0.1 meters

(Chapter 5)

The average crown width is determined by the average of at least four crown radii, multipled by two, and to the nearest 0.1 metres.

- Diameter under bark

(Chapter 5)

The actual diameter under bark is calculated as the diameter over bark deducted with the width of the bark at the two sides. The diameter under bark of five years ago is calculated as the actual diameter under bark less the increment of the last five years of the tree at both sides. The diameter under bark is expressed in 0.1 centimetres.

- Basal area per plot

(Chapter 5)

The actual basal area per plot is calculated as the total basal areas of all the trees in the plot. The basal area per plot of five years ago is calculated on the basis of the estimated diameter under bark of five years ago of all trees in the plot. Basal area per plot is expressed in 0.1 m2.

- Volume per plot

(Chapter 5)

The actual volume per plot is calculated as the total volume of all the trees in the plot. The volume per plot of five years ago is calculated on the basis of the estimated diameter under bark of five years ago of all the trees in the plot. Volume per plot is expressed in 0.1 m3.

- Thinning

(Chapter 5)

If a thinning has taken place in the five-year period between the two years of determination of diameter, basal area per plot and volume per plot, this will be indicated (Yes = 1, No = 0). In an additional part the details of this thinning will be described as detailed as possible (including: thinning method, exact year of thinning, thinning intensity expressed as number of trees, basal area/ha, volume/ha).

- Deposition sampler code d_sampler_deposition

(Chapter 6)

The following codes shall be used for the samplers for deposition:

| Samplers for deposition | Code |
| --- | --- |
| Throughfall | 1 |
| Bulk deposition | 2 |
| Wet-only deposition | 3 |
| Stemflow | 4 |
| Fog | 5 |
| Frozen fog (rime) | 6 |
| Air concentration | 7 |
| Others | 9 |

- First or last date of monitoring period

(Chapter 3b, 6, 7, 10a)

The first and final dates of each monitoring period shall be stated on the forms, using the same format as the date of observation, assessment and analysis (see item(3)).

A monitoring period shall consist of one or more measuring periods. The measuring periods within one monitoring period should have the same length. The minimum length of a measuring period is one week, the maximum one month.

When it is necessary to use different measuring periods during the year (e.g. weekly in summer and monthly in winter), two separate monitoring periods shall be identified and the results shall be reported separately on the forms.

- Number of measuring periods

(Chapter 3b, 6, 7)

The number of measuring periods in each monitoring period shall be indicated in the forms.

- Period

(Chapter 3b, 6)

The measuring period number in which the sample has been collected shall be stated. Each year (on or around 1 January) a new set of measuring periods will be stated. When samples from several measuring periods are combined before analysis, the exact details of the mixing shall be stated in the Annex to the document with background information. The number of the first measuring period shall be used to indicate the period for analysis (e.g. when the samples from period 9, 10, 11, and 12 are combined into a single sample for the analysis, this sample will be given the period number 9).

- Sampler Model – national / harmonised D_SAMPLER_DEPOSITION_HARMON

(Chapter 6)

| Sampler Model | Code |
| --- | --- |
| National | 1 |
| Harmonised | 2 |

- Sampler Height – deposition

(Chapter 6)

The height of the collecting surface above ground level in meters.

- Sampler Surface – deposition

(Chapter 6)

The area of the collecting surface for one sampler in m². In case of stemflow submit the basal area of the sampled trees which is also used for calculation of stemflow values (see Annex 3 in deposition sub-manual).

- V_Sampling – missing/adjusted/estimated samples D_SAMPLING_DEPOSITION

(Chapter 6)

Code to be used to explain missing/adjusted/estimated samples:

| code | description |
| --- | --- |
| 1 | normal sampling |
| 2 | contamination |
| 3 | sampler destroyed |
| 4 | sampling not performed |
| 5 | overflow, minimal precipitation derived from sampler volume given in 'quantity' |
| 6 | overflow, estimated precipiation (e.g. from meteorological station) given in 'quantity', estimation method specified in column 'Observations' or in the DAR-Q files |
| 7 | too little volume for any analyses |
| 8 | too little volume for analyses of SOME parameters |
| 9 | too little volume for any analyses; sample pooled with following sample period |

- Plot/instrument code

(Chapter 7)

All instruments that are installed in or near the plot are given an observation plot/instrument code. This code consists of the plot number (up to four digits) and a sequential number for all instruments (up to 99).

When instruments are replaced or added, new codes are applied (e.g. the fifth instrument in plot 1234 will thus receive code 1234.05).

- Location d_location

(Chapter 7)

The location of the instrument is indicated:

| Code | Location of the instrument |
| --- | --- |
| S | instrument is located on site, i.e. in (the bufferzone) of the plot. This could be under the canopy, above the canopy or in the forest soil. |
| F | instrument is located in a (nearby) open field in the forest area. |
| W | instrument is located at a weather station (in general outside the forest area). |
| O | instrument is located somewhere else. |

- Variable d_variable

(Chapter 7)

Indication of the variable that is measured with the instrument

| Code | Variable measured |
| --- | --- |
| AT | Air temperature (°C; mean, min and max value) |
| PR | Precipitation (sum of daily values; total precipitation including snow, etc.) |
| RH | Relative humidity (%; mean, min and max value) |
| WS | Wind speed (m/s; mean and max value) |
| WD | Wind direction (°; prevailing wind direction, 0o = North, 270o= West) |
| SR | Solar radiation (W/m²; mean values) |
| UR | UV b radiation (W/m²; mean values) |
| TF | Throughfall (mm; sum of daily values) |
| SF | Stemflow (mm; sum of daily values) |
| ST | Soil temperature (°C; mean, min and max value) |
| MP | Matric potential in the soil (khPa; mean, min and max value) |
| WC | Water content in the soil (m³∙m-³; mean, min and max value) |
| XX | Other codes for additional variables my be used, but should be specified in the DAR-Q; code to be specified with data base management before submission |

- Instrument information D_INSTRUMENT

(Chapter 7)

Vertical position

The vertical position (height or depth) of the instruments shall be indicated in meters with a plus (=height above the ground) or a minus sign (depth below the ground) using the format of 2 digits and 2 decimals.

Instrument code

The following codes shall be used for the samplers and recording method of data:

| Samplers and recording method of data | Code |
| --- | --- |
| Manual reading and recording on paper | 10 |
| Mechanical recording (manual reading and recording on paper) | 20 |
| Direct paper recording | 30 |
| Digital recording (in stand alone situation) | 40 |
| Digital recording (integrated datalogger) | 50 |

Details on the equipment shall be stated in the Data Accompanying Report.

Scanning interval in seconds (automatic instruments only).

The interval between two consecutive data storage moments shall be stated in minutes.

- Variables to be assessed in the meteorological monitoring

(Chapter 7)

Precipitation and throughfall

The precipitation will be stated as the daily sum in mm.

Temperature (air and soil)

The temperature will be stated in oC. The daily mean, daily minimum and daily maximum are to be submitted.

Relative Humidity

The relative humidity will be stated as the daily mean, the minimum and the maximum value reached per day.

Wind speed

The wind speed will be stated as the daily mean and the maximum value reached per day.

Wind direction

The wind direction will be stated as the prevailing wind per day. The windrose will be split into 12 sections of 30o starting from 15o onwards. The most frequent wind direction is reported by its middle value e.g.: 30 o = the sector 15 o -45 o, 60 o for the sector 45 o -75 o, 90 o for the sector 75 o -105 o, etc. , 0 ° = the sector 345° - 15°

Solar radiation and UV b radiation

The solar radiation and the UV b radiation will be stated as the daily mean value.

Stemflow

The stemflow will be calculated to mm precipitation and will be stated as the daily sum.

Matric potential in the soil

The matric potential in the soil will be stated in hPa, as the daily mean, the minimum and the maximum value reached per day.

Water content in the soil

The water content in the soil will be stated in Vol. % as the daily mean, the minimum and the maximum value reached per day.

- Completeness

(Chapter 7)

The completeness is an indicator of the coverage of the scanning and storing procedures and is stated in % using the format of up to three digits ("100" = 100% = complete).

- Origin of data d_origin_meteo

(Chapter 7, Meteo)

| Code | Origin |
| --- | --- |
| 1 | Data measured on plot |
| 2 | Data measured on nearby meteo station |
| 3 | Modelled data for gap filling |
| 4 | Modelled data |
| 9 | Missing value (data field must be blank) |

- Status of data d_status_data

(Chapter 7, Meteo)

| Code | Status |
| --- | --- |
| 1 | Raw data / not calibrated |
| 2 | Validated data, not calibrated |
| 3 | Validated data, calibrated |
| 9 | Missing value (data field must be blank) |

- Codes for FAO Texture Classes d_texture_class

(Chapter 3b)

The FAO Texture Triangle distinguished 12 classes of which the codes are given below (FAO, 1990):

| Texture Class | Code |
| --- | --- |
| Clay | C |
| Loam | L |
| Clay loam | CL |
| Silt | Si |
| Silt loam | SiL |
| Silty clay | SiC |
| Silty clay loam | SiCL |
| Sandy clay | SC |
| Sandy clay loam | SCL |
| Sandy loam | SL |
| Loamy sand | LS |
| Sand | S |

- PFH pit ID (same as in BioSoil)

(Soil Water, Soil)

In general, samples from at least 3 pits on each depth with replicates are taken for the soil water determination. The PFH pit ID is the Pit ID of the main pit on the plot for which the profile description was made and submitted with the PRF and the PFH file. In case that the profile description was made already during the BioSoil project or during a comparable assessment it is the same ID.

The linkage between Soil Water data (SWC form) and Soil Profile description data (PRF and PFH) is built by a combination of the data fields <country code>, <plot number>, <PRF pit ID>, and <Horizon number>.

- SW pit ID

(Soil Water)

In general, samples from at least 3 pits on each depth with replicates are taken for the soil water determination. The Pit ID of the actual Soil Water pit indicates where the soil water sample was taken. In case that this pit is the one on the plot for which the profile description was made (submitted with forms .PRF and .PFH) it is the same as the PFH pit ID (see (69)). In all other cases it is a new one.

Soil Water data from SWC and SWA forms are linked using the combination of the data fields <country code>, <plot number>, <SW pit ID> (70), <code depth layer> (71), and <replicate>.

- Code for depth level – Layers Soil Water d_depth_level_sw

(Soil Water)

The linkage between the SWA and the SWC table is built by the key parameters of SWC. This means that each observation/line in SWA can be linked to exactly one observation/line in SWC by the combination of country, plot, date, SW_pit, depth_layer, and replicate. Each line in SWC is identified by such a specific combination of the respective values. Each line in SWA is identified by such a specific combination of the respective values and, in addition, a specific value for matric pressure.

This data model implies that the depth layer code must be more or less unique. It is not possible to have to samples (one from 10 to 20 cm and one from 15 to 20cm) at the same pit with the identic depth layer code „M12“. Only 3 codes are fixed which are the mandatory depth layers M02, M24, and M48 (see table below). All others have to be defined by the submitting partner, making sure that each combination of country, plot, date, SW_pit, depth_layer, and replicate is unique in SWC and each observation in SWA can be linked to exactly one observation in SWC.

In general, the depth layer code is built by an “O“ in case of organic layers or a “M” in case of mineral layers. Each succeed by two numbers. The first number is the decimeter of the upper limit and the second number it the decimeter of the lower limit.

Example:

A mineral layer from 19 cm to 22 cm would be “M12”, the layer below from 22 to 35cm would be “M23”. A layer below from 35 to 38cm would be “M33” and the next from 38 to 51cm would be “M35”. The depth layer codes M02, M24, and M48 are exclusively defined for the upper and lower limits in the table below:

| Example for Code | Upper limit | Lower limit | Layers description |
| --- | --- | --- | --- |
| Oxx |  |  | Organic layer (Forest floor > 5 cm thick  mandatory) |
| O10 | -10 | 0 | Sample ring depth lower limit is top of mineral layer (depth is “0”) and upper limit is 10 cm above (value “-10”) |
| Mxx |  |  | Mineral layer |
| M01 | 0 | 10 | Mineral soil between 0 and 10 cm (optional) |
| M12 | 10 | 20 | Mineral soil between 10 and 20 cm (optional) |
| M02 | 0 | 20 | Mineral soil between 0 and 20 cm (mandatory) |
| M24 | 20 | 40 | Mineral soil between 20 and 40 cm (mandatory) |
| M48 | 40 | 80 | Mineral soil between 40 and 80 cm (mandatory) |
| M81 | 80 | 100 | Mineral soil below 80cm to 1m (optional) |
| M82 | 80 | 200 | Mineral soil below 80cm to 2m (optional) |

The code is specified in order to allow for a fast evaluation of mandatory soil water data classified as foreseen in the above table. The sample ring depths allow a more detailed description.

Examples for SWC and SWA tables and the linkage of data between them:

!SWC

!Sequence, country, plot, date, PFH_pit, horizon, SW_pit, depth_layer, ring_depth_upper, ring_depth_lower, replicate, bulk_density, date_analysis, other_observations

1 22  901 010709 901A  1  901A M02   0  20 1  810 010809 testdata

2 22  901 020709 901B  1  901B M02   0  20 1  820 020809 testdata

3 22  901 050709 901B  2  901B M24  20  40 1  850 050809 testdata

….

The data line below (sequence number 142) is linked by the values for country, plot, date, SW_pit, depth_layer, and replicate to the second observation in SWC:

!SWA

!Sequence, country, plot, date, SW_pit, depth_layer, replicate, water_content, matric_pressure, date_analysis, other_observations

….

142 22  901 020709 901B M02 1   0.15       -1 020809 testdata

….

- Code for depth level – Layers D_DEPTH_LEVEL_Soil

(Chapter 3a, 3b)

| CODE | description |
| --- | --- |
| OL | Not saturated organic layer, litter horizon |
| OF | Not saturated organic layer, fermented horizon |
| OFH | Not saturated organic layer, fermented plus humus horizon (sampled together if OH horizon is not thicker than 1 cm) |
| OH | Not saturated organic layer, humus horizon |
| Hf | Saturated organic layer, poorly decomposed |
| Hfs | Saturated organic layer, fragmentized and partly oxidized |
| Hs | Saturated organic layer, well decomposed |
| H05 | Organic (peat) soil between 0 and 5 cm |
| H51 | Organic (peat) soil between 5 and 10 cm |
| H01 | Organic (peat) soil between 0 and 10 cm |
| H12 | Organic (peat) soil between 10 and 20 cm |
| H24 | Organic (peat) soil between 20 and 40 cm |
| H48 | Organic (peat) soil between 40 and 80 cm |
| M05 | Mineral soil between 0 and 5 cm |
| M51 | Mineral soil between 5 and 10 cm |
| M01 | Mineral soil between 0 and 10 cm |
| M12 | Mineral soil between 10 and 20 cm |
| M24 | Mineral soil between 20 and 40 cm |
| M48 | Mineral soil between 40 and 80 cm |

- Code for Parent Material (after Lambert et al., 2003(*) d_parent_material

(Chapter 3a)

The parent material code must be selected from the list provided below. It includes four levels: Major Class, Group, Type and Subtype.

Depending on the level of detail available to describe the dominant and secondary parent materials, i.e. Major Class or Group or Type or Sub-type, the user will choose any one of the codes provided in the table.

Whenever possible, it is recommended to identify as precisely as possible the exact type of parent material, using the full 4 digit code. For example, calcareous sandstone (1211) is preferable to sandstone (1210) or to psammite (1200). The later should be used either if the type of sandstone has can precisely been defined, or when more than one type of sandstone is present in the plot.

| Major Class level |  | Group level |  | Type level |  | Sub-type level |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0000 | No information | 0000 | No information | 0000 | No information | 0000 | No information |
| 1000 | consolidated-clastic- sedimentary rocks | 1100 | psephite or rudite | 1110 | conglomerate | 1111 | pudding stone |
|  |  |  |  | 1120 | breccia |  |  |
|  |  | 1200 | psammite or arenite | 1210 | sandstone | 1211 | calcareous sandstone |
|  |  |  |  |  |  | 1212 | ferruginous sandstone |
|  |  |  |  |  |  | 1213 | clayey sandstone |
|  |  |  |  |  |  | 1214 | quartzitic sandstone / orthoquartzite |
|  |  |  |  |  |  | 1215 | micaceous sandstone |
|  |  |  |  | 1220 | arkose |  |  |
|  |  |  |  | 1230 | graywacke | 1231 | feldspathic graywacke |
|  |  | 1300 | pelite, lutite or argilite | 1310 | claystone / mudstone | 1311 | Kaolinite |
|  |  |  |  |  |  | 1312 | Bentonite |
|  |  |  |  | 1320 | siltstone |  |  |
|  |  | 1400 | facies bound rock | 1410 | flysch | 1411 | sandy flisch |
|  |  |  |  |  |  | 1412 | clayey and silty flysch |
|  |  |  |  |  |  | 1413 | conglomeratic flysch |
|  |  |  |  | 1420 | molasse |  |  |
| 2000 | sedimentary rocks (chemically precipitated, evaporated, or organogenic or biogenic in origin) | 2100 | calcareous rocks | 2110 | limestone | 2111 | hard limestone |
|  |  |  |  |  |  | 2112 | soft limestone |
|  |  |  |  |  |  | 2113 | marly limestone |
|  |  |  |  |  |  | 2114 | chalky limestone |
|  |  |  |  |  |  | 2115 | detrital limestone |
|  |  |  |  |  |  | 2116 | carbonaceous limestone |
|  |  |  |  |  |  | 2117 | lacustrine or freshwater limestone |
|  |  |  |  |  |  | 2118 | Travertine/calcareous sinter |
|  |  |  |  |  |  | 2119 | Cavernous limestone |
|  |  |  |  | 2120 | dolomite | 2121 | Cavernous dolomite |
|  |  |  |  |  |  | 2122 | calcareous dolomite |
|  |  |  |  | 2130 | marlstone |  |  |
|  |  |  |  | 2140 | marl | 2141 | chalk marl |
|  |  |  |  |  |  | 2142 | gypsiferous marl |
|  |  |  |  | 2150 | chalk |  |  |
|  |  | 2200 | evaporites | 2210 | gypsum |  |  |
|  |  |  |  | 2220 | anhydrite |  |  |
|  |  |  |  | 2230 | halite |  |  |
|  |  | 2300 | siliceous rocks | 2310 | chert, hornstone, flint |  |  |
|  |  |  |  | 2320 | diatomite / radiolarite |  |  |
| 3000 | igneous rocks | 3100 | acid to intermediate plutonic rocks | 3110 | granite |  |  |
|  |  |  |  | 3120 | granodiorite |  |  |
|  |  |  |  | 3130 | diorite | 3131 | quartz diorite |
|  |  |  |  |  |  | 3132 | gabbro diorite |
|  |  |  |  | 3140 | syenite |  |  |
|  |  | 3200 | basic plutonic rocks | 3210 | gabbro |  |  |
|  |  | 3300 | ultrabasic plutonic rocks | 3310 | peridotite |  |  |
|  |  |  |  | 3320 | pyroxenite |  |  |
|  |  | 3400 | acid to intermediate volcanic rocks | 3410 | rhyolite | 3411 | Obsidian |
|  |  |  |  |  |  | 3412 | quartz porphyrite |
|  |  |  |  | 3420 | dacite |  |  |
|  |  |  |  | 3430 | andesite | 3431 | porphyrite (interm,) |
|  |  |  |  | 3440 | phonolite | 3441 | tephritic phonolite |
|  |  |  |  | 3450 | trachyte |  |  |
|  |  | 3500 | basic to ultrabasic volcanic rocks | 3510 | basalt |  |  |
|  |  |  |  | 3520 | diabase |  |  |
|  |  |  |  | 3530 | pikrite |  |  |
|  |  | 3600 | dike rocks | 3610 | aplite |  |  |
|  |  |  |  | 3620 | pegmatite |  |  |
|  |  |  |  | 3630 | lamprophyre |  |  |
|  |  | 3700 | pyroclastic rocks (tephra) | 3710 | tuff/tuffstone | 3711 | agglomeratic tuff |
|  |  |  |  |  |  | 3712 | block tuff |
|  |  |  |  |  |  | 3713 | lapilli tuff |
|  |  |  |  | 3720 | tuffite | 3721 | sandy tuffite |
|  |  |  |  |  |  | 3722 | silty tuffite |
|  |  |  |  |  |  | 3723 | clayey tuffite |
|  |  |  |  | 3730 | volcanic scoria/ volcanic breccia |  |  |
|  |  |  |  | 3740 | volcanic ash |  |  |
|  |  |  |  | 3750 | ignimbrite |  |  |
|  |  |  |  | 3760 | pumice |  |  |
| 4000 | metamorphic rocks | 4100 | weakly metamorphic rocks | 4110 | (meta-)shale / argilite |  |  |
|  |  |  |  | 4120 | slate | 4121 | graphitic slate |
|  |  | 4200 | acid regional metamorphic rocks | 4210 | (meta-)quartzite | 4211 | quartzite schist |
|  |  |  |  | 4220 | phyllite |  |  |
|  |  |  |  | 4230 | micaschist |  |  |
|  |  |  |  | 4240 | gneiss |  |  |
|  |  |  |  | 4250 | granulite (sensu stricto) |  |  |
|  |  |  |  | 4260 | migmatite |  |  |
|  |  | 4300 | basic regional metamorphic rocks | 4310 | greenschist | 4311 | Prasinite |
|  |  |  |  |  |  | 4312 | Chlorite |
|  |  |  |  |  |  | 4313 | talc schist |
|  |  |  |  | 4320 | amphibolite |  |  |
|  |  |  |  | 4330 | eclogite |  |  |
|  |  | 4400 | ultrabasic regional metamorphic rocks | 4410 | serpentinite | 4411 | greenstone |
|  |  | 4500 | calcareous regional metamorphic rocks | 4510 | marble |  |  |
|  |  |  |  | 4520 | calcschist, skam |  |  |
|  |  | 4600 | rocks formed by contact metamorphism | 4610 | contact slate | 4611 | nodular slate |
|  |  |  |  | 4620 | hornfels |  |  |
|  |  |  |  | 4630 | calsilicate rocks |  |  |
|  |  | 4700 | tectogenetic metamorphism rocks or cataclasmic metamorphism | 4710 | tectonic breccia |  |  |
|  |  |  |  | 4720 | cataclasite |  |  |
|  |  |  |  | 4730 | mylonite |  |  |
| 5000 | unconsolidated deposits (alluvium, weathering residuum and slope deposits) | 5100 | marine and estuarine sands | 5110 | pre-quaternary sand | 5111 | tertiary sand |
|  |  |  |  | 5120 | quaternary sand | 5121 | holocene coastal sand with shells |
|  |  |  |  |  |  | 5122 | delta sand |
|  |  | 5200 | marine and estuarine clays and silts | 5210 | pre-quaternary clay and silt | 5211 | tertiary clay |
|  |  |  |  |  |  | 5212 | tertiary silt |
|  |  |  |  | 5220 | quaternary clay and silt | 5221 | Holocene clay |
|  |  |  |  |  |  | 5222 | Holocene silt |
|  |  | 5300 | fluvial sands and gravels | 5310 | river terrace sand or gravel | 5311 | river terrace sand |
|  |  |  |  |  |  | 5312 | river terrace gravel |
|  |  |  |  | 5320 | floodplain sand or gravel | 5321 | floodplain sand |
|  |  |  |  |  |  | 5322 | floodplain gravel |
|  |  | 5400 | fluvial clays, silts and loams | 5410 | river clay and silt | 5411 | terrace clay and silt |
|  |  |  |  |  |  | 5412 | floodplain clay and silt |
|  |  |  |  | 5420 | river loam | 5421 | terrace loam |
|  |  |  |  | 5430 | overbank deposit | 5431 | floodplain clay and silt |
|  |  |  |  |  |  | 5432 | floodplain loam |
|  |  | 5500 | lake deposits | 5510 | lake sand and delta sand |  |  |
|  |  |  |  | 5520 | lake marl, bog lime |  |  |
|  |  |  |  | 5530 | lake silt |  |  |
|  |  | 5600 | residual and redeposited loams from silicate rocks | 5610 | residual loam | 5611 | stony loam |
|  |  |  |  |  |  | 5612 | clayey loam |
|  |  |  |  | 5620 | redeposited loam | 5621 | running-ground |
|  |  | 5700 | residual and redeposited clays from calcareous rocks | 5710 | residual clay | 5711 | clay with flints |
|  |  |  |  |  |  | 5712 | ferruginous residual clay |
|  |  |  |  |  |  | 5713 | calcareous clay |
|  |  |  |  |  |  | 5714 | non-calcareous clay |
|  |  |  |  |  |  | 5715 | marly clay |
|  |  |  |  | 5720 | redeposited clay | 5721 | stony clay |
|  |  | 5800 | slope deposits | 5810 | slope-wash alluvium |  |  |
|  |  |  |  | 5820 | colluvial deposit |  |  |
|  |  |  |  | 5830 | talus scree | 5831 | Stratified slope deposits |
| 6000 | unconsolidated glacial deposits / glacial drift | 6100 | morainic deposits | 6110 | glacial till | 6111 | boulder clay |
|  |  |  |  | 6120 | glacial debris |  |  |
|  |  | 6200 | glaciofluvial deposits | 6210 | outwash sand, glacial sand |  |  |
|  |  |  |  | 6220 | outwash gravels glacial gravels |  |  |
|  |  | 6300 | glaciolacustrine deposits | 6310 | varves |  |  |
| 7000 | eolian deposits | 7100 | loess | 7110 | loamy loess |  |  |
|  |  |  |  | 7120 | sandy loess |  |  |
|  |  | 7200 | eolian sands | 7210 | dune sand |  |  |
|  |  |  |  | 7220 | cover sand |  |  |
| 8000 | organic materials | 8100 | peat (mires) | 8110 | rainwater fed moor peat (raised bog) | 8111 | folic peat |
|  |  |  |  |  |  | 8112 | fibric peat |
|  |  |  |  |  |  | 8113 | terric peat |
|  |  |  |  | 8120 | groundwater fed bog peat |  |  |
|  |  | 8200 | slime and ooze deposits | 8210 | gyttja, sapropel |  |  |
|  |  | 8300 | carbonaceaous rocks (caustobiolite) | 8310 | lignite (brown coal) |  |  |
|  |  |  |  | 8320 | hard coal |  |  |
|  |  |  |  | 8330 | anthracite |  |  |
| 9000 | anthropogenic deposits | 9100 | redeposited natural materials | 9110 | sand and gravel fill |  |  |
|  |  |  |  | 9120 | loamy fill |  |  |
|  |  | 9200 | dump deposits | 9210 | rubble/rubbish |  |  |
|  |  |  |  | 9220 | industrial ashes and slag |  |  |
|  |  |  |  | 9230 | industrial sludge |  |  |
|  |  |  |  | 9240 | industrial waste |  |  |
|  |  | 9300 | anthropogenic organic materials |  |  |  |  |

(*) J.J. Lambert, J. Daroussin, M. Eimberck, C. Le Bas, M. Jamagne, D. King and L. Montanarella. 2003. Soil Geographical Database for Eurasia & The Mediterranean: Instructions Guide for Elaboration at scale 1:1,000,000, Version 4.0. European Soil Bureau  Research Report N°8.  EUR 20422 EN 64 pp. Office for Official Publications of the European Communities, Luxembourg.

- Sampler number

(Chapter 3b)

The samplers in the plot shall be numbered in a permanent way (1-99).

- Sampler code d_sampler_ss (ffprd_fmd_soil_sampler_type)

(Chapter 3b)

The following codes shall be used for the samplers for soil solution:

| Code | Samplers for soil solution |
| --- | --- |
| 1 | Tension lysimeter |
| 2 | Zero tension lysimeter |
| 3 | Centrifugation |
| 4 | Saturation extraction |
| 9 | Other |

- Sampling depth

(Chapter 3b)

The sampling depth in metres below the surface of the mineral soil (e.g. -0.40).

- Survey number

(Chapter 8)

Each time (day), or situation (inside outside fence), that an assessment of the ground vegetation is made on a given plot, a survey number is given (identical on both PLV and VEM forms). By combining the plot number with the survey number a unique plot/survey number is created.

- Fencing d_fence

(Chapter 8)

As the vegetation can be very different inside and outside a fence, it was decided that in principle the ground vegetation is surveyed always outside the fence. In case that also inside the fence a survey is carried out this should be reported as a separate survey and the fencing code be indicated:

1	Yes, survey within the fence

2 	No, survey was done outside fenced area.

- Total sampled area

(Chapter 8)

The total sampled area (CSA) shall be 400 m2. In the Data Accompanying Report the exact details of the number of repetitions and the location/orientation of the ground vegetation plots shall be given.

- Height and cover of layers

(Chapter 8)

The estimated cover of the tree layer, the shrub layer, the herb layer and the moss layer shall be submitted as % of the total sampled area.

The estimated cover of bare soil and litter shall be submitted as separate “layers” as % of the total sampled area (effective surfaces covered by visible mineral soil and rocks and visible litter (even if under shrubs, herbs or trees). The submission is done using up to 4 digits for cover values with a floating decimal separator (between 0.01 and 9999).

The average height of the layers shall be given in meters using up to 4 digits

|  | height (in m) | cover (in %) |
| --- | --- | --- |
| Tree layer | ======== | x |
| Shrub layer | x | x |
| Herb layer | x | x |
| Moss layer | ======== | x |
| Bare soil “layer” | ======== | x |
| Litter “layer” | ======== | x |

- Layers D_layer_surface

(Chapter 8)

The following layers are defined.

| Code | Layer |
| --- | --- |
| 1 | Tree layer (only ligneous and all climbers) > 5 m height |
| 2 | Shrub layer (only ligneous an all climbers) > 0.5 m height |
| 3 | Herb layer (all non-ligneous, and ligneous < 0.5m height) |
| 4 | Moss layer (i.e. terricolous bryophytes and lichens) |

- Species code d_species_list noch ausstehend (>og)

(Chapter 8, 10)

The species code exists of the code for the family (999), the code for the genus (999), the code for the species (999). Family, genus, and species codes are separated by a dot (“.”). Determination at Genus level, using ‘999’ as the code for sp/spp (‘species pluralis’), is the minimal submission requirement; the “Other observations” text field should be used for additional information (e.g. if there are obvious more than one unknown species of the same Genus within the plot).

The coded lists for vascular plants and cryptogams are available via the internet page of the responsible Expert Panel of ICP Forests (www.icp-forests.org/EPbiodiv.htm). In case that species will occur that are not included on this list, the National Focal Centre will take contact with the Expert Panel prior to the formal data submission to the European data centre. The Expert Panel will assign a new (999.999.999) code and include it on the list available on the internet. The new species will be submitted to the European data centre with the new code. An additional list of nationally important or problematic species can be prepared and maintained by the NFC if regarded necessary.

- Cover of plant species

(Chapter 8)

Countries are free in the assessment of the abundance/cover of the plant species. The submission of this cover is done in % using up to 5 digits with a floating decimal separator (between 0.01, 99.99 and 100.0). In the DAR the complete assessment methods, as well as the adopted conversion to % shall be specified.

- Other observations

(Chapter 1, 2, 3a, 3b, 4, 5, 6, 7, 8, 9, 10a, 10b, 11)

Any supplementary comments.

- Description of instrument

(Chapter 7)

Text format

- Quality information from the laboratory

(Chapters 4, 6)

Not longer valid:

Laboratory ringtests are carried out under the responsibility of the Expert Panels/Working Groups of the ICP Forests. Expert Panels and the Programme Coordinating Centre of ICP Forests maintain lists with ringtest results per laboratory. These results can be linked to the data sets in the database. If there is no ringtest for the current year, the ringtest number from the previous test has to be used. If the laboratory did not participate in the most recent ringtest, a blanc shall be used.

For all surveys with parameters which are measured at laboratories so called “Laboratory Quality Assurance” Forms (.LQA) are defined (s. section 4) and have to be submitted with the data of the respective survey.

- Sample quantity

(Chapter 6)

In the DEM and DEO forms, the submission of with no analysis (e.g. no precipitation, no analytical results) should be distinguished from periods with missing data:

- For periods without precipitation, the start and end date has to be filled in and the column “sample quantity” has to be filled in with “0”, leaving all the other columns blank and in the column “Observations” the following remark has to be made “no precipitation”

- For periods with missing data (e.g. lost samples), the start and end date has to be filled in and the precipitation quantity has to be indicated (if available) and in the column “Observations” the following remark has to be made “sample lost”

- Removals and mortality d_removal_mortality_ccGR; field “applicable_Growth” = y

(Chapter 5)

| Tree alive and diameter or circumference measurable (note this is different than a missing value) |  |
| --- | --- |
| 01 | tree alive, in current and previous inventory (formerly blank) |
| 02 | new alive tree (ingrowth) |
| 03 | alive tree (present but not assessed in previous inventory) |
| 04 | alive tree but tree not longer in growth sample due to heavy disturbances (e.g. heavy storm damage) |
| Tree removed, disappeared |  |
| 11 | planned utilization (as in CC) |
| 12 | utilization for biotic reason (as in CC) |
| 13 | utilization for abiotic reason (as in CC) |
| 14 | cut, reason unknown |
| 18 | reason for disappearance unknown (as in CC) |
| Tree still alive and standing, but no tree crown measurements taken or height measurements should not be used in stand or growth calculations. |  |
| 21 | lop-sided or hanging tree (as in CC) |
| 22 | not applicable, use 24 or 25 instead |
| 23 | not applicable |
| 24 | breakage of the tip(s) of the tree (shoot) |
| 25 | tree not in height growth sample |
| 29 | other reasons, specify |
| Standing dead (at least 1.3 m in height) |  |
| 31 | tree with intact crown, biotic reason (as in CC) |
| 32 | tree with intact crown, abiotic reason (as in CC) |
| 33 | crown breakage |
| 34 | stem breakage, below crown base and above 1.3 m |
| 38 | tree with intact crown, unknown cause of death (as in CC) |
| Fallen alive or dead, (height below 1.3 m or tree stem or crown touches the ground at one place) |  |
| 41 | abiotic reasons (as in CC) |
| 42 | biotic reasons (as in CC) |
| 48 | unknown cause (as in CC) |

In cases that more than two remarks are needed, chose the most important remark and explain the second remark in the comment column: for example a loop-sided ingrowth tree without height measurements would receive the code 02 and comments 21: loop-sided.

- Compound air quality d_compound_aq

(Chapter 10)

| Code | Description | Unit |
| --- | --- | --- |
| NH3 | NH3 | µg NH3/m³ |
| NO2 | NO2 | µg NO2/m³ |
| O3 | O3 | ppb |
| SO2 | SO2 | µg SO2/m³ |

- Variable codes (not needed from 2009 onwards) D_variable_aq

(Chapter 10)

The variable codes are compound specific:

|  | O3 | NH3 | NO2 | SO2 |
| --- | --- | --- | --- | --- |
|  | ppb or ppbh | µg_N/m3 | µg_N/m3 | µg_S/m3 |
| Mean concentration | O3_MC | NH3_MC | NO2_MC | SO2_MC |
| MC daytime (8 - 20) | O3_MCd | NH3MCd | NO2MCd | SO2MCd |
| MC nighttime (20-8) | O3_MCn | NH3MCn | NO2MCn | SO2MCn |
| Max. Concentration | O3_Max | NH3Max | NO2Max | SO2Max |
| AOT40 | AOT40 |  |  |  |
| AOT60 | AOT60 |  |  |  |

- Plot number in Air Quality files d_sampler_passive

(Air Quality)

See also (2) for format of this field. From 2009 onwards plot number is specified instead of active station ID. Each active sampler device is identified by plot number and sampler_ID (see (92)). These parameters are also used for link to the data submitted with form .AQA.

General situation:

In forms PAC, PPS, AQP and AQA passive sampler data (PPS and AQP) and hourly continuous analyzer data (PAC and AQA) measured at Intensive Monitoring Plots [IMP] are reported. Besides measurements taken at IMPs, passive samplers can also be co-located with continuous analyzers at Air Quality Stations [AQ_ST] (e.g. regional or EMEP Air Quality Stations, but not IMP) for quality control. In this case, for each of the exposure periods of the passive samplers (typically 2-weeks), both the measured value and its corresponding mean value measured with continuous analyzers for the same period have to be reported under form COL. If there are IMP with continuous analyzers and passive samplers measuring in parallel, then, both forms PAC, PPS, AQP and AQA, and COL as well have to be filled. In form COL, AQ_STs are identified with an S before the number (e.g. S004), while IMPs are identified only with plot numbers (e.g. 102).

- Sampler ID

(Air Quality)

For continuous analyzers: If for a given pollutant and plot measurements are taken with more than one continuous analyzer, identify them with a different sampler_ID (e.g. with succesive numbers: 01, 02, 03, ...)

For passive samplers: If for a given pollutant and plot, measurements are taken using several passive samplers (replicates), identify them with a different sampler_ID (e.g. with succesive numbers: 01, 02,03, ...)

- Passive sampler manufacturer

(Air Quality)

| Passive sampler manufacturer | Code |
| --- | --- |
| CEAM with shelter | 01 |
| CEAM without shelter | 02 |
| Gradko with shelter | 03 |
| Gradko without shelter | 04 |
| Gradko combined SO2/NO2 with shelter | 05 |
| Gradko combined SO2/NO2 without shelter | 06 |
| IVL with shelter | 07 |
| IVL without shelter | 08 |
| Ogawa with shelter | 09 |
| Ogawa without shelter | 10 |
| Passam with shelter | 11 |
| Passam without shelter | 12 |
| Others with shelter (specify in "Observations") | 13 |
| Others without shelter (specify in "Observations") | 14 |

- Start Hour air quality measurement

(Air Quality)

First Hour of measurement period (.PAC) or hour of measurement data (form .AQA), respectively.

“00” to “23”, Hour in UTC, the period between 00:00 and 01:00 hours is referred to hour “00”, the period between 01:00 and 02:00 to hour “01”, etc.. Value “0“ is not valid for this data field.

- End Hour air quality measurement

(Air Quality)

Last Hour of sampling period.

“00” to “23”, Hour in UTC, the period between 00:00 and 01:00 hours is referred to hour “00”, the period between 01:00 and 02:00 to hour “01”, etc.. Value “0“ is not valid for this data field.

- Continuos Analysers co-located with passive samplers

(Air Quality)

Y=Continuous analyzers and passive samplers co-located. If Yes, fill in also form COL.

- Lowest evaluation in a circular area around ozone measurement

(Air Quality)

Draw a 2.5 km or 5.0 km, respectively, radius circle with the center being the point of ozone measurements (either with continuous analyzers of with passive samplers), and provide the lowest altitude in this area.

- Survey type D_SURVEY_OZ

(Chapter 10)

| Code | Description |
| --- | --- |
| LTF | Ozone injury assessment on main tree species at Intensive Monitoring Plot (requires to submit a LTF form) |
| LSS | Ozone injury assessment on LESS plots (requires to submit a LSS form) |
| OTS | Ozone injury assessment on other symptomatic species outside the LESS plots (requires to submit a OTS form) |

- Soil moisture d_soil_moisture

(Chapter 10)

| Code | Description |
| --- | --- |
| 1 | Wet or damp (riparian zones and wet or damp areas along a stream, meadow or bottom land) |
| 2 | Moderately dry (grassland or meadow, or North or East facing slopes) |
| 3 | Very dry (exposed rocky edges) |

- Percentage of symptomatic leaves d_symp_leaves

(Chapter 10)

Percentage of symptomatic leaves for actual year's leaves or needles (C), and the needles of last year (C+1) in code.

| Code | Description |
| --- | --- |
| 0 | No injury, none of the leaves or needles injured |
| 1 | In broadleaves, 1%-5% of the leaves show ozone symptoms; in conifers, 1%-5% of the needle surface is affected |
| 2 | In broadleaves, 6%-50% of the leaves show ozone symptoms; in conifers, 6%-50% of the needle surface is affected |
| 3 | In broadleaves, 51-100% of the leaves show ozone symptoms; in conifers, 51-100% of the needle surface is affected. |

- Sample sizes at specified precision level D_PRECISION_OZ

(Chapter 10)

Precision Level "10" or "20" is to be submitted with form PLL.

| Length of the light exposed forest edge. | Possible 2x1 m non overlapping quadrates | Adjusted sample size (FPC adjusted), 10% error code: 10 | Adjusted sample size (FPC adjusted), 20% error code: 20 |
| --- | --- | --- | --- |
| 10 | 5 | 5 | 4 |
| 15 | 8 | 7 | 6 |
| 20 | 10 | 9 | 7 |
| 25 | 13 | 11 | 8 |
| 30 | 15 | 13 | 9 |
| 35 | 18 | 15 | 10 |
| 40 | 20 | 17 | 11 |
| 45 | 23 | 18 | 12 |
| 50 | 25 | 20 | 12 |
| 60 | 30 | 23 | 13 |
| 70 | 35 | 26 | 14 |
| 80 | 40 | 28 | 15 |
| 90 | 45 | 31 | 16 |
| 100 | 50 | 33 | 16 |
| 150 | 75 | 42 | 18 |
| 200 | 100 | 49 | 19 |
| 250 | 125 | 54 | 20 |
| 300 | 150 | 59 | 21 |
| 350 | 175 | 62 | 21 |
| 400 | 200 | 65 | 21 |
| 450 | 225 | 67 | 22 |
| 500 | 250 | 69 | 22 |
| 600 | 300 | 73 | 22 |
| 700 | 350 | 75 | 22 |
| 800 | 400 | 77 | 23 |
| 900 | 450 | 79 | 23 |
| 1000 | 500 | 81 | 23 |
| 2000 | 1000 | 88 | 23 |

- Validation status d_validated

(Chapter 10)

| Code | Description |
| --- | --- |
| NR | Not revised. Material was not sent to the validation center for validation |
| Y | Sent to the validation centre, which confirmed that the symptoms were produced by ozone |
| N | Sent to the validation centre, which confirmed that the symptoms were not produced by ozone |

- Type of validation d_type_validation

(Chapter 10)

The ozone symptom has been validated by the validation centre based on:

| Code | Description |
| --- | --- |
| L | Leaves |
| M | Microscopy |
| P | Photos |
| LP | Leaves and photos |
| LM | Leaves and microscopy |
| MP | Microscopy and photos |
| LMP | Leaves, microscopy and photos |

- Quadrat number

(Chapter 10)

This field is used to identify the assessments of ozone injury at each single rectangle (quadrat). Number the different rectangles (quadrats) progressively. E.g., if 10 rectangles (quadrats) are assessed, this field is used to identify rectangles 1, 2, 3….10.

- Foliage & Litterfall & Ground Vegetation & Deposition– pretreatment methods D_PRETREATMENT_FOLFGV

(QA forms)

| Code | Pretreatment methods | FO | LF | GB | DP |
| --- | --- | --- | --- | --- | --- |
| 0 | No information | X | X | X | X |
| 1 | No pretreatment | X | X | X | X |
| 2 | Extractions | X | X | X |  |
| 2.3 | Extraction aqua regia | X | X | X |  |
| 2.7 | Extraction H2O | X | X | X |  |
| 2.8 | Extraction HNO3 | X | X | X |  |
| 3 | Wet ashings at room pressure (open system) | X | X | X |  |
| 3.1 | Wet ashing HNO3 | X | X | X |  |
| 3.11 | Wet ashing HNO3 /H2SO4 | X | X | X |  |
| 3.12 | Wet ashing aqua regia | X | X | X |  |
| 3.2 | Wet ashing HNO3/HF | X | X | X |  |
| 3.21 | Wet ashing HClO4/H2O2 | X | X | X |  |
| 3.22 | Wet ashing HClO4/H2SO4 | X | X | X |  |
| 3.3 | Wet ashing HNO3/HClO4 | X | X | X |  |
| 3.31 | Wet ashing H2SO4/H2O2 | X | X | X |  |
| 3.32 | Wet ashing H2SO4/K2CrO7 | X | X | X |  |
| 3.4 | Wet ashing HNO3/HClO4/HF | X | X | X |  |
| 3.5 | Wet ashing HNO3/H2O2 | X | X | X |  |
| 3.51 | Kjeldahl H2SO4/ Se-catalyst | X | X | X |  |
| 3.52 | Kjeldahl H2SO4/Cu-catalyst | X | X | X |  |
| 3.53 | Kjeldahl H2SO4/Ti-Cu-catalyst | X | X | X |  |
| 3.54 | Kjeldahl H2SO4/Hg-catalyst | X | X | X |  |
| 3.6 | Wet ashing HNO3/HClO4 /H2SO4 | X | X | X |  |
| 3.7 | Wet ashing HNO3/HClO4/CaCl2 | X | X | X |  |
| 3.8 | Wet ashing HNO3/HClO4/H2O2 | X | X | X |  |
| 3.9 | Wet ashing HNO3,/HClO4/HCl | X | X | X |  |
| 4 | Pressure digestions (closed system) | X | X | X |  |
| 4.1 | Pressure digestion HNO3 | X | X | X |  |
| 4.2 | Pressure digestion HNO3/HF | X | X | X |  |
| 4.3 | Pressure digestion HNO3/HClO4 | X | X | X |  |
| 4.4 | Pressure digestion HNO3/HClO4/HF | X | X | X |  |
| 4.5 | Pressure digestion HNO3/H2O2, | X | X | X |  |
| 5 | Microwave pressure digestions (closed system) | X | X | X |  |
| 5.1 | Microwave digestion HNO3 | X | X | X |  |
| 5.2 | Microwave digestion HNO3/HF | X | X | X |  |
| 5.3 | Microwave digestion HNO3/HClO4 | X | X | X |  |
| 5.4 | Microwave digestion HNO3/HClO4/HF | X | X | X |  |
| 5.5 | Microwave digestion HNO3/H2O2 | X | X | X |  |
| 5.6 | Microwave digestion HNO3/H2O2/HF | X | X | X |  |
| 5.7 | Microwave digestion HNO3/H2O2/HCl | X | X | X |  |
| 5.8 | Microwave aqua regia | X | X | X |  |
| 6 | Dry ashings | X | X | X |  |
| 6.1 | Dry ashing dissolution with HNO3 | X | X | X |  |
| 6.2 | Dry ashing dissolution with HNO3/MgNO3 | X | X | X |  |
| 6.3 | Dry ashing dissolution with HNO3/HF | X | X | X |  |
| 6.4 | Dry ashing dissolution with HNO3/HCl | X | X | X |  |
| 6.5 | Dry ashing dissolution with HCl | X | X | X |  |
| 6.6 | Dry ashing dissolution with HCl/HF | X | X | X |  |
| 6.7 | Dry ashing dissolution with H2SO4 | X | X | X |  |
| 7 | Oxygen ashings | X | X | X |  |
| 7.1 | Oxygen ashing Schöniger | X | X | X |  |
| 7.2 | Oxygen ashing Wickbold | X | X | X |  |
| 7.3 | Oxygen ashing calorimetric bomb | X | X | X |  |
| 8.2 | Hydrolysis with K2S2O8 + H3BO3 + NaOH |  |  |  | X |
| 8.3 | Persulfate digestion (K2S2O8 + H2SO4) |  |  |  | X |
| 8.4 | Persulfate digestion (K2S2O8 + NaOH) |  |  |  | X |
| 8.7 | Other deposition pretreatment (please specify per email) |  |  |  | X |
| 9 | X-ray-pretreatments and other pretreatments | X | X | X |  |
| 9.1 | Material pressed (pellet) | X | X | X |  |
| 9.2 | Material melted and formed (tablet) | X | X | X |  |
| 9.5 | Melting (NaOH) | X | X | X |  |

- Foliage, Soil Solution, Soil, Litterfall, Deposition – determination methods d_determination

(QA forms)

| Code | Determination method | Foliage/Litter | Soil | Soil Solution, Deposition |
| --- | --- | --- | --- | --- |
| 0 | No information | x |  |  |
| 1 | No detection | x | x | x |
| 10 | Elemental-analyzers | x | x | x |
| 11 | Kjeldahl-apparatus | x | x | x |
| 11.1 | Kjeldahl-apparatus (Tecator) | x | x | x |
| 11.2 | Kjeldahl-apparatus (Gerhardt) | x | x | x |
| 11.3 | Kjeldahl-apparatus (Büchi) | x | x | x |
| 12 | N-Analyzer | x | x | x |
| 12.1 | N-Analyzer (Heraeus/Elementar ) | x | x | x |
| 12.2 | N-Analyzer (Vario) | x | x | x |
| 12.3 | N-Analyzer (Leco) | x | x | x |
| 13 | C-Analyzer | x | x | x |
| 13.1 | C-Analyzer (Leco) | x | x | x |
| 13.2 | TOC Analyzer | x | x | x |
| 13.3 | C-Analyzer (Heraeus/Elementar) | x | x | x |
| 14 | S-Analyzer | x | x | x |
| 14.1 | S-Analyzer (Leco)x | x | x | x |
| 15 | C/N-Analyzer | x | x | x |
| 15.1 | C/N-Analyzer (Carlo-Erba=CE Instruments) | x | x | x |
| 15.2 | C/N-Analyzer (Leco) | x | x | x |
| 15.3 | C/N-Analyzer (Heraeus/Elementar) | x | x | x |
| 15.4 | C/N-Analyzer (Vario) | x | x | x |
| 15.5 | C/N-Analyzer (Hekatech) | x | x | x |
| 16 | C/S-Analyzer | x | x | x |
| 16.1 | C/S-Analyzer (Leco) | x | x | x |
| 17 | C/N/S-Analyzer | x | x | x |
| 17.1 | C/N/S-Analyzer (Leco) | x | x | x |
| 17.2 | C/N/S-Analyzer (Heraeus/Elementar) | x | x | x |
| 17.3 | C/N/S-Analyzer (Thermo Electron) | x | x | x |
| 17.4 | C/N/S-Analyzer (Carlo-Erba=CE Instruments) | x | x | x |
| 18 | C/N/H-Analyzer | x | x | x |
| 18.1 | C/N/H-Analyzer (Leco) | x | x | x |
| 18.2 | C/H/N-Analyzer (Heraeus/Elementar) | x | x | x |
| 19 | C/H/N/S-Analyzer | x | x | x |
| 20 | Mono-Atom-Spectrometry-Techniques | x | x | x |
| 21 | AAS-flame technique | x | x | x |
| 21.1 | AAS-flame technique (C2H2/Air) | x | x | x |
| 21.2 | AAS-flame technique (C2H2/N2O) | x | x | x |
| 22 | AAS-flameless (electrothermal technique) | x | x | x |
| 24 | AAS-hydride technique | x | x | x |
| 25 | AAS-cold vapor technique | x | x | x |
| 25.1 | AAS-LECO/ALTEC Mercury Analyzer | x | x | x |
| 26 | AFS-hydride-technique | x | x | x |
| 28 | AES-Flame photometer | x | x | x |
| 30 | Multi-Atom-Spectrometry-techniques | x | x | x |
| 31 | ICP-AES without Ultrasonic nebulisation | x | x | x |
| 32 | ICP-AES with Ultrasonic nebulisation | x | x | x |
| 35 | ICP-MS | x | x | x |
| 40 | Physical techniques | x | x | x |
| 41 | X-ray-energy dispersive | x | x | x |
| 42 | X-ray-wavelength dispersive | x | x | x |
| 45 | Neutron activation analysis (NAA) | x | x | x |
| 47 | Gamma-spectroscopy | x | x | x |
| 48 | Laser diffraction | x | x | x |
| 50 | UV-VIS-spectrophotometry-techniques | x | x | x |
| 51 | Colorimetric N-Determination | x | x | x |
| 51.1 | Indophenol-blue-method | x | x | x |
| 51.2 | Flow Injection (FIAS)-NH3-Membrane-diffusion 566 nm | x | x | x |
| 51.3 | Continuous flow method, Indophenol blue | x | x | x |
| 52 | Colorimetric S-Determination | x | x | x |
| 52.1 | Nephelometry | x | x | x |
| 52.2 | Turbidimetry | x | x | x |
| 53 | Colorimetric P-Determination | x | x | x |
| 53.1 | Molybdene-blue-method | x | x | x |
| 53.2 | Vanadium-Mo-blue-method | x | x | x |
| 53.3 | Continuous flow method, Molybdene-blue | x | x | x |
| 54 | Colorimetric B-Determination | x | x | x |
| 54.1 | Azomethin – H | x | x | x |
| 54.2 | Carmine | x | x | x |
| 55 | Colorimetric C-Determination |  | x | x |
| 60 | Ion-chromatographic techniques | x | x | x |
| 61.1 | Anion-Chromatography w. chemical suppression | x | x | x |
| 61.2 | Anion-Chromatography w. electr. suppression | x | x | x |
| 62.1 | Kation-Chromatography w. chemical suppression | x | x | x |
| 62.2 | Kation-Chromatography w. electr. suppression | x | x | x |
| 70 | Electrochemical methods | x | x | x |
| 71 | Conductimetry | x | x | x |
| 71.1 | Conductometric titration | x | x | x |
| 72 | Potentiometry | x | x | x |
| 72.1 | pH |  | x | x |
| 72.2 | other ion selective electrodes | x | x | x |
| 73 | Potentiometric titrations | x | x | x |
| 74 | Stripping potentiometry | x | x | x |
| 75 | Voltammetry | x | x | x |
| 76 | Polarography | x | x | x |
| 77 | Amperometry | x | x | x |
| 78 | Electrophoresis | x | x | x |
| 79 | Redox potential | x | x | x |
| 80 | Classical analytical techniques | x | x | x |
| 81 | Gravimetry | x | x | x |
| 81.1 | Pipette |  | x | x |
| 81.2 | Hydrometer |  | x | x |
| 82 | Titration | x | x | x |
| 82.1 | NH4-back titration | x | x | x |
| 82.2 | Thiocyanate-titration | x | x | x |
| 82.3 | FeNH4SO4-Titration | x | x | x |
| 82.4 | Barimetric titration | x | x | x |
| 82.5 | AgNO3-Titration | x | x | x |
| 83 | Calcimeter (Scheibler unit) |  | x | x |
| 84 | Carbon determinations |  | x | x |
| 84.1 | Loss on ignition |  | x | x |
| 84.2 | Walkley-Black |  | x | x |
| 84.3 | Tjurin method |  | x | x |
| 90 | other detections | x | x | x |
| 91 | Calculation |  | x | x |

- QA parameter

(QA forms)

a) Laboratories

For each parameter the laboratory has to evaluate the quantification limit (in unit of parameter) and to use a control chart over the year. Then the mean of the control chart and the standard deviation in [%] (is coefficient of variation = stdev / mean) has to be evaluated and to be submitted.

b) Ringtests

For each parameter the results from the last ring test have to be submitted. The participation (yes=1, no =0), the ring test number and the lab ID are the same for all parameters. The percentage of results within tolerable limit (see your individual ring test report) for each parameter has to be reported; if the percentage is lower than 50 % the laboratory has to requalify. In this case success of the requalification (yes=1, no =0) has to be reported. The percentage of results within tolerable limit after requalification can be submitted in the last column of the form.

Please fill up with blanks in the fields <ring_test_number>, <Laboratory_ID>, and <percentage_within> in case that there was no participation in a ring test concerning the respective parameter.

- Foliage and Ground Vegetation biomass – parameter d_parameter_fogv

(QA forms)

| code | Compound | Units |
| --- | --- | --- |
| N | Nitrogen | mg/g |
| S | Sulphur | mg/g |
| P | Phosphor | mg/g |
| Ca | Calcium | mg/g |
| Mg | Magnesium | mg/g |
| K | Potassium | mg/g |
| C | Carbon | g/100g |
| Zn | Zinc | μg/g |
| Mn | Manganese | μg/g |
| Fe | Iron | μg/g |
| Cu | Copper | μg/g |
| Pb | Lead | μg/g |
| Cd | Cadmium | ng/g |
| B | Boron | μg/g |

- Soil Solution – parameter D_parameter_ss

(QA forms)

| code | parameter | unit |
| --- | --- | --- |
| pH | pH value |  |
| Cond | conductivity | µS/cm |
| K | Potassium | mg/l |
| Ca | Calcium | mg/l |
| Mg | Magnesium | mg/l |
| N_NO3 | Nitrate-N | mg/l |
| S_SO4 | Suphate-S | mg/l |
| Alkalin | Alkalinity | µmolc/l |
| Al | Aluminium | mg/l |
| DOC | Dissolved Organic Carbon | mg/l |
| Na | Sodium | mg/l |
| Al_labile | labile Aluminium | mg/l |
| Fe | Iron | mg/l |
| Mn | Manganese | mg/l |
| P | total Phosphor | mg/l |
| N_NH4 | Ammonium- N | mg/l |
| Cl | Chloride | mg/l |
| Zn | Zinc | µg/l |
| Cu | Copper | µg/l |
| Cr | Chromate | µg/l |
| Ni | Nickel | µg/l |
| Pb | Lead | µg/l |
| Cd | Cadmium | µg/l |
| Si | Silicium | mg/l |
| P_PO4 | Phosphate | mg/l |

- Deposition – parameter d_parameter_dp

(QA forms)

| code | parameter | unit |
| --- | --- | --- |
| pH | pH value |  |
| Cond | Conductivity | µS/cm |
| K | Potassium | mg/l |
| Ca | Calcium | mg/l |
| Mg | Magnesium | mg/l |
| Na | Sodium | mg/l |
| N_NH4 | Ammonium- N | mg/l |
| Cl | Chloride | mg/l |
| N_NO3 | Nitrate- N | mg/l |
| S_SO4 | Sulphate- S | mg/l |
| Alkalin | Alkalinity | μeq/l |
| N_total | total Nitrogen | mg/l |
| DOC | Dissolved Organic Carbon | mg/l |
| Al | Aluminium | μg/l |
| Mn | Manganese | μg/l |
| Fe | Iron | μg/l |
| P_PO4 | Phosphate- P | mg/l |
| Cu | Copper | μg/l |
| Zn | Zinc | μg/l |
| Hg | Mercury | μg/l |
| Pb | Lead | μg/l |
| Co | Cobalt | μg/l |
| Mo | Molybdenum | μg/l |
| Ni | Nickel | μg/l |
| Cd | Cadmium | μg/l |
| S_total | total Sulphur | mg/l |
| C_total | total Carbon | mg/l |

- Litterfall – parameter d_parameter_LF

(QA forms)

| code | parameter | unit |
| --- | --- | --- |
| N | Nitrogen | mg/g |
| S | Sulphur | mg/g |
| P | Phophor | mg/g |
| Ca | Calcium | mg/g |
| Mg | Magnesium | mg/g |
| K | Potassium | mg/g |
| C | Carbon | g/100g |
| Na | Sodium | mg/g |
| Zn | Zinc | μg/g |
| Mn | Manganese | μg/g |
| Fe | Iron | μg/g |
| Cu | Copper | μg/g |
| Pb | Lead | μg/g |
| Al | Aluminium | mg/g |
| Cd | Cadmium | ng/g |
| B | Bor | μg/g |

- Air Quality – parameter d_parameter_AQ

(QA foms)

| code | parameter |
| --- | --- |
| O3 | Ozone |
| NH3 | Ammonia |
| NO2 | Nitric Oxide |
| NO3 | Nitrate |
| SO2 | sulphur dioxide |

- Soil – parameter d_parameter_SO

(QA forms)

| Code | parameter |
| --- | --- |
| Pclay | percentage clay |
| Psilt | percentage silt |
| Psand | percentage sand |
| pH_CaCl2 | pH in CaCl2 |
| pH_H2O | pH in H2O |
| org_C | Organic Carbon |
| Total_N | total Nitrogen |
| CaCO3 | Carbonate |
| Acid_exch | Exchangeable Acidity |
| Al_exch | Exchangeable Aluminium |
| Ca_exch | Exchangeable Calcium |
| Fe_exch | Exchangeable Iron |
| K_exch | Exchangeable Potassium |
| Mg_exch | Exchangeable Magnesium |
| Mn_exch | Exchangeable Manganese |
| Na_exch | Exchangeable Sodium |
| Free_H+ | Free Acidity |
| Al_extr | Extractable Aluminium |
| Ca_extr | Extractable Calcium |
| Cd_extr | Extractable Cadmium |
| Cr_extr | Extractable Chromium |
| Cu_extr | Extractable Copper |
| Fe_extr | Extractable Iron |
| Hg_extr | Extractable Mercury |
| K_extr | Extractable Potassium |
| Mg_extr | Extractable Magnesium |
| Mn_extr | Extractable Manganese |
| Na_extr | Extractable Sodium |
| Ni_extr | Extractable Nickel |
| P_extr | Exctractable Phosphor |
| Pb_extr | Extractable Lead |
| S_extr | Extractable Sulphur |
| Zn_extr | Extractable Zinc |
| Al_tot | total Aluminium |
| Ca_tot | total Calcium |
| Fe_tot | total Iron |
| K_tot | total Potassium |
| Mg_tot | total Magnesium |
| Mn_tot | total Manganese |
| Na_tot | total Sodium |
| Al_react | Reactive Aluminium |
| Fe_react | Reactive Ferric |

- Soil – CODE_REMOVAL_COMPOUNDS d_removal_comp_SO

(QA forms)

| CODE | Description |
| --- | --- |
| 1 | No removal |
| 2 | Removal of organic carbon |
| 2.1 | Hydrogen peroxide |
| 2.2 | Pre-ignition at 850°C |
| 3 | Removal of soluble salts and gypsum |
| 3.1 | Washing with water |
| 4 | Removal of carbonates |
| 4.1 | Hydrochloric acid |
| 4.2 | Hydrochloric acid/Calcium chloride |
| 5 | Removal of OC and carbonates |
| 6 | Removal of OC, carbonates, soluble salts and gypsum |

- Soil – sieving / milling d_sieving_so

(QA forms)

| CODE | soil – sieving/milling |
| --- | --- |
| 1 | Sieving and/or crushing |
| 1.1 | mesh size < 2 mm |
| 1.2 | other mesh size |
| 2 | Milling |
| 2.1 | To 150 micrometer |
| 2.2 | As fine as possible |
| 9 | Other method |

- Soil – Digestion/Extraction method – pretreatment d_pretreatment_so

(QA forms)

| code | pretreatment Soil |
| --- | --- |
| 1 | No pretreatment |
| 2 | Extractions |
| 2.11 | Single BaCl2 extraction |
| 2.12 | Triple BaCl2 extraction |
| 2.2 | Extraction KCl |
| 2.4 | Extraction total with HF/HClO4 |
| 2.5 | Extraction total with LiBO2 |
| 2.6 | Extraction with Acid Ammonium Oxalate |
| 2.7 | Extraction H2O |
| 2.8 | Extraction HNO3 |
| 2.9 | Extraction CaCl2 |
| 3 | Wet ashings at room pressure (open system) |
| 3.1 | Wet ashing HNO3 |
| 3.11 | Wet ashing HNO3 /H2SO4 |
| 3.12 | Wet ashing aqua regia |
| 3.2 | Wet ashing HNO3/HF |
| 3.21 | Wet ashing HClO4/H2O2 |
| 3.22 | Wet ashing HClO4/H2SO4 |
| 3.3 | Wet ashing HNO3/HClO4 |
| 3.31 | Wet ashing H2SO4/H2O2 |
| 3.32 | Wet ashing H2SO4/K2CrO7 |
| 3.4 | Wet ashing HNO3/HClO4/HF |
| 3.5 | Wet ashing HNO3/H2O2 |
| 3.51 | Kjeldahl H2SO4/ Se-catalyst |
| 3.52 | Kjeldahl H2SO4/Cu-catalyst |
| 3.53 | Kjeldahl H2SO4/Ti-Cu-catalyst |
| 3.54 | Kjeldahl H2SO4/Hg-catalyst |
| 3.6 | Wet ashing HNO3/HClO4 /H2SO4 |
| 3.7 | Wet ashing HNO3/HClO4/CaCl2 |
| 3.8 | Wet ashing HNO3/HClO4/H2O2 |
| 3.9 | Wet ashing HNO3,/HClO4/HCl |
| 4 | Pressure digestions (closed system) |
| 4.1 | Pressure digestion HNO3 |
| 4.2 | Pressure digestion HNO3/HF |
| 4.3 | Pressure digestion HNO3/HClO4 |
| 4.4 | Pressure digestion HNO3/HClO4/HF |
| 4.5 | Pressure digestion HNO3/H2O2, |
| 5 | Microwave pressure digestions (closed system) |
| 5.1 | Microwave digestion HNO3 |
| 5.2 | Microwave digestion HNO3/HF |
| 5.3 | Microwave digestion HNO3/HClO4 |
| 5.4 | Microwave digestion HNO3/HClO4/HF |
| 5.5 | Microwave digestion HNO3/H2O2 |
| 5.6 | Microwave digestion HNO3/H2O2/HF |
| 5.7 | Microwave digestion HNO3/H2O2/HCl |
| 5.8 | Microwave aqua regia |
| 6 | Dry ashings |
| 6.1 | Dry ashing dissolution with HNO3 |
| 6.2 | Dry ashing dissolution with HNO3/MgNO3 |
| 6.3 | Dry ashing dissolution with HNO3/HF |
| 6.4 | Dry ashing dissolution with HNO3/HCl |
| 6.5 | Dry ashing dissolution with HCl |
| 6.6 | Dry ashing dissolution with HCl/HF |
| 6.7 | Dry ashing dissolution with H2SO4 |
| 7 | Oxygen ashings |
| 7.1 | Oxygen ashing Schöniger |
| 7.2 | Oxygen ashing Wickbold |
| 7.3 | Oxygen ashing calorimetric bomb |
| 9 | X-ray-pretreatments and other pretreatments |
| 9.1 | Material pressed (pellet) |
| 9.2 | Material melted and formed (tablet) |
| 9.5 | Melting (NaOH) |

- start_date, end_date

(QA forms)

In order to enable the coding of the change of the analyzing laboratory during a year the fields start_date and end_date were integrated into the QA forms. The start_date is the first (analysing) date from which the laboratory analysed the data from the respective plot, sampler and parameter. The end_date is the last day of the analyses of the respective plot sampler and parameter combination. Only in case, that a laboratory has been changed during one monitoring year, it would be nessecary to use an additional data row with the respecting time period in which the new laboratory has been analysed the data.

The format of the format is the same as described in explanatory item 3 (DDMMYY).

- Event D_event

(Pheno, PHI, PHE)

| Code | Description |
| --- | --- |
| 1 | Flushing |
| 2 | Colour changes |
| 3 | Leaf/needle fall |
| 4 | Leaf or crown damage |
| 5 | Other damage |
| 6 | Lammas shoots / secondary flushing |
| 7 | Flowering |

- Installation date – Pheno tree selection

(Pheno)

Date at which the trees for phenology monitoring were selected and the visible part of the crown and the observation direction were defined.

- Code for visible part of crown d_visible_crown

(Pheno)

| code | description |
| --- | --- |
| 1 | top of the crown visible |
| 2 | middle of the crown visible |
| 3 | top and the middle of the crown visible |

- Direction FROM where observations are made d_direction

(Pheno)

| Code | Description |
| --- | --- |
| 1 | North |
| 2 | North-east |
| 3 | East |
| 4 | South-east |
| 5 | South |
| 6 | South-west |
| 7 | West |
| 8 | North-west |

- Vertical direction FROM where the observations are made d_vertical_from

(Pheno)

| Code | Description |
| --- | --- |
| 1 | From below |
| 2 | At crown level |
| 3 | From above |

- Score of event d_event_score

(Pheno)

The score gives the relative share of affected tree compartments of the observed crown in case of intensive survey. In case of observation on plot level it is the proportion of the forest crown affected.

| Code | Description | score |
| --- | --- | --- |
| 1 |  | <1% |
| 2 | Infrequent or slight | 1 – 33% |
| 3 | Common or moderate | >33 – 66% |
| 4 | Abundant or severe | >66% – 99% |
| 5 |  | >99% |
| Codes for flowering and damage events: |  |  |
| 6 | Flowering / Damage absent |  |
| 7 | Flowering / Damage present |  |
| Codes for flowering events: |  |  |
| 7.1 | Flowering sparse (optional) |  |
| 7.2 | Flowering moderate (optional) |  |
| 7.3 | Flowering abundant or mast (optional) |  |

- Method used for making the observation d_method_PH

(Pheno)

| Code | Description |
| --- | --- |
| 1 | field observation |
| 2 | digital camera |
| 3 | both field observation and digital camera |

- Name of submitted digital image files

(Pheno, LAI)

The name is built from the code of the country as described in Explanatory item (1), four digits for the plot number (2), and 6 digits for date of observation (3). In case of a photo this would read e.g. “040534000114050913541301.jpg”. In case that more than one tree is observed by a movie, as many records (lines) in the submission file .PHD are to be submitted as the number of observed trees is (see example below).

name of image will be for submission:
XXPPPPNNNNDDDDDDTTTTTTSS.jpg

- XX – country code (ICP Forests manual, Expl. Item (1))

- PPPP – plot number (ICP Forests manual, Expl. Item (2)); "9" and 3 further letters for assigning a location not being a ICP Forests / FutMon plot

- NNNN – measurement point number (LAI Field Protocol) or tree number (Pheno; Expl. Item (19)). Use 9999 in case that plot representative image or image which is relevant on more than one trees, respectively, is submitted.

- DDDDDD – date of image production (DayMonthYear: e.g. 140509)

- TTTTTT – time of image production (HHMMSS)

- Sequence number (01, 02, 03,.)  to indicate which photo in a respective time.

Example: Country Germany, Plot 534, observation from 14th of May 2009, Trees number 2, 5, 6, 8, 10, 12, 15, 16, 19, and 20 were observed with a movie in format .m2v leads to the following name of the movie:

040534000214050913541301.m2v

for file .PHD:

The respective lines in the .PHD should be ("camera type 1" example for <other observations>:

!Sequence, plot, tree, tree_species, event, file, other_observations

(452 other lines with information preceeding)

00453 0534 02 020 07 040534000214050913541301.m2v camera type 1

00454 0534 05 020 07 040534000514050913541301.m2v camera type 1

00455 0534 06 020 07 040534000614050913541301.m2v camera type 1

00456 0534 08 020 07 040534000814050913541301.m2v camera type 1

00457 0534 10 020 07 040534001014050913541301.m2v camera type 1

00458 0534 12 020 07 040534001214050913541301.m2v camera type 1

00459 0534 15 020 07 040534001514050913541301.m2v camera type 1

00460 0534 16 020 07 040534001614050913541301.m2v camera type 1

00461 0534 19 020 07 040534001914050913541301.m2v camera type 1

00462 0534 20 020 07 040534002014050913541301.m2v camera type 1

…   (further records / lines if needed)

- storey description - layers (absence or description of more than one storey layer) d_storey

(LAI)

| Code | Name | Description | Example |
| --- | --- | --- | --- |
| 1 | single main storey | canopy consists of only one main storey | Climax beech forest |
| 2 | second storey | canopy consists of two layers | s. below 2.1 to 2.3 |
| 2.1 |  |  | seed trees above main storey |
| 2.2 |  |  | main storey with under storey (e.g. nature regeneration or underplanting) |
| 2.3 |  |  | second storey consists of other tree species than main storey with deviating height development / age |
| 3 | multi storey | more than 2 layers | seed trees / main storey / under storey |
| 4 | "plenter" structure | canopy is of high vertical diversity | "Plenter" structure |
| 5 | disturbed structure | discontinuous canopy (diameter of holes with at least one crown diamter extent) with or without natural regeneration or underplanting | stand disturbed e.g. by wind throw or insect attacks |
| 6 | main storey / -ies and ground vegetation above 1,5m | LAI measurement at 1,5m most probably influenced by ground vegetation | Urtica dioica |
| 6.1 | single main storey | s. 1 and ground veg. > 1.5m |  |
| 6.2 | second storey | s. 2 and ground veg. > 1.5m |  |
| 6.3 | multi storey | s. 3 and ground veg. > 1.5m |  |
| 6.4 | "plenter" structure | s. 4 and ground veg. > 1.5m |  |
| 6.5 | disturbed | s. 5 and ground veg. > 1.5m |  |

- Parameter to be specified with photo submission d_parameter_la

(LAI) (to be updated after first data submission)

| Code | Description |
| --- | --- |
| Aper | Aperature of camera |
| Expo | Exposure time (e.g. -2u is -2 steps below automatic UNDER canopy, -1a is -1 step below automatic ABOVE canopy ) |
| Camera | Brand and model |
| Lens | Canopy analyzer – other (to be described in detail) |
| Sun | Short description of light condition |
|  | Additional parameters may be specified and submitted with up to 6 characters but must be announced to the data centre at least 1 week before submission for harmonization issues |

- Method for LAI determination d_determination_la

(LAI)

According to Field Protocol on LAI measurement:

| Code | Description |
| --- | --- |
| 10 | Direct determination (Litterfall) |
| 21 | Canopy analyzer – LICOR 2000 |
| 22 | Canopy analyzer – TRAC |
| 23 | Canopy analyzer – other (to be described in detail) |
| 31 | digital camera – WinScanopy |
| 32 | digital camera – HemiView |
| 33 | digital camera – GLA |
| 34 | digital camera – other (to be described in detail) |
| 40 | Photo mosaic |
| 99 | Other (to be described in detail) |

- LAI – results d_results_la

(LAI)

| Code | LAI parameter | Method for determination (s. Expl. Item (128) |
| --- | --- | --- |
| 100 | LAI | Direct determination (Litterfall) |
| 210 | LAI 2000G | Canopy analyzer – LICOR 2000 |
| 220 | LAI plot mean value indicated by “99“ in field “number of measurement point” | Canopy analyzer – TRAC |
| 222 | Gap Fraction plot mean value indicated by “99“ in field “number of measurement point” | Canopy analyzer – TRAC |
| 225 | PAIe plot mean value indicated by “99“ in field “number of measurement point” | Canopy analyzer – TRAC |
| 230 | LAI | Canopy analyzer – other |
| 310 | LAI 2000G | digital camera – WinScanopy |
| 311 | Tot. Openess | digital camera – WinScanopy |
| 312 | Gap Fraction | digital camera – WinScanopy |
| 313 | LAI 2000G | digital camera – WinScanopy |
| 314 | ClumpFact(2000G) | digital camera – WinScanopy |
| 320 | LAI | digital camera – HemiView |
| 330 | LAI 2000G | digital camera – GLA |
| 331 | Tot. Openess | digital camera – GLA |
| 332 | Gap Fraction | digital camera – GLA |
| 340 | LAI 2000G | digital camera – other (to be described in detail) |
| 341 | Tot. Openess | digital camera – other (to be described in detail) |
| 342 | Gap Fraction | digital camera – other (to be described in detail) |
| 400 | LAI | Photo mosaic |
| 999 | LAI | Other (to be described in detail) |

- Matric pressure – Volumetric water content

(Soil Water)

Determinations of volumetric water content made in the FutMon project should be done according to following pre-set matric pressures:

| pF | kPa | volumetric soil water content (m3.m-3) |
| --- | --- | --- |
| 0.0 | 0 | (= Total Porosity) Mandatory |
| 1.0 | -1 | Mandatory |
| 1.7 | -5 | Mandatory |
| 2.0 | -10 | Optional |
| 2.5 | -33 | Mandatory |
| 3.0 | -100 | Optional |
| 3.4 | -250 | Optional |
| 4.2 | -1500 | Mandatory |

- Code of the WRB Reference Soil Group (2006) d_soil_group

(Soil)

| Code | Description |  | Code | Description |
| --- | --- | --- | --- | --- |
| AC | Acrisol |  | KS | Kastanozem |
| AB | Albeluvisol |  | LP | Leptosol |
| AL | Alisol |  | LX | Lixisol |
| AN | Andosol |  | LV | Luvisol |
| AT | Anthrosol |  | NT | Nitisol |
| AR | Arenosol |  | PH | Phaeozem |
| CL | Calcisol |  | PL | Planosol |
| CM | Cambisol |  | PT | Plinthosol |
| CH | Chernozem |  | PZ | Podzol |
| CR | Cryosol |  | RG | Regosol |
| DU | Durisol |  | SC | Solonchak |
| FR | Ferralsol |  | SN | Solonetz |
| FL | Fluvisol |  | ST | Stagnosol |
| GL | Gleysol |  | TC | Technosol |
| GY | Gypsisol |  | UM | Umbrisol |
| HS | Histosol |  | VR | Vertisol |

- Code of the WRB Qualifier (1 till 6) d_soil_adjective

(Soil)

| Code | Qualifier |  | Code | Qualifier |  | Code | Qualifier |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ap | Abruptic |  | ge | Gelic |  | pe | Pellic |
| ae | Aceric |  | gt | Gelistagnic |  | pt | Petric |
| ac | Acric |  | gr | Geric |  | pc | Petrocalcic |
| ao | Acroxic |  | gi | Gibbsic |  | pd | Petroduric |
| ab | Albic |  | gc | Glacic |  | py | Petrogleyic |
| ax | Alcalic |  | gl | Gleyic |  | pg | Petrogypsic |
| al | Alic |  | gb | Glossialbic |  | pp | Petroplinthic |
| aa | Aluandic |  | gs | Glossic |  | ps | Petrosalic |
| au | Alumic |  | gz | Greyic |  | px | Pisoplinthic |
| an | Andic |  | gm | Grumic |  | pi | Placic |
| aq | Anthraquic |  | gy | Gypsic |  | pa | Plaggic |
| am | Anthric |  | gp | Gypsiric |  | pl | Plinthic |
| ar | Arenic |  | ha | Haplic |  | po | Posic |
| ai | Aric |  | hm | Hemic |  | pf | Profondic |
| ad | Aridic |  | hi | Histic |  | pr | Protic |
| az | Arzic |  | ht | Hortic |  | pu | Puffic |
| br | Brunic |  | hu | Humic |  | ra | Reductaquic |
| ca | Calcaric |  | hg | Hydragric |  | rd | Reductic |
| cc | Calcic |  | hy | Hydric |  | rg | Regic |
| cm | Cambic |  | hf | Hydrophobic |  | rz | Rendzic |
| cb | Carbic |  | hb | Hyperalbic * |  | rh | Rheic |
| cn | Carbonatic |  | hl | Hyperalic |  | ro | Rhodic |
| cl | Chloridic |  | hc | Hypercalcic |  | ru | Rubic |
| cr | Chromic |  | hd | Hyperdystric |  | rp | Ruptic |
| ce | Clayic |  | he | Hypereutric |  | rs | Rustic |
| co | Colluvic |  | hp | Hypergypsic |  | sz | Salic |
| cy | Cryic |  | ho | Hyperochric |  | sa | Sapric |
| ct | Cutanic |  | hs | Hypersalic |  | sn | Silandic |
| dn | Densic |  | hk | Hyperskeletic |  | sl | Siltic |
| dr | Drainic |  | wc | Hypocalcic |  | sk | Skeletic |
| du | Duric |  | wg | Hypogypsic |  | so | Sodic |
| dy | Dystric |  | wl | Hypoluvic |  | sc | Solodic |
| ek | Ekranic |  | ws | Hyposalic |  | sm | Sombric |
| nd | Endoduric |  | wn | Hyposodic |  | sd | Spodic |
| ny | Endodystric |  | ir | Irragric |  | sp | Spolic |
| ne | Endoeutric |  | ll | Lamellic |  | st | Stagnic |
| nf | Endofluvic |  | la | Laxic |  | sq | Subaquic |
| ng | Endogleyic |  | le | Leptic |  | su | Sulphatic |
| nl | Endoleptic |  | lg | Lignic |  | ty | Takyric |
| ns | Endosalic |  | lm | Limnic |  | te | Technic |
| et | Entic |  | lc | Linic |  | tf | Tephric |
| ed | Epidystric |  | li | Lithic |  | tr | Terric |
| ee | Epieutric |  | lx | Lixic |  | ba | Taptantic |
| el | Epileptic |  | lv | Luvic |  | bv | Taptovitric |
| ea | Episalic |  | mg | Magnesic |  | ti | Thionic |
| ec | Escalic |  | mf | Manganiferric |  | tp | Thixotropic |
| eu | Eutric |  | mz | Mazic |  | td | Tidalic |
| es | Eutrosilic |  | ml | Melanic |  | tx | Toxic |
| fl | Ferralic |  | ms | Mesotrophic |  | tn | Transportic |
| fr | Ferric |  | mo | Mollic |  | tu | Turbic |
| fi | Fibric |  | mi | Mollicglossic |  | um | Umbric |
| ft | Floatic |  | na | Natric |  | ug | Umbriglossic |
| fv | Fluvic |  | ni | Nitic |  | ub | Urbic |
| fo | Folic |  | nv | Novic |  | vm | Vermic |
| fp | Fractipetric |  | nt | Nudilithic |  | vr | Vertic |
| fa | Fractiplinthic |  | om | Ombric |  | vt | Vetic |
| fg | Fragic |  | oc | Ornithic |  | vi | Vitric |
| fu | Fulvic |  | os | Ortsteinic |  | vo | Voronic |
| ga | Garbic |  | oa | Oxyaquic |  | xa | Xanthic |
|  |  |  | ph | Pachic |  | ye | Yermic |

The following codes are to be used for buried soils only. To use with the specifier Thapto:

| Code | Description |  | Code | Description |
| --- | --- | --- | --- | --- |
| AC | Acrisolic |  | KS | Kastanozemic |
| AB | Albeluvisolic |  | LP | Leptosolic |
| AL | Alisolic |  | LX | Lixisolic |
| AN | Andosolic |  | LV | Luvisolic |
| AT | Anthrosolic |  | NT | Nitisolic |
| AR | Arenosolic |  | PH | Phaeozemic |
| CL | Calcisolic |  | PL | Planosolic |
| CM | Cambisolic |  | PT | Plinthosolic |
| CH | Chernozemic |  | PZ | Podzolic |
| CR | Cryosolic |  | RG | Regosolic |
| DU | Durisolic |  | SC | Solonchakic |
| FR | Ferralsolic |  | SN | Solonetzic |
| FL | Fluvisolic |  | ST | Stagnosolic |
| GL | Gleysolic |  | TC | Technosolic |
| GY | Gypsisolic |  | UM | Umbrisolic |
| HS | Histosolic |  | VR | Vertisolic |

- Code of the WRB Specifier (1 till 6) d_soil_specifier

(Soil)

Note: if no specifier is needed, this field will have no value.

| Code | Description |  | Code | Description |  | Code | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| d | Bathi |  | h | Hyper |  | r | Para |
| c | Cumuli |  | w | Hypo |  | t | Proto |
| n | Endo |  | o | Ortho |  | b | Thapto |
| p | Epi |  |  |  |  |  |  |

- Code of WRB diagnostics (1 till 10) d_soil_diagnostic

(Soil)

You may provide information of up till 10 diagnostic horizons, properties or materials. This field contains the code of the concerning horizon, property or material.

a) Diagnostic horizons

| Code | Description | Code | Description | Code | Description |
| --- | --- | --- | --- | --- | --- |
| hab | Albic horizon | hgy | Gypsic horizon | hpx | Pisoplinthic horizon |
| haq | Anthraquic horizon | hhi | Histic horizon | hpa | Plaggic horizon |
| ham | Anthric horizon | hht | Hortic horizon | hpl | Plinthic horizon |
| hlv | Argic horizon | hhg | Hydragric horizon | hsz | Salic horizon |
| hcc | Calcic horizon | hir | Irragric horizon | hsm | Sombric horizon |
| hcm | Cambic horizon | hml | Melanic horizon | hsd | Spodic horizon |
| hcy | Cryic horizon | hmo | Mollic horizon | hty | Takyric horizon |
| hdu | Duric horizon | hna | Natric horizon | htr | Terric horizon |
| hfl | Ferralic horizon | hni | Nitic horizon | hti | Thionic horizon |
| hfr | Ferric horizon | hpc | Petrocalcic horizon | hum | Umbric horizon |
| hfo | Folic horizon | hpd | Petroduric horizon | hvr | Vertic horizon |
| hfg | Fragic horizon | hpg | Petrogypsic horizon | hvo | Voronic horizon |
| hfu | Fulvic horizon | hpp | Petroplinthic horizon | hye | Yermic horizon |

b) Diagnostic properties

| Code | Description | Code | Description |
| --- | --- | --- | --- |
| pap | Abrupt textural change | pgl | Gleyic colour pattern |
| pab | Albeluvic tonguing | prp | Lithological discontinuity |
| pan | Andic properties | prd | Reducing conditions |
| pad | Aridic properties | pcc | Secondary carbonates |
| ple | Continuous rock | pst | Stagnic colour pattern |
| pfl | Ferralic properties | pvr | Vertic properties |
| pgr | Geric properties | pvi | Vitric properties |

c) Diagnostic materials

| Code | Description | Code | Description |
| --- | --- | --- | --- |
| mte | Artefacts | mmn | Mineral material |
| mca | Calcaric material | mhi | Organic material |
| mco | Colluvic material | moc | Ornithogenic material |
| mfv | Fluvic material | mti | Sulphidic material |
| mgp | Gypsiric material | mek | Technic hard rock |
| mlm | Limnic material | mtf | Tephric material |

- Depth of appearance of diagnostic (1 till 10)

(Soil)

The depth of the upper limit of the diagnostic horizon/property/material in cm from upper limit of mineral soil is provided in this field. Note that on peat soils thicker than 40 cm, the 0 cm line is located at the upper limit of the peat layer.

- WRB publication code d_wrb_pub

(Soil)

Several editions of the WRB 2006 soil classification system exist. Here you should put a code referring the year of publication of the reference document used for the soil classification.

The version used for Soil Classification has to be submitted with field "WRB publication code" in form .PRF using the code in the table below:

| code | WRB publication (URL) |
| --- | --- |
| 06en | http://www.fao.org/ag/Agl/agll/wrb/doc/wrb2006final.pdf |
| 07en | http://www.fao.org/ag/agl/agll/wrb/doc/wrb2007_corr.pdf |
| 07ge | http://www.bgr.bund.de/cln_101/nn_336362/DE/Themen/Boden/Produkte/Schriften/Downloads/ WRB__deutsche__Ausgabe,templateId=raw,property=publicationFile.pdf/ WRB_deutsche_Ausgabe.pdf |
| XX99 | other publication; respective link must be submitted in the field "observation text" and be announced to the FSCC at least 4 weeks before data submission for adoption or correction. |

document specification:

| code | Reference document |
| --- | --- |
| 06en | IUSS Working Group WRB, 2006. World Reference Base for soil resources 2006. 2nd edition. World Soil Resource Reports N°. 103. FAO. Rome. |
| 07en | IUSS Working Group WRB. 2007. World Reference Base for Soil Resources 2006, first update 2007. World Soil Resources Reports No. 103. FAO, Rome. |
| 07ge | IUSS Working Group WRB. 2007. World Reference Base for Soil Resources 2006. Erstes Update 2007. Deutsche Ausgabe. – Übersetzt von Peter Schad. Herausgegeben von der Bundesanstalt für Geowissenschaften und Rohstoffe, Hannover. |

- Mean highest and mean lowest ground water table d_ground_water_table

(Soil)

| Code | Description |
| --- | --- |
| 0 | No groundwater table |
| 1 | Groundwater table between 0 - 50 cm |
| 2 | Groundwater table between 50 - 100 cm |
| 3 | Groundwater table between 100 - 150 cm |
| 4 | Groundwater table between 150 - 200 cm |
| 5 | Groundwater table below 200 cm |

- Type of water table replaced by (151)

(Soil)

| Code | Description |
| --- | --- |
| 0 | No water table |
| 1 | Perched water table |
| 2 | Permanent water table |

- Land Use type of the soil profile replaced by (152)

(Soil)

| Code | Description |
| --- | --- |
| 311 | broad-leaved forest |
| 312 | coniferous forest |
| 313 | mixed forest |

- Repetition

(Soil)

This is the order number of the composite when several composites are analysed for the same plot and depth layer. The first composite is numbered 1, the second composite is numbered 2, etc.

- Layer limit superior/inferior

(Soil)

The upper (lower) limit of the layer depth (in centimetres). The limit between the organic and mineral layer corresponds to 0 cm. For organic layers (OL, OF, OH, OFH, Hf, Hfs, Hs), the limits are negative values. For mineral layers (M05, M01, M51, M12, M24, M48) the limits are positive numbers.

- Number of subsamples

(Soil)

Number of subsamples in the composite.

- Code horizon (horizon number)

(Soil, Soil Water)

Identification number of the horizon (horizon 1 = 1, horizon 2 = 2, etc.) The horizon is further indentified by the horizon designation (144) in the xx2009.PFH file.

- Horizon designation

(Soil)

The horizon designation is a combination of several symbols:

(144).1 A number that gives information about discontinuities, i.e. the number of materials in which the soil has formed. This number is stored in the field HORIZON_DISCONTINUITY.

(144).2 One or two capital letters that designate the type of master horizon (or transitional horizon). These letters are stored in the field HORIZON_MASTER.

(144).3 Lowercase letters that designate subordinate characteristics of the horizon. These letters are stored in the field HORIZON_SUBORDINATE.

(144).4 A number that designate vertical subdivisions. This number is stored in the field HORIZON_VERTICAL.

(144).1 Horizon discontinuity

This field contains a number to indicate a discontinuity in the horizon designation (mandatory if exists).

- When the soil has formed entirely in one kind of material, a zero (0) is used (the field is NOT empty).

- When the soil has formed in several materials:

  - The upper part of the soil profile (corresponding to the first material) will be designated without number 1.

  - The part of the soil profile corresponding to the second material will be designated with the number 2.

  - The part of the soil profile corresponding to the third material will be designated with the number 3.

(144).2 Horizon master

This field contains the code of the master horizon, following the descriptions below.

| Code | Description |
| --- | --- |
| H | H horizon |
| O | O horizon |
| OL | OL horizon |
| OF | OF horizon |
| OH | OH horizon |
| A | A horizon |
| E | E horizon |
| B | B horizon |
| C | C horizon |
| R | R(ock) layer |
| I | I(ce) layer |
| AB, BA, EB, BE, BC, CR, etc. | Transitional horizon dominated by properties of one master horizon (symbolised by the first letter) but having subordinate properties of another master horizon (symbolized by the second letter). |
| E/B, B/E, B/C etc. | Transitional horizon in which distinct parts have recognizable properties of two kinds of master horizons. |

(144).3 Horizon subordinate

| a | Evidence of cryoturbation |
| --- | --- |
| b | Buried horizon |
| c | Concretions or nodules |
| d | Dense layer (physically root-restrictive, not used in combination with m) |
| f | Frozen soil (not used in combination with l) |
| g | Strong gleying |
| h | Accumulation of organic matter |
| i | Slickensides |
| j | Jarosite accumulation |
| k | Accumulation of pedogenetic carbonates |
| m | Strong cementation or induration (pedogenetic, massive) |
| n | Pedogenetic accumulation of exchangeable sodium |
| o | Residual accumulation of sesquioxides (pedogenetic) |
| p | Ploughing or other artificial disturbance |
| q | Accumulation of pedogenetic silica |
| r | Strong reduction |
| s | Illuvial accumulation of sesquioxides |
| t | Illuvial accumulation of clay |
| u | Urban and other man-made materials |
| v | Plinthite |
| w | Development of colour or structure in B (only used with B) |
| x | Fragipan |
| y | Pedogenetic accumulation of gypsum |
| z | Pedogenetic accumulation of salts more soluble than gypsum |

(144).4 Horizon vertical

A number is given to designate the vertical subdivision of a master horizon on the basis of structure, texture, colour, etc. The number 1 is used to designate the upper part of the master horizon. The number 2 the part of the master horizon situated below, etc. If there is no vertical subdivision, a 0 (zero) shall be submitted.

- Horizon limit superior/inferior

(Soil)

The upper/lower limit of the horizon depth (in centimetre). The limit between organic and mineral horizons corresponds to 0 cm. For organic horizons, the limits are negative values. For mineral horizons, the limits are positive values. Note that on peat soils thicker than 40 cm, the 0 cm line is located at the upper limit of the peat layer.

- Horizon distinctness d_hori_distinct

(Soil)

The distinctness of the lower horizon boundary refers to the thickness of the boundary zone in between adjacent horizons. The topography of the boundary indicates its shape.

| Code | Distinctness | (cm) |
| --- | --- | --- |
| 1 | Extremely abrupt | 0.3 - 1 cm |
| 2 | Very abrupt | 1 – 2 cm |
| 3 | Abrupt | 0 - 2 cm |
| 4 | Clear | 2 - 5 cm |
| 5 | Gradual | 5 - 15 cm |
| 6 | Diffuse | >15 cm |

- Horizon Topography D_HORI_TOPOGRAPHY

(Soil)

The topography of the boundary indicates its shape.

| Code | Topography |  |
| --- | --- | --- |
| 1 | Smooth | Nearly plane surface |
| 2 | Wavy | Pockets shallower than they are wide |
| 3 | Irregular | Pockets deeper than they are wide |
| 4 | Broken | Discontinuous |
| 5 | Complex |  |

- Structure d_soil_structure

(Soil)

| Code | Description |
| --- | --- |
| 1 | Platy |
| 2 | Prismatic |
| 3 | Columnar |
| 4 | Angular blocky |
| 5 | Subangular blocky |
| 6 | Granular |
| 7 | Crumbly |
| 8 | Massive |
| 9 | Single grain |
| 10 | Wedge-shaped (e.g. slickensides) |

- Code coarse fragments D_soil_coarse_fragments

(Soil)

| Code | Code Signification |
| --- | --- |
| 9 | No stones or gravel |
| 1 | Very few (< 5% by volume) |
| 2 | Few (5 -15% by volume) |
| 3 | Frequent or many (15 – 40% by volume) |
| 4 | Very frequent, very many (40 – 80% by volume) |
| 5 | Dominant or skeletal (> 80% by volume) |

- Code porosity d_soil_code_porosity

(Soil)

| Code | Description |
| --- | --- |
| 1 | very low (< 2 % by volume) |
| 2 | low (2 – 5 % by volume) |
| 3 | medium (5 – 15 % by volume) |
| 4 | high (15 – 40 % by volume) |
| 5 | very high (> 40 % by volume) |

- Mean highest and mean lowest ground water table D_GROUND_WATER_TABLE

(Soil)

| Code | Description |
| --- | --- |
| 0 9 | No groundwater table |
| 1 | Groundwater table between 0 – 50 cm |
| 2 | Groundwater table between 50 – 100 cm |
| 3 | Groundwater table between 100 – 150 cm |
| 4 | Groundwater table between 150 – 200 cm |
| 5 | Groundwater table between below 200 cm |

- Type of water table D_ WATER_TABLE_type

(Soil)

| Code | Description |
| --- | --- |
| 9 | No water table |
| 1 | Perched water table |
| 2 | Permanent water table |

- Root abundance D_ROOT_ABUNDANCE

(Soil)

The abundance of roots should be reported for four different size classes, using the codes ‘0, 1, 2, 3 or 4’ based on the frequency as number of roots/dm2.

| Code | Size class: | Very fine <0.5 mm | Fine 0.5-2 mm | Medium 2-5 mm | Coarse >5 mm |
| --- | --- | --- | --- | --- | --- |
|  | Abundance: |  |  |  |  |
| 9 | None | 0 | 0 | 0 | 0 |
| 1 | Very few | 1 - 20 | 1 - 20 | 1 - 2 | 1 - 2 |
| 2 | Few | 20 - 50 | 20 - 50 | 2 - 5 | 2 - 5 |
| 3 | Common | 50 - 200 | 50 - 200 | 5 - 20 | 5 - 20 |
| 4 | Many | >200 | >200 | >20 | >20 |

- Sample Code Litter Fractions D_litter_sample

(Litterfall)

| code | Fraction of Litterfall |
| --- | --- |
| 10 | Total |
| 11 | Foliar litter |
| 11.1 | Foliar litter of main tree species |
| 11.2 | Foliar litter of other tree species |
| 12 | Non foliar litter total |
| 13 | Flowering total |
| 13.1 | Flowering main tree species |
| 13.2 | Other Flowering |
| 14 | Fruiting/seeds total |
| 14.1 | Fruiting/seeds (main species + green cones) |
| 14.2 | Fruit Capsules (main species + empty cones) |
| 14.3 | Rest of fruiting |
| 15 | Budshells Bud scales |
| 16 | Twigs/branches |
| 17 | Fines and Frass |
| 19 | Other biomass |

- Functional Group – Ground Vegetation Biomass Analyses d_functional_group

(Ground Vegetation Biomass)

| code | Functional group |
| --- | --- |
| 1 | Bryophytes |
| 2 | Lichens |
| 3 | Ferns |
| 4 | Grasses |
| 5 | Herbs |
| 6 | Foliage of deciduous shrubs |
| 6b | Stems of deciduous shrubs |
| 7 | Foliage of evergreen shrubs |
| 7b | Stems of evergreen shrubs |
| 8 | Rest |

- Two letter code of Surveys d_surveyS

(LAI)

| code | Survey |
| --- | --- |
| SI | System Instalment |
| CC | Crown Condition |
| SO | Soil |
| SS | Soil Solution |
| FO | Foliage |
| GR | Growth and Yield |
| GV | Ground Vegetation |
| DP | Deposition |
| MM | Meteorology |
| PH | Phenology |
| OZ | Ozone |
| AQ | Air Quality |
| LF | Litterfall |
| LA | Leaf Area Index |
| TV | Tree Vitality |
| SW | Soil Water |
| C1 | Cronw Condition Level1 |
| F1 | Foliage Level1 |
| S1 | Soil Level 1 |

- Measurement or Average Value d_measurement_average

TV

| code | Survey |
| --- | --- |
| 1 | Measurement |
| 2 | Average value |

- Continuous dentrometer or girthband d_dendrometer

TV

| code | Survey |
| --- | --- |
| 1.1 | Point dendrometer |
| 1.2 | Circumference dendrometer |
| 2 | Girthband measurement |

- Perennial d_perennial

Ozone

| code | Survey |
| --- | --- |
| P | Perennial |
| A | Annual |