| Sample pretreatment and storage method | The influence of storage under different conditions on the chemical composition of deposition and soil water samples | Deposition samples and soil solution |
| --- | --- | --- |

# Problem:

Storage of water samples can lead to changes of the chemical composition of the samples. The mostly changing parameters are pH, DOC, IC, NH4, NO3 and Ntot.

The following parameters have an effect on the changes of the chemical composition:

- Time of storage

- Temperature

- Type of filtration before storage

The first unintended „storage“ happens in the field: weekly or 2-weekly sampling means „storage“ in the field for up to 2 weeks under uncertain conditions! Therefore the organized storage test gives also some informations about the possible changes of the chemical composition of the samples in the field.

# Description of the test:

13 different samples (5 stemflow-, 5 throughfall- and 3 soil water samples) from 5 different plots were taken in the field within one day after rainfall and brought to the laboratory.

Each sample was divided into 16 subsamples; 8 of them were filtrated with washed paper filters (Sartorius „Schwarzband“, 12-15-µm).

Respectively 2 filtered and 2 unfiltered subsamples were stored under different temperature conditions: 4 °C,  8 °C,  20 °C  and  30 °C

Each of the 208 subsamples were measured after 0, 2, 5, 13, 27 and 86 days for the following parameters: pH, TC, IC, NH4, NO3 and Ntot (1248 samples analysed)

- Results:

1. Most of the tested samples changed their chemical composition during storage.

2. Changing parameters were pH, TC / TOC, NH4 and Ntot.

3. The changes in unfiltrated samples normally were higher than in filtrated samples, but in filtrated samples there were also changes.

4. In the majority of cases the changes were higher at higher temperatures .

5. The pH increased; filtered samples with pH < 4,9 didn´t change.

6. TC / TOC decreased; typically the decrease was higher with increasing temperature.

7. NH4 can increase and decrease; typically the increase or decrease was higher or faster with increasing temperature.

8. NO3 didn´t change in the tested samples.

9. In filtered samples stored at 4°C only the TC /TOC content decreased; the other parameters didn´t change.

10. The results for the unfiltered samples stored up to 13 days at different temperatures show many different changes for pH, TOC / TC / IC, NH4 and Ntot. That means, that there must be many changes for these parameters in the different samplers in the field before the samples were collected.

# Literature:

---

# Executive laboratory:

Institute: Nordwestdeutsche Forstliche Versuchsanstalt

Name of contact person: N. König

Street: Graetzelstrasse 2

City: D-37079 Goettingen

Country: Germany

E-mail: nils.koenig@nw-fva.de

- Tables, graphs, appendix:

Table 1 : Characteristics of the stored samples

| plot | sample typ |  | tree species | pH | TC |
| --- | --- | --- | --- | --- | --- |
| Lüss | Stemflow | ST | oak | 3,8 | 350 |
| Lüss | soil water | LY |  | 4 | 70 |
| Solling | throughfall | KR | spruce | 4 | 20 |
| Solling | Stemflow | ST | beech | 4,3 | 10 |
| Solling | soil water | LY |  | 4,5 | 5 |
| Göttinger Wald | Stemflow | ST | beech | 4,9 | 5 |
| Solling | throughfall | KR | beech | 5 | 5 |
| Harste | Stemflow | ST | beech | 5 | 10 |
| Harste | throughfall | KR | beech | 5 | 5 |
| Lüss | Stemflow | ST | beech | 5,2 | 10 |
| Göttinger Wald | throughfall | KR | beech | 5,4 | 5 |
| Lüss | throughfall | KR | oak/beech | 6 | 10 |
| Göttinger Wald | soil water | LY |  | 8 | 70 |

Many detailed results and graphs can be seen on the ICP-Forests-website (reports from the expert panel deposition meeting in Oslo 2006).